3GPP TSG-RAN WG2 #98	R2-1704378

Hangzhou, China, 14 – 19th May 2017

Agenda item:	10.4.1.1

Source:	Ericsson (rapporteur)

Title:	Report of email discussion [97bis#16][NR] QoS parameters (Ericsson)

WID/SID:	NR_newRAT-Core

Document for:	Discussion

# 1	Introduction

This document collects the email discussion on the need for additional or clarified QoS parameters from a radio interface point of view because of the SA2 LS in [1]. The scope of this email discussion as per the RAN2 outcome is listed below.

[97bis#16][NR] QoS parameters (Ericsson)

Evaluate the QoS parameters provided by SA2 in their LS and the potential new QoS parameters. Any new parameters need to be justified based on some deficiency in the parameters from SA2. Can also consider whether any of the parameters are useful to be known in the UE.

Intended outcome: Email discussion report to next meeting so that an LS response can be sent from the next meeting.

Deadline:  Thursday 27/04/2017

The outcome should result in input to SA2 and RAN3 in their discussions on possible changes to existing definitions or in defining additional QoS information. Companies are invited to comment on input from companies. This helps the rapporteur to draft an LS in a timely manner.

Note: A draft LS to be created at the end of this email discussion.

# 2		SA2 agreed system level QoS Information

Below are the so far agreed QoS related information per QoS flow (see TS 23.501):

- ARP

- Guaranteed Flow Bit Rate (GFBR) - UL and DL;

- Maximum Flow Bit Rate (MFBR) -- UL and DL.

- 5G QoS characteristics

  - Resource Type (GBR or Non-GBR);

  - Priority level;

  - Packet Delay Budget;

  - Packet Error Rate.

- Notification control (indicates whether notification to CN should be made if the QoS targets cannot be fulfilled for a GBR QoS flow during the lifetime of the QoS flow).

RAN2 has been asked to respond on whether the above QoS information is sufficient for RAN to efficiently provide the QoS required for the 5G use cases and requirements.

## 2.1 Existing QoS Information

In the RAN2 discussion in RAN2#97bis, some companies believe that the definitions could be improved, and complemented, so that wanted service treatment is well described and can be utilized by RAN to optimize both service and network performance. E.g. [2]-[4].

Question 1: Which of the above agreed QoS information and related definitions need clarification or amendments to unambiguously satisfy requirements from a radio interface perspective, if any?

Please describe company findings in detail and provide suggestions on possible recommendations if any.

Companies are invited to respond to issues brought by other with views, suggestions or clarifying views

| Company | QoS Information, Issue | Recommendation to SA2/RAN3 |
| --- | --- | --- |
| Vodafone | Notification control: The use of this parameter is not clear and not explained well. Till now the GBR information has to be respected by Radio and if Radio can not fulfill GBR, then the bearer setup has to be rejected. It is not clear what this information would cause on the CN side and what is the related impact to the Radio. It is also not clear what is the expected behavior of the Radio once the eNB sent such information to the CN. Can it release the bearer after X seconds or should it wait till CN does something. Also not clear at which point of time, Radio would send this information to the CN. Overall the whole concept behind this “notification control” is not really clear | As long the use case and corresponding/related actions from CN and Radio are not clear (and there is no explanation e.g. by discussion paper) to RAN2, RAN 2 should question the usefulness of this parameter and ask for clarification. |
| Ericsson | Packet Delay Budget and Packet Error Loss Rate: The current definition of PDB is defined as …” should be interpreted as a maximum delay with a confidence level of 98 percent..”. For most URLLC services it is not sufficient to deliver 98% of the packets within the delay budget. A packet that arrives too late is typically equally bad as a lost packet. For delay critical services like URLLC, packets delayed more than the PDB should rather be counted as lost, and included in the PELR requirement. However, the packets should not be discarded. The RAN should in these circumstances choose a configuration without many MAC HARQ and RLC ARQ retransmissions and possibly duplication. While a 2% delay of packets would achieve the required low PELR as per current definition, it would exceed the PDB for a non-negligible share of the packets and thereby not achieve the reliability requirement. | Recomment that SA2 should discuss how the RAN can be made aware of how to interpret the PELR and PDB requirement of a certain flow to fulfil the reliability requrements and if an indication (e.g. via N2) informs on whether delayed packets exceeding PDB should be counted as lost. |
| Ericsson | Priority Level parameter: The parameter ‘Priority level’ is defined in LTE as the priority to get radio resources (23.203): “…a scheduler shall meet the PDB of an SDF aggregate on Priority level N in preference to meeting the PDB of SDF aggregates on next Priority level greater than N..” and “…the treatment of traffic exceeding an SDF aggregate's GBR are out of scope of this specification…” has several issues: - flows with high priority will get absolute priority to transmit all offered data within the PDB and at high load, there will not be any resources left for flows of lower priority - definition does not consider that packet flow rates may be better handled by AQM in these situations - It does not state how Priority level shall be used to distribute the remaining resources among the flows | The Priority Level parameter should be clarified as “The priority to get sufficient radio resources to fulfil QoS requirements”. It should also be clarified how to distribute resources between flows after the requirements have been met. RAN 2 should recommend SA2 to clarify the Priority Level definition to enable that spare resources are distributed between all flows, taking Priority Level into account. |
| Huawei HiSilicon | Notification control: agree with Vodafone. Packet Delay Budget, Packet Error Loss Rate and and Priority Level parameter: these three parameters are reused from LTE system. For URLLC services, SA2 is studying how to fulifll their requrements and have a note in TS 23.501 that “this table will be extended/updated to support service requirement for 5G, e.g. ultra low latency service”. As this work is ongoing in SA2, we don’t see any particular reason asking SA2 further clarification so far on the support of URLLC services. | RAN2 should ask SA2 regarding the use/motivation of notification control, and what the expected RAN2 behavior is when the requirements of GBR QoS flows can not be fulfilled. There is no need asking SA2 regarding QoS parameters supporting URLLC services since SA2 is considering to extend/update the 5QI table. |
| Nokia | Notification Control: agree with Vodafone about the ambiguity since it essentially leaves the procedure open ended (what happens if a requested parameter cannot be fulfilled). |  |
| Nokia | Individual packet level parameters (such as packet delay budget, packet error rate) is enforced in a per-packet basis without considering the inter-dependencies or coupling between multiple packets. These parametrs do not consider what would it take to delay or discard a specific packet from higher protocol layer point of view (would it introduce retransmission, rate adaptation, etc.), or what would be the impact on the end user perceived service quality (e.g., would it risk delaying the start of video playback or introduce buffering in an ongoing video session). | To compensate for the issues, QoS parameters should enable to capture goals/targets not only on packet level or QoS flow bit rate level but also on end user perceived quality level. |
| Nokia | QoS flow level aggregate data rates (GFBR/MFBR). These targets are enforced on individual QoS flows without to considering the inter-dependencies or coupling between QoS flows sharing the same resources. Defining and enforcing these targets may not be sufficient to deliver the right level of end user perceived quality (or “quality of experience”) for end users. | To compensate for the issues, QoS parameters should enable to capture goals/targets not only on packet level or QoS flow bit rate level but also on end user perceived quality level. |
| ZTE | Notification Control: Agree that the RAN expected bahaviour after notification is sent in unclear. | Ask SA2 regarding the expected behavior when the requirements for GBR QoS flows can not be fulfilled and notification is sent |
| ZTE | Packet Delay Budget and Packet Error Loss Rate: We share the concern from Ericsson but also acknowledge that SA2 is further working on this. | No strong view (we are not against raising this issue in a LS to SA2) |
| vivo | Notification control: agree with Vodafone. | RAN2 should ask SA2 to clarify the expected handling and results by the notification procedure. |
| CATT | Notification Control: I have the same concern with VF, Huawei, how “notification control” works and how it brings benefits. |  |
| CATT | Packet delay budget, packet error loss rate and priority level parameters: We agree with Huawei, these legacy LTE QoS parameters can be re-used as defined in SA2 to fulfil the requirement of URLLC service. How to interpret the PELR and PDB requirement of a certain flow to fulfil the reliability and delay requirements are left for implementation. |  |
| Intel | Notification control: Agree with others that the expected handling after sending this is unclear. How long should the RAN wait to see if CN is taking any action? Interaction with other RAN level congestion control, codec rate control etc. is also unclear. Packet Delay Budget and Packet Error Loss Rate: Clarification about the relationship between the two would be useful as pointed out by others. | Ask SA2 for clarification on the intended and allowed RAN behavior after sending Notification control and also on the relationship between Delay budget and Packet error rate. |
| Samsung | Notification control: In general, we tend to agree with concerns from other companies that this mechanism has a bit vague purpose in a sense that it is not clear when this notification is generated by RAN (only once per a flow, or for every packet in a flow, or periodically); and furthermore it is not clear what CN can actually do. Since the CN is not aware of exact radio conditions, should it just drop/remove the whole flow? | RAN2 should ask SA2 on usefulness of this mechanism and anticipated behavior for both RAN and CN side. |
| Samsung | Priority level: We do not mind clarifying if something is not clear, but our understanding is that it is up to the network implementation how it accounts for this parameter. We do not fully understand a comment from Ericsson that "flows with high priority will get absolute priority to transmit", there is no notion of the absolute priority in the current definition, as far as we can see. | We cannot see any big issue with the current definition, but are also to clarify it if something substantial is missing. |

Conclusions:

9 companies contributed to this discussion:

-	Several companies see it usefult to ask SA2 to calify the use and corresponding actions from CN related to the notification control to CN, if the QoS targets cannot be fulfilled in RAN.

-	There is support in recommending SA2 to clarify how to interpret the Packet Delay Budget and Packet Error Loss Rate requirement of a certain flow to fulfil reliability requrements.

## 2.2	Additional QoS Information relevant to RAN

In addition to the so far agreed QoS information, some new parameters may be beneficial to consistently meet wanted service treatment and performance.

Question 2: Which additional QoS Information would RAN benefit from to achieve high service and network performance to consistently satisfy the requirements of new services such as URLLC and other, if any?

For any new QoS Information, include descriptions on how/why this information is beneficial in RAN; including justification on why the additions are not fulfilled by the (so far agreed) QoS information defined in SA2.

Companies are invited to respond to input brought by other with views, suggestions or clarifying views.

| Company | QoS information and its description | Recommendation to SA2/RAN3 |
| --- | --- | --- |
| Vodafone | In general as we learned from 3G and 4G, the amount of information to be provided from CN to Radio has to be limited to an absolutely minimum and be reflected inside QCIs. I believe it makes sense to have a discussion on the amount of QCIs to be standardized. Any on top of QCIs parameters shall be included only if there is an absolutely need for and it should be clear why they can not be included inside QCIs. One example which could be considered as redundant information in my view is Preferred Minimum Flow Bit Rate from [2]: It is already possible to realise the intended behaviour in LTE by assigning of Prioritized Bit Rate to the logical channel and map a corresponding Non-GBR bearer/floow to a corresponding logical channel. This mechanism already today allows not downgrading a Non GBR bearer and I am not sure what proposed Preferred Minimum Flow Bit would add on top. From today prespective, I think we shall consider LTE QoS parameters as a basic and not really add anything. It is also not clear why e.g. Packet Delay Budget (PDB) can not be incorporated into GBR QCI and e.g. QCI 25 would indicate that it is GBR bearer and PDB is 99%. In this case, there is also no need to define any new GBR classes; it is just GBR with a certain characteristics. | In my view we should indicate to SA2/RAN3, that we would like to keep the current QCI structure and if we feel that current parameters are not sufficant we should ask the groups to incorporate it into the new QCIs while defining them and not create a new parameters to be send over interfaces (e.g. PDB can be incorporated as a requirement into a new GBR QCI). We should not start defining R99 Frame work with many parameters leading to the confusion and making the system more complex. |
| Ericsson | Preferred Bit Rate: In order to assign a corresponding Prioritized Bit Rate for non-GBR bearers/flows; so that the RAN is able to give guarantees with respect to the offered data rate, the RAN should be made aware of the application’s (flow’s, bearer’s) requirements. The QoS characteristics (PDB, priority) currently define only how the RAN should treat this bearer/flow compared to other bearers/flows and what Packet Delay Budget to maintain. Most services require a certain minimum bitrate to perform satisfactory and thus a “preferred Bit-Rate”, indicating what amount of resources to prioritize for the flow is needed. | SA2 should discuss the use of a parameter for indicating explicitly to the RAN a “Preferred Minimum Bit Rate” for non-GBR flows. The scheduler in the RAN may benefit in using this information to aim to allocate bitrate for the application to perform satisfactory, and aim to fulfil PDB for flows with offered load below the Preferred Minimum Bit Rate. |
| Ericsson | Averaging Time Window: The bitrate parameters GBR and MBR are fundamental for the QoS concept, but it is not defined how to measure it for any services except Voice. The RAN must maintain a moving average of the data rate offered to each flow and use this filtered value to determine whether it fulfils the GBR or the “Preferred Minimum Bit Rate”. The filter parameters (averaging window) have significant impact on the actual scheduler behaviour and throughput observed on application level. By introducing a definition of an “Averaging Time Window”, observability of QoS fulfilment can be consistent throughout the network | SA2 should discuss CN providing (for GBR- and Non-GBR flows) an “Averaging Time Window” that the RAN may use to asset the data rate of the flow consistently over the NW. |
| Ericsson | Periodic transmission: The traffic generated by some services show a well-defined periodicity pattern. If this periodicity is known to RAN, it can lead to significantly more effective RAN performance (e.g. use of SPS). | SA2 should discuss CN provided GBR- and Non-GBR flow "Periodicity" information, that RAN may use to determine the Periodicity of a flow. |
| Huawei HiSilicon | We generally agree with Vodafone to limit the QoS information with limited values/current 5QI table. But we envision the following additional QoS information which is benificial for RAN handling. Out of order delivery of QoS flows: Last RAN2#97bis meeting agreed that “It is desirable to disable PDCP reordering”. It requires the gNB to base some knowledge/characterisics of QoS flow(s) to make such decision. We think that SA2 input is helpful for RAN’s decision. Slice-level QoS parameters: e.g., per slice aggregate maximum bit rate (slice-AMBR) which is used for limit the aggregate bit rate across all non-GBR QoS flows for a specific slice. But this may be discussed in SA2 first. As for the “preferred bit rate”, “average time window” and “periodic transmission”, we think more motivation and clarification is needed, which could be discussed at RAN2#98 meeting. | SA2 should discuss which QoS flow(s) is allowed with out of order delivery, and which kind of information could notify to RAN to disable the PDCP reordering functionality. |
| Qualcomm | Averaging Time Window | Agree with Ericsson analysis above. In addition, this parameter is also useful for the UE to provide guidance for upper layer applications with adaptive rate adapation, e.g. VoLTE, DASH-based applications, etc. |
| Nokia | In general we agree with Huawei and VF that the number of traditional OoS parameters should be kept reasonable since they do not really make much difference in practice. | SA2 should discuss the provision of intent level parameters instead of detailed metrics that may not always be relevant when user satisfaction is at stake. |
| Nokia | End User Perceived Quality Optimization: Boolean (true/false) Set by the CN, true value indicates to the (R)AN that it should optimize the end user perceived quality of this QoS flow in addition to also fulfilling the targets of all other already agreed QoS parameters. The CN does not specify the means of optimizing the end user perceived quality, i.e., it is left for the (R)AN implementation. | Benefit over existing QoS parameters: The inter-dependency or coupling of simultaneous traffic flows causes that treatment applied to one flow has also effect on the performance of other flows, which share fully or partly the same resources (e.g., same (R)AN buffer, aggregation link, etc.). This effect cannot be efficiently captured by CN defined QoS rules as the CN lacks the visibility into the real time AN resource context that defines these inter-dependencies (e.g., to know which flows need to be optimized together and what optimization action applied to a given QoS flow would have impact on the service of others). |
| Nokia | Web Page Download Time: Integer (1-60) The value represents (in seconds) the desired time budget in which a web page should be delivered to the end user. If the QoS flow carries the traffic of multiple applications, the (R)AN may autonomously identify the IP flows within the QoS flow to which this end user perceived quality optimization target applies. | Benefit over existing QoS parameters: as opposed to packet delays, loss rates or explicit bitrates, which do not directly correlate with the user perceived quality and hence their enforcement is not the key to provide good user experience, this parameter enables to define a high-level target that the (R)AN can enforce (e.g, via smart dynamic access resource management and prioritization). E.g., the (R)AN can autonomously define and update the low-level bitrate parameter adaptively by observing how well the download progresses to comply with the high-level parameter, with no need to receive continuous updates from the CN for the low-level parameter. |
| Nokia | Multimedia Pre-Buffering Time: Integer (10-10000) The value represents (in milliseconds) the desired time budget (counted from the beginning of the download) in which a buffered multimedia content should trigger playback status. If the QoS flow carries the traffic of multiple applications, the (R)AN may autonomously identify the IP flows within the QoS flow to which this end user perceived quality optimization target applies. This parameter gives the boundaries within which the (R)AN is entitled to perform AN resource manipulation. | Benefit over existing QoS parameters: same as above for a different application type (which is responsible for a large share of mobile network data traffic). |
| Nokia | Good Multimedia Experience Maximum Bit Rate: Integer (10-100000) The value represents (in kilobits/second) a per multimedia session bit rate limit within which the (R)AN is entitled to dynamically manage the AN resource allocation for multimedia flows within the QoS flow to make sure that the end user perceived multimedia experience is good (e.g., low pre-buffering time, no stalling). | The (R)AN should also conform to other bit rate limits (QoS flow or higher aggregate level) defined by other QoS parameters applying to this QoS flow. Benefit over existing QoS parameters: same as above. |
| Nokia | Averaging Time Window: this makes sense in order to define how bursty service is tolerated in case the parameter also specifies not only the window for measurement but also the window for enforcement (e.g., if the window is small, the provided throughput should be more steady and less fluctuating as it has to be achieved in shorter periods than with a larger window). |  |
| ZTE | The suggestion to introduce any new parameters besides the already agreed QoS related information in SA2 should be provided together with a careful evaluation of whether they can be incorporated into the already agreed QoS framework. And also if finally, after careful evalution, a new parameter is agreed to be introduced, it’s better to incorporate it into the standardized QCI table (possibly creating new QCI values), rather than signaling it over the interface. | Any new parameters suggested to be introduced should be justified first, e.g. checking whether they can be incorporated into the agreed QoS framework. And if a new parameter is finally deemed to be introduced, it should preferably be incorporated into the standardized QCI table rather than signaling it over the interface. |
| vivo | Slice-AMBR: the parameter indicates the aggregate maximum bit rate which allows for all bearers/flows associated with a slice. The slice specific QoS parameter can guarantee the isolation of resource allocation in the resource shortage case and guarantee user experience, realize convenient billing for diverse Service level agreements as well. [R2-1701483] provides more detail descriptions. | SA2 should discuss the necessity of the parameter. |
| CATT | In general we should not increase the number of QoS parameters unnecessarily. There is some aspect is left to node implementation for the support of QoS. the minimum set of parameters are guaranteed for HO purposes. |  |
| CATT | No need for Perceived quality: | Perceived quality: this parameter is RAN node imeplementation related, gNB decides how to treat inter-dependency or coupling of simultaneous traffic flows over the same DRB, and how to ensure the other QoS requirement. |
| CATT | No need for Averaging time window | Averaging time window: question on wheatear this is for UL and/or DL. In DL why it is not possible to be left to the network implementation |
| CATT | No need for acket arrival pattern for assisting SPS configuration: | Packet arrival pattern for assisting SPS configuration: whether multiple UL SPS is allowed was raised in the last meeting, and no use case is identified. The only use case for UL SPS is VoIP, and the traffic pattern can be decided by eNB. |
| Intel | In general, our view is also that additional QoS parameters should be considered only if clear need is identified. With regard to preferred bit rate, what should the system behavior be if it cannot be met? Should a Notification control be sent to CN? If so, is it similar to GBR flows from service persective? | Clear motivation and justification should be studied before introducing new QoS parameters. |
| Samsung | There are already quite many QoS parameters, so a decision to introduce new ones should be well justified and motivated. In many cases "new" parameters are not that different from existing ones, and/or can be expressed through existing QoS values. End User Perceived Quality Optimization. We cannot see how a Boolean value can optimize the gNB implementation, especially taking into the fact that a perceived quality can many different things for different services. It seems that this is just a different name for the "priority level" parameter. Multimedia xxx parameters. It would be nice if a proponent provides further details on how it can be actually taken into account by the scheduler. End user players behave quite differently, and some of them (e.g. YouTube) continuously pre-buffers data in chunks of data. Since the gNB cannot know when the next period starts, it is not clear how it would help. Furthermore, it is not even clear how the CN would set the right value. Averaging window. We see some value in having this parameter as the target throughput may have different values depending on the interal over which it is measured. However, based on comments from Ericsson, Qualcomm and Nokia it seems that proponents have slightly different understanding on what this parameter is and how it can be used. |  |

Conclusions:

10 companies contributed to this discussion

While there are several company proposals with their motivation on new QoS parameters, there is little or no support for any new specific parameter exept for defining an Avraging window.

Most companies think the existing QoS parameters to abstract QoS requirements between CN and RAN are in general sufficient and should be maintained in NR/NGC.

## 2.3	QoS Information relevant to UE

Some QoS information parametes (other or like those in 2.1 and 2.2) may be useful to be known in the UE to meet wanted service treatment and performance.

Question 3: Which QoS information is beneficial to being known at the UE, if any?

For any new QoS Information, include descriptions on how/why this information is beneficial in RAN/UE; including justification.

Companies are invited to respond to input brought by other with views, suggestions or clarifying views.

| Company | QoS Information, description | Recommendation to SA2/RAN3 |
| --- | --- | --- |
| Huawei HiSilicon | Out of order delivery of QoS flows: But it does not mean that the UE should be aware of this QoS information. Instead, the gNB may configure this to UE to disable the reordering function at PDCP layer, e.g., via RRC signaling. | The same as our answers to question 2. |
| Qualcomm | GFBR, MFBR, and Preferred Bit Rate: The GBR and MBR from CN for GBR flows (same as SA2 definition). Preferred Bit Rate should also be provided to UE if adopted. Average time window: the window used by the network to compute moving average of bit rates | It is helpful for UE to perform UL scheduling to fulfil QoS requirements of GBR flows when GBR and non-GBR flows are aggregated in 1 DRB. For the same purpose, priority level and packet delay budget may also be useful for UE scheduling if there is no other restriction to network implementation. Note that we assume this average time window is used by the base station based on CN input. |

Conclusions:

2 companies contributed to this discussion.

The limited discussion leaves this point without conclusion.

# 4 	Conclusions

# Below find a discussion summary and rapporteur conlusion on QoS parameters:

# 2.1 Existing QoS Information:

-	Several companies see it useful to ask SA2 to clarify the use and corresponding actions from CN related to the notification control to CN, if the QoS targets cannot be fulfilled in RAN.

-	There is support in recommending SA2 to clarify how to interpret the Packet Delay Budget and Packet Error Loss Rate requirement of a certain flow to fulfil reliability requrements.

2.2 Additional QoS information relevant to RAN:

-	While there are several company proposals with their motivation on new QoS parameters, there is little or no support for any new specific parameter except for defining a parameter for Averaging window.

-	Most companies think the existing QoS parameters to abstract QoS requirements between CN and RAN are in general sufficient and should be maintained in NR/NGC.

2.3 QoS Information relevant to UE:

The limited discussion leaves this point without conclusion.

Based on the discussion and conclusions above it is proposed that:

Proposal 1: 	From RAN2 perspective the existing QoS parameters, and in particular the concept of QCI/5QI to abstract QoS requirements between CN and RAN should be maintained in NR/NGC.

Proposal 2: 	Several companies saw a need or at least a benefit in providing a “averaging window” as new QoS parameter via N2. The RAN may use in this parameter in its scheduling decision e.g. to enforce MBR and GRB.

Proposal 3: 	Some companies proposed to define various additional explicit parameters but none of these parameters got sufficient support from other companies and thus no additional recommendation is made to SA2.

Proposal 5:	RAN2 to ask SA2 to clarify the use and corresponding actions from CN related to the notification control to CN, if the QoS targets cannot be fulfilled in RAN

Proposal 4: 	Liason to SA2 with the conclusion in RAN2 as a result of the discussion on QoS information.

# References

[1]	R2-1702492 (S2-171618), LS on 5GS QoS framework and parameters, from SA2 to RAN2, SA WG2 Meeting #119, 13 - 17 Feb 2017, Dubrovnik, Croatia

[2]	R2-1702755	RAN aspects of QoS parameters, Ericsson

[3]	R2-1702637	QoS Parameters, Nokia, Alcatel-Lucent Shanghai Bell

[4]	R2-1702951	RAN QoS parameterization for 5G, Qualcomm Incorporated