3GPP TSG-RAN WG2 #98	Tdoc R2-1704380

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.3.2

Source:	Ericsson

Title:	PDCP PDU formats

Document for:	Discussion, Decision

# Introduction

The following agreements regarding NR PDCP PDU format have been made in RAN2#97bis.

Agreements on PDCP PDU format

-	PDCP SN length should be up to at least up 18 bits (for data bearer). FFS if there are use cases in which larger value is needed and which value for AM/UM.

-	PDCP SN length 12 bits is supported for RLC UM and AM and for both DRB(s) and SRB(s)

-	LTE PDCP PDU format for DRBs will be the reused

This contribution captures our view on the PDCP PDU format topic which was discussed in RAN2 email discussion [1]

# Discussion

The D/C field could be useful in case where PDCP control messages are carried over SRB. Currently the PDCP control PDU data carried on DRB is related to UP data (i.e. PDCP status report, Interspersed ROHC feedback packet, PDCP duplication) and having control PDUs for SRB would allow messages, such as PDCP duplication activation / deactivation carried over the SRB [2]. One could also see benefit of having unified format for DRBs and SRB in order to reduce implementation complexity.

D/C field is used with PDCP PDU formats for SRBs

We are not convinced of the usefulness of the P field, i.e. polled PDCP status reporting, during continuous operation. The PDCP transmitter has sufficient information to avoid HFN desynch, e.g. based on RLC AM status reporting (in which case the PDCP status report would be redundant), as well as in RLC UM, where HFN desynch avoidance is solvable by implementation (e.g. based on HARQ observation). At exceptional link failures, PDCP status report is triggered as part of the RRC procedure handling mobility/RLF. Furthermore, it should be considered that together with the P-bit as introduced for LWA also a light-weight PDCP status report format was introduced (without bitmap), otherwise e.g. frequent polling would have led to a significant overhead.

P-bit could be included to enable forward compatibility for LWA. However, since the LWA feature is not in scope of this release, the P-field should not exists as the LWA may not be introduced in the same form as it is specified in LTE.

Email discussion [1] pointed out question about whether to include P-field for SRBs. Having P-field is used to request status reports from data PDUs for a specific DRB and having it for SRB does not have use case defined.

P-field is not used with SRBs or DRBs

Email discussion [1] raised a question about PDCP PDU type field. A PDU type field is mandatory in order to identify the type of the PDU. Replacing the Type-field with other solution, which identifies the PDCP PDU may introduce undesirable complexity to the system. Therefore, there is no value in removing the type field.

The length of the type field is defined to be 3-bits in LTE. Currently there are no use cases, which would require increase of the length. Therefore, we propose that the 3-bit length is kept as a baseline. However, the type field should not be limited to 3-bits if there are use cases for extending it [2].

PDCP PDU type field is present in all NR PDCP control PDUs

Length of the PDCP type field is 3 bits. It is FFS whether PDCP type field length should be increased

If ROHC handles on-link reordering or not depends on which ROHC profile is used. RFC3095-based profiles i.e. ROHCv1 is very sensitive to reordering between compressor and decompressor. ROHC-TCP (RFC6846) and ROHCv2 RFC5225 have built-in tolerance to on-link reordering. Above all, the discussion in NR should focus on later versions of ROHC rather than early versions that are depreciated.

ROHC feedback messages are always optional and insensitive to reordering. Implementations may ignore the feedback messages if they for some reason would arrive very late, but this would not break compression. If the ROHC feedback is received twice or received out-of-order, there is no problem in ROHC operation. Additionally, the ROHC feedback interval can be viewed as infrequent, which results that risk of reordering of 2 feedback packets is close to zero.

Since there is no problem in ROHC duplication or out-of-order delivery, PDCP SN should not be included in PDCP control PDUs.

PDCP Control PDUs does not include PDCP SN

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	D/C field is used with PDCP PDU formats for SRBs

Proposal 2	P-field is not used with SRBs or DRBs

Proposal 3	PDCP PDU type field is present in all NR PDCP control PDUs

Proposal 4	Length of the PDCP type field is 3 bits. It is FFS whether PDCP type field length should be increased

Proposal 5	PDCP Control PDUs does not include PDCP SN

# References

R2-170xxxx. Summary of EmailDisc 97bis 23 PDCP PDU format. Huawei

R2-1704374. PDCP dynamic reconfiguration. Ericsson