3GPP TSG-RAN WG2 #98	R2-1704929

Hangzhou, China, 15th – 19th May 2017

Agenda Item: 	7.1

Source:	Apple

Title:	Report of email discussion [97bis#06][LTE/LAA] LAA/WiFi sharing

Document for:		Discussion

# 1	Introduction

The issue of LAA UEs sharing HW components between LAA and Wi-Fi was discussed in [1][2][3][4]. It was agreed RAN2 aim to address the LAA/WiFi sharing case, with detailed agreements in RAN2#97bis[5]:

Agreements

1: Extending/simplifying IDC indication is used to address LAA/WiFi sharing issues.

2: The extended/simplified IDC indication for LAA Tx/Rx sharing does not require the implementation of the full legacy R11 IDC on both UE and network.

3: Spec change for LAA Tx/Rx sharing solution is from Rel-13 and onwards.

Post-RAN2#97bis email discussion was allocated to proceed with details of solutions on extending/simplifying IDC indication to address this issue.

[97bis#06][LTE/LAA] LAA/WiFi sharing (Apple)

Progress details of the solution to Extending/simplifying IDC indication to address LAA/WiFi sharing issues.

Intended outcome: Report to next meeting

Deadline:  Thursday 27/04/2017

# 2    Discussion

Design details related to mechanisms and extending/simplifying IDC indication signalling for LAA/WiFi sharing issue are discussed here, in order to provide the solid basis for the CR(s). The following details are covered during this email discussion:

- How to define the UE capability for LAA/WiFi sharing?

  - Using a new UE capability

  - Linking to LAA configuration

- Is the UE configured by the network to enable IDC indication for LAA/WiFi sharing?

  - The configuration should explicitly enable IDC for LAA/WiFi sharing

  - Could the UE consider IDC indication for LAA/WiFi sharing enabled by default when LAA is configured by network?

- IDC indication signaling details for LAA/WiFi sharing

  - List of affected carrier frequencies

  - List of LAA cells configured

  - etc.

- Can the UE suspend or resume once it sends the indication?

- Is stage 2 change needed to accommodate LAA/WiFi sharing issue for IDC indication?

- Others...

## 2.1 How to define the UE capability for LAA/WiFi sharing?

Companies are invited to provide details regarding their opinion on the UE capability, introducing a new UE capability for LAA TX/RX sharing solution or linking to UE support of LAA?

| Company | How to define the UE capability for LAA/WiFi sharing? |
| --- | --- |
| Intel | The agreement is to introduce the indication for Rel-13 onwards. Our thought is that we can introduce a new UE capability from Rel-13 onwards (in theory, Rel-13 UE capability signalling can still be added when Rel-14 has not been frozen). The new capability bit can only be set if the UE supports LAA. The Rel-13 network supporting the indication will configure the UE to enable the indication. If not the UE is not allowed to enable the indication. For linking to UE support of LAA, it means that UE implicitly enables the indication once the UE is configured with LAA configuration. One downside of this is that a Rel-13 eNB not supporting the indication may receive unknown indication from the UE. It is not clear to us whether it is allowed. |
| Apple | Introduce a new UE capability to indicate UE support IDC indication for LAA TX/RX sharing purpose. A UE that supports LAA sharing indication does not have to support R11 in device TDM and FDM functionalities. |
| Ericsson | Usually RAN2 talks about capability bits after a feature has been standardized. So it seems too early for RAN2 to decide if a new capability is needed for this at this point in time. |
| BlackBerry | In order to enable the network to configure the UE with the feature, it needs to be aware of its capability. So, adding a UE capability would be a cleaner way to do this since this changing the Rel-13 feature. It is also feasible to mandate this for UEs implementing LAA from Rel-13 (i.e. implicitly link this to LAA) if all companies agree (we have no specific preference eitherway). |
| Qualcomm | Agree with Intel and Apple to introduce a new UE capability from R13. |
| MediaTek | A new capability can be introduced for LAA/WiFi sharing issue. The capability can be indicated only if LAA feature capability is also indicated. |
| Nokia, Alcatel-Lucent Shanghai Bell | Assuming this feature is introduced, a new capability is needed for testing purposes, no matter from which release the feature is introduced: Since Rel-13 has been frozen, so we cannot assume all UEs and all networks support this new functionality. If we want to make the capability more usable, we can make the capability conditionally mandatory for Rel-14 UEs supporting LAA to ensure it is used more widely. The capability should be per UE since this is a UE HW issue. Either Rel-13 or Rel-14 is fine to us. For signalling, since this is an extension of IDC, it could go to Other-Parameters as e.g. inDeviceCoexInd-LAA, with the semantics being e.g. inDeviceCoexInd-LAA Indicates whether the UE supports in-device coexistence indication for LAA. A UE that indicates support for this may not be able to utilize LAA at the same time as Wifi. It also seems that the capability does not require TDD/FDD differentiation, since it would only apply to LAA bands (which is TDD by nature). |
| Samsung | We have same opinions as Intel, which follows general principles in LTE: A new UE capability should be introduced from Rel-13. |

Observation 1: Among the 8 companies provided feedback, 7 companies thought a new UE capability should be introduced; 1 company thought it’s too early to introduce UE capability at this point in time since the feature has not been finished.

## 2.2 Is the UE configured by the network to enable IDC indication for LAA/WiFi sharing?

Companies are invited to provide details regarding whether the UE should be configured by the network to enable IDC indication for LAA/WiFi sharing and how. UE can be configured either implicitly or explicitly. In the case of implicit configuration, the UE configured with LAA SCell and supporting the IDC indication for LAA/WiFi sharing will assume that it is enabled to use the IDC indication for LAA/WiFi sharing. In the case of explicit configuration, the UE has to be configured by the network to enable IDC indication for LAA/WiFi sharing.

| Company | About network configuration to enable IDC indication for LAA/WiFi sharing |
| --- | --- |
| Intel | Upon the UE indicating support of the IDC indication for LAA/WiFi sharing in the UE capability, it is expected that the eNB supporting the indication will enable the indication once the UE is configured with LAA (which is in the same message or prior to LAA being configured). |
| Apple | Both implicitly and explicitly configuration by network to enable IDC indication for LAA/WiFi sharing are fine. Please also be noted that IDC indication for LAA/WiFi sharing purpose would only be useful when it is enabled together with LAA configuration. |
| Ericsson | The eNB configures which features the UE applies, and that principle should apply also now. Further, we assume that the simplification should be generic and could be applied also in non-LAA scenarios so we would like to avoid tying any potential simplification to a certain scenario. And then of course the name of the feature should not be something like "IDC indication for LAA/WiFi sharing". |
| BlackBerry | We prefer to align this with eLWA suspend/resume operation. i.e. configuration by the network upon configuring LAA and then the UE is allowed to send these indciations to the network. Ofcourse the assumption is that both LAA network and UE support this feature as Apple mentioned, (i.e. the UE shall be configured by the network for these indications when LAA is configured). We donot agree that this should be a generic indication. The legacy IDC indication already exists to indicate “generic” IDC issues and this new indication is now proposed specifically for the purpose of LAA and we should keep the scope of the discussion focussed on this aspect. Infact one of the main issues with the current IDC indication is that it is generic and there is no specific UE behaviour upon sending this indication and there is no specific network reaction specified either. Instead we should have specific mechanism to solve this issue with a specified UE behaviour (like in eLWA suspend/resume). |
| Qualcomm | Network configures/enables the UE to use this feature. We think we should design the signalling only based on LAA use case, but we should not put artificial limitations to prevent applying the new signaling to other possible scenarios of “temporary SCell unavilability”. |
| MediaTek | Same view as Intel. |
| Nokia, Alcatel-Lucent Shanghai Bell | Just as with normal IDC, allowance of the new feature is needed. Otherwise networks may not be able to cope with the UE actions. Explicit configuration is also needed for legacy-compatibility: What if a legacy LAA eNB configures normal IDC for a UE that supports the new IDC for LAA/Wifi sharing IDC? Therefore, a new parameter enabling this functionality is probably needed, even if we use the existing IDC signalling framework. |
| Samsung | Again same as Intel to follow the general principles: Network should explicitly configure the feature. |

Observation 2: 8 companies provided feedback, and all the 8 companies thought the UE should be configured by the network to enable IDC indication for LAA/WiFi sharing.

## 2.3 IDC indication signaling details for LAA/WiFi sharing

Companies are invited to provide details of what information are needed on the indication, how the IDC indication can be extended/simplified to address the LAA/WiFi sharing problem (i.e. whether the existing indication can be reused with some simplifications or a new indication is needed).

| Company | IDC indication signaling details for LAA/WiFi sharing |
| --- | --- |
| Intel | The UE is only needed to indicate the affected DL frequencies. Like in IDC, the IDC indication for LAA/WiFi RX sharing can only be triggered for frequencies for which a measurement object is configured. This will allow indication of such RX sharing, even if a LAA cell is not configured yet. This will prevent a LAA cell from being configured when the receiver is not available. We support the reuse of the existing indication, however, some simplifications are needed with the use of indication. There is no need for providing any time domain configuration or information as well as UL CA combination. There is also no need for the UE to perform autonomous denial as well as following the measurement requirement restriction as mentioned in Section 2.5. |
| Apple | In IDC indication for LAA/WiFi sharing purpose, the current IDC Indication could be simplified by allowing UE to report "affectedCarrierFreqList" the affected LAA-frequencies. It’s expected eNB could take this into account and deactivate the affected LAA SCells. |
| Ericsson | As RAN2 has identified, in case the LAA operation and WiFi operation causes problem in the device which the device cannot solve by itself, the in-device coexistence indication can be used to indicate the problems to the eNB. In case the WiFi operation is such that LAA operation is not feasible at all, e.g. continious WiFi communication, the UE will report "affectedCarrierFreqList" the affected LAA-frequencies. The eNB may then deconfigure the LAA carrier on the problematic frequency/frequencies. In case the UE determines that there is some time-pattern in the WiFi operation (e.g. scanning) the UE can indicate this to the eNB by indicating DRX-setting which could help solving the coexistence problems, or it could provide a subframe pattern in "idc-SubframePatternList". The eNB may configure a DRX configuration to help the UE, or deconfigure the LAA carrier if that is deemed suitable by the eNB. The IDC indication has one FDM component and one TDM component. Some companies indicated that the TDM-solution may not be helpful in this scenario. We believe that the IDC solution, at least, could allow the UE to omit the TDM-indications if a TDM-solution is not possible. In 5.6.9.3 in RRC we have the following text. From the yellow it seems that the UE must always include a TDM indication (either DRX or subframe pattern), so RAN2 could consider to amend the yellow text if needed. The UE shall set the contents of the InDeviceCoexIndication message as follows: 1> if there is at least one E-UTRA carrier frequency, for which a measurement object is configured, that is affected by IDC problems: 2> include the field affectedCarrierFreqList with an entry for each affected E-UTRA carrier frequency for which a measurement object is configured; 2> for each E-UTRA carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly; 2> include Time Domain Multiplexing (TDM) based assistance information: 3> if the UE has DRX related assistance information that could be used to resolve the IDC problems: 4> include drx-CycleLength, drx-Offset and drx-ActiveTime; 3> else (the UE has desired subframe reservation patterns related assistance information that could be used to resolve the IDC problems): 4> include idc-SubframePatternList; 3> use the MCG as timing reference if TDM based assistance information regarding the SCG is included; To simplify/clarify the IDC indication we could make it clear in the above that the TDM information does not have to be included. A suggestion was provided in R2-1703931. |
| BlackBerry | The indication should be clear and unambiguous in that it should indicate “hardware contention/unavailability of LAA TX/RX”. A per band/Sub-band-of-5GHz-band indication may suffice in our opinion. Ofcourse the UE should send this indication only for the carriers for which a measurement object exists (or LAA SCell is configured). It has always been clear that the TDM information is optional in the current IDC indication configuration. Clarifying this further is not necessary and in anycase, doesn’t solve the issue that is under discussion. |
| Qualcomm | After the simplification, the UE should be able to send an IDC message with only the following parameters in case of concurrent WLAN-LAA sharing use case (unpredictable WLAN usage). - A new parameter which is either “suspend/resume” or a new cause value to indicate this is to deactivate the Scells in the affectedCarrierFreqList (existing R11 IE) - A new parameter to indicate the “suspension duration” - affectedCarrierFreqList (existing R11 IE) In addition, for background WLAN scan use case (predictable WLAN usage), the UE may use periodic sharing via existing TDM patterns, as summarized in the table below which is from the IDC message ASN.1. It may be good to add 10ms DRX active Time to the table, to enhance existing TDM patterns to give finer granuality. Table 4.4.2 Allowed (DRX cycle, DRX active time) pairs in 36.331 v11.6 (2014 Jan) DRX Active Time (ms): LTE On & WiFi Off. DRX Cycle Length, i.e., LTE (On+Off) (ms); 40; 20; 30. 64; 20; 30; 40; 60. 80; 20; 30; 40; 60. 128; 20; 30; 40; 60; 80; 100. 160; 20; 30; 40; 60; 80; 100. 256; 20; 30; 40; 60; 80; 100. |
| MediaTek | A new field can be added into IDC message for indicating the LAA/WiFi sharing issue to specify affected cedarrier and range of ISM band. We also agree some simplifications are needed. There is no need for providing any time domain configuration or information as well as UL CA combination. There is also no need for the UE to perform autonomous denial as well as following the measurement requirement restriction as mentioned in Section 2.5. |
| Nokia, Alcatel-Lucent Shanghai Bell | There seem to be two simple choices: - Reuse existing fields (either plain IDC or UL CA part) OR - Define new fields indicating currently affected serving cells. To allow UE to indicate all of possible IDC indications, the second option seems to be more future-proof (otherwise we are disallowing use of IDC for vanilla IDC or UL CA purposes). Hence, we should extend the message with new fields for this use case. |
| Samsung | Even though we prefer to define a new message, it is okay to re-use the existing IDC message (non-critical extension) or the UEAssistanceInformation message. In case the IDC message is re-used, UE should not be required to include the existing fields (e.g. affectedCarrierFreqList, tdm-AssistanceInfo, etc.). |

Observation 3: 8 companies provided feedback. Details of how to simplify/extend IDC indication seems not quite converged.

Observation 3a: 5 companies thought new field or information element should be introducted, e.g., LAA sharing issue indication, band indication, suspend/resume indication, affected serving cells etc.

Observation 3b: 4 companies were ok to reuse "affectedCarrierFreqList"; 5 companies thought TDM and UL CA information is not needed for LAA sharing case; 1 company thought TDM granularity should be enhanced.

## 2.4 Can the UE suspend or resume once it sends the indication?

Companies are invited to provide comments on whether UE can suspend or resume once it sends the indication.

| Company | Can the UE suspend or resume once it sends the indication? |
| --- | --- |
| Intel | Upon sending the indication of an affected LAA SCell frequency, UE is expecting the eNB to perform deactivation of the affected LAA SCell, but should be able to handle eNB just suspending transmission over the frequency affected or releasing the affected SCell. On indicating a previously affected LAA SCell frequency is no longer affected, UE is expecting the eNB to activate of LAA SCell if it is previously deactivated and if it wish to, but should be able to handle eNB just resuming transmission over the frequency affected if it is still activated. Basically, once the indication is sent (either for affected LAA SCell frequency or for previously affected LAA SCell frequency is no longer affected), the procedure is assumed to end at the UE. |
| Apple | Both UE suspends/resumes data on SCells and network deactivate SCells are fine, we have no strong preference. Slightly prefer UE can take inmediate action to protect ISM when sharing problem happens and maximum data transmission on SCell when sharing problems are not there. |
| Ericsson | If the UE sends the IDC indication to the network can send take actions so that the UE no longer is using the problematic frequency, e.g. deconfigure carriers on problematic frequencies. So then of course traffic is "suspended" on that frequency. When the IDC problems no longer are applicable the UE can send an updated IDC indication which indicates that there is no problems any longer (given that the prohibit timer is not running of course). And the eNB could, if it wants, configure carriers on those frequencies which was earlier released. It is not clear to us that anything else should be added. |
| BlackBerry | Our preference is that the UE action upon sending this is aligned with the eLWA suspend/resume operation. Then we have a feature that is aligned between eLWA and LAA for solving this issue. |
| Qualcomm | Once UE sends the IDC message, the UE may take actions immediately to suspend/resume transmission and reception on the Scell(s). |
| MediaTek | With a suspend-resume mechanism, we think the UE can suspend autonomously (by deactivating LAA SCell), but for resume the network will have to explicitly activate the SCell once it receives the resume indication from the UE. |
| Nokia, Alcatel-Lucent Shanghai Bell | Network should decide on the action, just as with IDC. It is expected that a smart network that enables this feature also knows how to cope with the UE indication. |
| Samsung | Assuming user preferences (i.e. using Wi-Fi) take precedence over network configurations (i.e. LAA), UE should be able to suspend once it sends the positive indication. However, for resume, we assume network will configure/ activate the LAA SCells again, and can follow the legacy principles (assuming network deconfigure/ deactivate once it receives the positive indication). |

Observation 4: 8 companies provided feedback. 4 companies thought UE could suspend/resume the affected LAA Scells when LAA/Wifi sharing problems occur/release; 2 companies thought UE could suspend but the network should activate resuming; 2 companies thought network should take the actions.

## 2.5 Is stage 2 change needed to accommodate LAA/WiFi sharing issue for IDC indication?

Companies are invited to provide details on whether stage 2 change is needed to accommodate LAA/WiFi sharing issue for IDC indication, if yes, provide details of what kind of change is needed.

| Company | Stage 2 change needed? |
| --- | --- |
| Intel | As mentioned in Section 2.3, some simplications are needed for the indication of LAA/WiFI sharing and this includes not performing the measurement requirement as in the different phases of the IDC as in TS36.300 Section 23.4.2 (Table 23.4.2-1). Also, as part of the simplications, the UE may not provide all the assistance information to the eNB (i.e. DRX and time domain pattern for TDM solution). The following statement needs to be updated for the case the IDC Indication is used for LAA/WiFi sharing: To assist the eNB in selecting an appropriate solution, all necessary/available assistance information for both FDM and TDM solutions is sent together in the IDC indication to the eNB. |
| Apple | Yes. In section 23.4 of TS36.300, clarification is needed for the following purposes: - To address LAA/WiFi sharing problem. - To clarify that the extended/simplified IDC indication for LAA Tx/Rx sharing does not require the support of the legacy R11 IDC on both UE and network. |
| Ericsson | As seen in the following, we have already captured in stage-2 the LAA+WiFi scenario: "IDC problem can happen when the UE (intends to) uses WLAN on the overlapped carrier/band or adjacent carrier/band to the unlicensed carrier used for LAA operation " Since RAN2 has not agreed on any details of a "simplificiation", it is too early to say whether any Stage-2 changes are needed. |
| BlackBerry | Stage 2 updates can be captured as necessary once the details are finalised. |
| Qualcomm | If we change Stage-3, it is good to update Stage-2 to keep specs consistent. |
| MediaTek | Agree with Intel. Other clarifications may also be needed based on the exact solution agreed. |
| Nokia, Alcatel-Lucent Shanghai Bell | If the feature is introduced, it might also need to be reflected in section 23.4 of Stage-2. The exact text should be worked on with more care after the signalling has been decided, to account for the exact decisions. |
| Samsung | Agree with Intel. |

Observation 5: 8 companies provided feedback. 7 companies thought stage 2 update is needed for alignment based on the exact decisions of this feature. 1 company thought it’s too early to decide whether stage 2 change is needed.

## 2.6 Others

Companies can provide other topics on the signaling details that should be addressed in order to facilitate analysis of the spec impact, if there is any.

| Company |  |
| --- | --- |
| Qualcomm | We think we should design the signalling only based on LAA use case, but we should not put artificial limitations to prevent applying the new signaling to other possible scenarios of “temporary SCell unavilability”. |
| Nokia, Alcatel-Lucent Shanghai Bell | If the feature is introduced, we should have the CRs to all relevant specifications ready at the same time, i.e. at least: - TS36.300: Stage-2 description of the new feature and its usage - TS36.331: RRC signalling enabling the new functionality - TS36.306: UE capability descriptions related to the new functionality It is not clear if other specifications are affected by this functionality. |

# 3    Conclusion

Observation 1: Among the 8 companies provided feedback, 7 companies thought a new UE capability should be introduced; 1 company thought it’s too early to introduce UE capability at this point in time since the feature has not been finished.

Observation 2: 8 companies provided feedback, and all the 8 companies thought the UE should be configured by the network to enable IDC indication for LAA/WiFi sharing.

Observation 3: 8 companies provided feedback on details of how to simplify/extend IDC indication.

Observation 3a: 5 companies thought new field or information element should be introducted, e.g., LAA sharing issue indication, band indication, suspend/resume indication, affected serving cells etc.

Observation 3b: 4 companies were ok to reuse "affectedCarrierFreqList"; 5 companies thought TDM and UL CA information is not needed for LAA sharing case; 1 company thought TDM granularity should be enhanced.

Observation 4: 8 companies provided feedback. 4 companies thought UE could suspend/resume the affected LAA Scells when LAA/Wifi sharing problems occur/release; 2 companies thought UE could suspend but the network should activate resuming; 2 companies thought network should take the actions.

Observation 5: 8 companies provided feedback. 7 companies thought stage 2 update is needed for alignment based on the exact decisions of this feature. 1 company thought it’s too early to decide whether stage 2 change is needed.

Based on companie’s input and the observations, follow proposals are made to address the LAA/WiFi sharing problems:

Proposal 1: Introduce a new UE capability for LAA/WiFi sharing and this can only be set if UE supports LAA. It is for Rel-13 onwards.

Proposal 2: Network configuration is added to config the UE to enable IDC indication for LAA/WiFi sharing.

Proposal 3: For the details of the signalling in the IDC indication for LAA/WiFi sharing, one proposed way forward is to have:

- The existing affectedCarrierFreqList is used to indicate the affected carrier.

- TDM and UL CA combination information is not needed for LAA/WiFi sharing.

- New explicit indication to indicate that it is LAA/WiFi sharing issue

Proposal 4: UE can suspend the transmission and reception on the affected LAA SCells after sending the IDC indication for LAA/WiFi sharing problems, but the resuming should depend on the network activation.

Proposal 5: Stage 2 update is needed for alignment across the specs based on the exact decisions of this feature.

# 4    References

[1]		R2-168596: Concurrent RAT operation with RX sharing, Qualcomm Incorporated, Convida Wireless.

[2]		R2-164092: Concurrent RAT operation with NR, Qualcomm Incorporated, Convida Wireless

[3]		R2-1701756: “Concurrent LAA/Wi-Fi Using RX Sharing”, Apple, Qualcomm Incorporated, Broadcom, Intel Corporation, Interdigital communications, Convida Wireless

[4]		R2-1702572: Report of email discussion [97#57][LTE/LAA] LAA/WiFi sharing, Apple

[5]		RAN2#97bis Chairman notes