3GPP TSG-RAN WG2 #98 	R2-1706092

# Hangzhou, China, 15th-19th May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1023 | rev | 2 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LAA/WiFi sharing indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2017-04-27 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introducing an LAA related signalling to allow the UE to inform the eNB about its LAA SCell(s) Tx/Rx unavailability when LAA SCell(s) is configured or activated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Upate of the IDC indication solution to cover LAA/WiFi sharing problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Both UE supporting LAA through Sharing and network will face performance issues in LAA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 23.4.1, 23.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS36.331 CR2830 TS36.306 CR1461 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 23.4	Interference avoidance for in-device coexistence

### 23.4.1	Problems

In order to allow users to access various networks and services ubiquitously, an increasing number of UEs are equipped with multiple radio transceivers.

For example, a UE may be equipped with LTE, WiFi, and Bluetooth transceivers, and GNSS receivers. Due to extreme proximity of multiple radio transceivers within the same UE operating on adjacent frequencies or sub-harmonic frequencies, the interference power coming from a transmitter of the collocated radio may be much higher than the actual received power level of the desired signal for a receiver. This situation causes In-Device Coexistence (IDC) interference and is referred to as IDC problems. The challenge lies in avoiding or minimizing IDC interference between those collocated radio transceivers, as current state-of-the-art filter technology might not provide sufficient rejection for certain scenarios (see 3GPP TR 36.816 [50]). IDC problem can happen when the UE (intends to) uses WLAN on the overlapped carrier/band or adjacent carrier/band to the unlicensed carrier used for LAA operation. If there is a risk of IDC problem which cannot be avoided (e.g. by level of regulation), the IDC functionality for a UE should be configured by the eNB when the UE is configured for LAA operation.

A further interference-related problem is the unavailability on certain LAA SCells, when related UE hardware components, such as antennas, are shared between LAA SCells and other radio technologies such as WiFi.

### 23.4.2	Solutions

When a UE experiences IDC problems that it cannot solve by itself and a network intervention is required, it sends an IDC indication via dedicated RRC signalling to report the IDC problems to the eNB. The UE may rely on existing LTE measurements and/or UE internal coordination to assess the interference and the details are left up to UE implementation.

NOTE:	For instance, the interference is applicable over several subframes/slots where not necessarily all the subframes/slots are affected and consists of interference caused by the aggressor radio to the victim radio during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds.

A UE that supports IDC functionality indicates its IDC capabilities to the network, and the network can then configure by dedicated signalling whether the UE is allowed to send an IDC indication. The IDC indication can only be triggered for frequencies for which a measurement object is configured and when:

-	for the primary frequency, the UE is experiencing IDC problems that it cannot solve by itself;

-	for a secondary frequency, regardless of the activation state of the corresponding SCell, the UE is experiencing or expects to experience upon activation IDC problems that it cannot solve by itself;

-	for a non-serving frequency, the UE expects to experience IDC problems that it cannot solve by itself if that non-serving frequency becomes a serving one.

When notified of IDC problems through an IDC indication from the UE, the eNB can choose to apply a Frequency Division Multiplexing (FDM) solution or a Time Division Multiplexing (TDM) solution:

-	The basic concept of an FDM solution is to move the LTE signal away from the ISM band by e.g., performing inter-frequency handover within E-UTRAN, removing SCells from the set of serving cells, or in case of uplink CA operations, allocate uplink PRB resources on CC(s) whose inter-modulation distortion and harmonics does not fall into the frequency range of the victim system receiver.

-	The basic concept of a TDM solution is to ensure that transmission of a radio signal does not coincide with reception of another radio signal. LTE DRX mechanism is used to provide TDM patterns (i.e. periods during which the LTE UE may be scheduled or is not scheduled) to resolve the IDC issues. DRX based TDM solution should be used in a predictable way, i.e. the eNB should ensure a predictable pattern of unscheduled periods by means of DRX mechanism.

To assist the eNB in selecting an appropriate solution, all necessary/available assistance information for both FDM and TDM solutions is sent together in the IDC indication to the eNB. The IDC assistance information contains the list of E-UTRA carriers suffering from IDC problems, the direction of the interference and, depending on the scenario (see TR 36.816 [50]), it also contains TDM patterns or parameters to enable appropriate DRX configuration for TDM solutions on the serving E-UTRA carrier. Furthermore, the IDC indication can also be configured to include uplink CA related assistance information containing the victim system as well as the list of supported uplink CA combinations suffering from IDC problems. The IDC indication is also used to update the IDC assistance information, including for the cases when the UE no longer suffers from IDC problems. In case of inter-eNB handover, the IDC assistance information is transferred from the source eNB to the target eNB.

Hardware sharing problem is indicated to the eNB when a UE experiences hardware sharing problems for LAA SCells. When notified of hardware sharing problems through an IDC indication from the UE, the UE may expect eNB to deactivate the LAA SCell(s).

It’s allowed that a UE can be configured to indicate both IDC indication and hardware sharing indication.

IDC interference situation can be divided into following three phases as shown in Figure 23.4.2-1:

-	Phase 1: The UE detects start of IDC interference but does not initiate the transmission of the IDC indication to the eNB yet.

-	Phase 2: The UE has initiated the transmission of the IDC indication to the eNB and no solution is yet configured by the eNB to solve the IDC issue.

-	Phase 3: The eNB has provided a solution that solved the IDC interference to the UE.

Figure 23.4.2-1: Different phases of IDC interference related operations by UE

In different phases, UE behaviours related to RRM, RLM, and CSI measurements are shown in Table 23.4.2-1.

Table 23.4.2-1: RRM/RLM/CSI measurements in different phases of IDC interference

| Phases of IDC Interference | RRM Measurements | RLM Measurements | CSI Measurements |
| --- | --- | --- | --- |
| Phase 1 | Up to UE implementation and RRM measurement requirements (see TS 36.133 [21]) apply | Up to UE implementation and RLM measurement requirements (see TS 36.133 [21]) apply | Up to UE implementation and CSI measurement requirements (see TS 36.101 [52]) apply |
| Phase 2 | Unless TDM is not avaialbe, UE shall ensure the measurements are free of IDC interference and RRM measurement requirements (see TS 36.133 [21]) apply | Unless TDM is not avaialbe, UE shall ensure the measurements are free of IDC interference and RLM measurement requirements (see TS 36.133 [21]) apply (NOTE 1) |  |
| Phase 3 | Unless TDM is not avaialbe, UE shall ensure the measurements are free of IDC interference and RRM measurement requirements (see TS 36.133 [21]) apply | Unless TDM is not avaialbe, UE shall ensure the measurements are free of IDC interference and RLM measurement requirements (see TS 36.133 [21]) apply |  |
| NOTE 1: The UE should attempt to maintain connectivity to LTE in this phase meaning that RLM measurements are not impacted by IDC interference. If no solution is provided within a time which is up to UE implementation, the UE may need to declare RLF or it may continue to deny the ISM transmission. In DC, when the UE experiences IDC problems in SCG, if no solution is provided within a time which is up to UE implementation, the UE may need to declare RLF in SCG or it may continue to deny the ISM transmission in SCG. NOTE 2: If the UE determines in Phase 2 that the network does not provide a solution that resolves its IDC problems, it performs measurements as defined for Phase 1. NOTE 3: If the IDC indication message reports the IDC interference on a neighbour frequency, it performs RRM measurements for that frequency as defined for Phase 2. |  |  |  |

In addition, once configured by the network, the UE can autonomously deny LTE UL transmission in all phases to protect ISM in rare cases if other solutions cannot be used. Conversely, it is assumed that the UE also autonomously denies ISM transmission in order to ensure connectivity with the eNB to perform necessary LTE procedures, e.g., RRC connection reconfiguration and paging reception, etc. The network may configure a long-term denial rate by dedicated RRC signalling to limit the amount of LTE UL autonomous denials. Otherwise, the UE shall not perform any LTE UL autonomous denials.

### 23.4.2A	Solutions for hardware sharing problem

Hardware sharing indication (see TS 36.331 [16]) can be used when a UE experiences hardware sharing problems for SCells. The solution for hardware sharing problem does not require the support of the IDC solutions defined in section 23.4.2.

A UE that supports hardware sharing indication indicates its capability to the network. The UE can send the IDC indication only when it is configured to provide IDC indication and at least one LAA SCell is configured or a measurement object for an LAA SCell frequency exists and the UE is experiencing hardware sharing problems that it cannot solve by itself.

When notified of hardware sharing problems through an IDC indication from the UE, the eNB may deactivate the LAA SCell(s) or suspend transmissions on this SCell until the issue is resolved.

END OF CHANGES