3GPP TSG-RAN2 Meeting #98	R2-1706114

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1028 | rev | 2 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resources selection for PRACH triggered by a PDCCH order in eNB-IoT(Option2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson, Huawei/HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2017-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PRACH initiated by a PDCCH order has been introduced from R13 NB-IoT. The PRACH resource can be explicitly indicated by eNB through PDCCH order. However, without reserved PRACH resource and non-contention based random access procedure, the benefit of PRACH initiated by a PDCCH order cannot be fully achieved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the restrictions that NB-IoT UE doesn’t support non-contention based random access procedure. Impact analysis Impacted functionality: The changes only impact the corrected functionality. Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE will fail to determine PRACH resource for PRACH transmission in some scenarios. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT UE doesn’t support non-contention based random access procedure and the benefit of PRACH initiated by a PDCCH order cannot be fully achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.331 CR1113 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

10.1.5	Random Access Procedure

The random access procedure is characterized by:

-	Common procedure for FDD and TDD;

-	One procedure irrespective of cell size and the number of serving cells when CA is configured;

The random access procedure is performed for the following events related to the PCell:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure, except for NB-IoT UE that only uses Control Plane CIoT EPS optimizations, as defined in TS 24.301 [20];

-	Handover, except for NB-IoT or when RACH-less HO is configured;

-	DL data arrival during RRC_CONNECTED requiring random access procedure:

-	E.g. when UL synchronisation status is "non-synchronised".

-	UL data arrival during RRC_CONNECTED requiring random access procedure:

-	E.g. when UL synchronisation status is "non-synchronised" or there are no PUCCH resources for SR available.

-	For positioning purpose during RRC_CONNECTED requiring random access procedure:

-	E.g. when timing advance is needed for UE positioning.

The random access procedure is also performed on a SCell to establish time alignment for the corresponding sTAG.

In DC, the random access procedure is also performed on at least PSCell upon SCG addition/modification, if instructed, or upon DL/UL data arrival during RRC_CONNECTED requiring random access procedure. The UE initiated random access procedure is performed only on PSCell for SCG.

Furthermore, the random access procedure takes two distinct forms:

-	Contention based (applicable to all six events, but the sixth event for positioning is applicable for NB-IoT only);

-	Non-contention based (applicable to only handover, DL data arrival, positioning and obtaining timing advance alignment for a sTAG).

Normal DL/UL transmission can take place after the random access procedure.

An RN supports both contention-based and non-contention-based random access. When an RN performs the random access procedure, it suspends any current RN subframe configuration, meaning it temporarily disregards the RN subframe configuration. The RN subframe configuration is resumed at successful random access procedure completion.

For NB-IoT, the random access procedure is performed on the anchor carrier or on a non anchor carrier based on system information.

<End of change>