3GPP TSG-RAN WG2 #99	R2-1708029

Berlin, Germany, 21st – 25th August 2017	(Resubmission of R2-1706626)

Agenda Item:	10.2.8

Source:	Ericsson

Title:	RACH-less HO in NR when UE is in CA or DC

Document for:	Discussion, Decision

# Introduction

In LTE, HO procedure is handled with RRC connection reconfiguration procedure with MobilityControlInfo. The procedure involves re-establishment of PDCP and RLC entities and MAC entity reset. In the RAN2#97bis meeting, it was agreed to support RRC involved handover with and without PDCP entity re-establishment. The agreements are as below.

Agreements

1: 2 handling of handover for AM mode:

-	1: LTE- like handover

-	2: No Key change, Data Recovery, RLC re-establishment

2: 2 handling of handover for UM mode:

-	1: LTE- like handover

-	2: No Key change, RLC re-established

3: 2 handling of handover for SRB:

-	1: LTE- like handover

-	2: No Key change, RLC re-established

Furthermore, LTE Rel-14 WI on handover enhancements introduced functionality in LTE to support RACH-less handover.

In this paper, we will discuss the necessity to further optimize HO procedure for the case where UE is working with CA (Carrier Aggregation) or when UE is in DC (Dual Connectivity).

# Discussion

The reason to propose new handover procedure for NR as discussed in RAN2#96, 97bis meeting is because RAN internal architecture of NR could be different from LTE. One alternative for RAN internal architecture is a split architecture where PDCP is in a CU (centralized unit) while RLC and MAC could be in distributed unit (DU). Under this architecture, when UE moves from one DU to another DU but still under the same CU, since PDCP entity does not change, it is therefore not necessary to re-establish PDCP entity in this case.

The requirement for a new handover procedure is not just from network architecture perspective, it can also come from handover procedure when UE changes its Primary Cell in CA or changes its MgNB when it is in DC.

According to LTE specification, if the PCell needs to be changed, a handover procedure is needed. The baseline HO procedure in LTE includes the following: 1) security KEY change, 2) MAC reset, RLC and PDCP re-establishment, 3) random access procedure to establish synchronization with the new PCell.

Even with the new procedure as just agreed for NR (i.e. no KEY change), the procedure still includes 1) MAC reset and RLC re-establishment, 2) random access procedure to the new PCell to establish synchronization.

Actually, when PCell needs to be changed, there are three different cases as listed below.

Case 1:	An existing SCell becomes the new PCell

Case 2:	A neighbor cell becomes the PCell, but at least one SCell remain as SCell after PCell is changed.

Case 3:	PCell is changed to a new Cell and no previous SCell remains as SCell after PCell is changed

For case 1, random access procedure may not be needed at new PCell as time and frequency synchronization is already obtained for this new PCell when it served as SCell. Then RACH-less procedure similar to the one proposed in LTE R14 can also be applied here.

So the optimized HO procedure when an existing SCell is changed to a PCell could be either 1) security KEY change, 2) MAC reset, RLC and PDCP re-establishment; or 1) MAC reset, RLC re-establishment

Therefore, we propose

For PCell change procedure in NR, if an existing SCell is changed to a PCell, RACH-less HO procedure can be applied together with/without KEY change and PDCP re-establishment.

Similarly, in NR DC, the UE connects with two network nodes, MgNB (Master gNB) and SgNB (Secondary gNB), MgNB is the control plane anchor for the UE, that is, the control plane termination point with NextGenCore (Next Generation Core) is at the MgNB. When MgNB needs to be changed, a handover procedure is needed as in LTE DC, and during this period random access procedure is required to establish synchronization with new MgNB. The handover procedure includes: 1) Security KEY change, 2) MAC reset, RLC and PDCP re-establishment, and 3) random access procedure to establish synchronization with new MgNB.

When MgNB needs to be changed, there are different cases as listed below.

Case 1:	SgNB is the new MgNB

Case 2:	A neighbor gNB becomes the new MgNB, but SgNB remain as SgNB for the new MgNB.

Case 3:	A neighbour gNB become the new MgNB, and old SgNB is removed.

For case 1, random access procedure may not be needed as UE has already obtained time and frequency synchronization with the new MgNB, i.e. old SgNB. RACH-less procedure similar to the one proposed in LTE R14 can also be applied here.

So the optimized HO procedure when SgNB is changed to MgNB could be 1) security KEY change, 2) MAC reset, RLC and PDCP re-establishment;

Therefore, we propose

For MgNB change procedure in NR DC, if existing sgNB is changed to MgNB, RACH-less HO procedure can be applied.

# Conclusion

Based on the discussion in section 2,  we propose the following:

Proposal 1	For PCell change procedure in NR, if an existing SCell is changed to a PCell, RACH-less HO procedure can be applied together with/without KEY change and PDCP re-establishment.

Proposal 2	For MgNB change procedure in NR DC, if existing sgNB is changed to MgNB, RACH-less HO procedure can be applied.