3GPP TSG-RAN WG2 #99	Tdoc R2-1708353

Berlin, Germany, 21st – 25th August 2017	Update of R2-1707176

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Enhanced HARQ feedback mode in SPS

Document for:	Discussion, Decision

# Introduction

In RAN2-98AH, the following agreements were made in the chairman’s notes for SPS [1]:

=>	Modelling in the MAC for grant-free will be discussed after the difference between the two schemes is better understood pending RAN1 progress.  RAN2 will aim to have a unified MAC operation for common functionalities between grant-free and UL SPS with understanding that there can be differences after input from RAN1.

=>	RAN2 understands that to support UL SPS similar to LTE a mode of operation in which RRC configuration (with no initial PHY resources) with L1 activation/deactivation needs to be supported.  RAN2 will continue discussion on UL SPS, with LTE functionality.

=>	A common RRC signalling can be design to allow the configuration of different UL transmissions schemes.

Further, in the LS from RAN1 [2], two different types of UL transmission without grant have been agreed to be specified in RAN1. Type 1 is different from type 2 at least on the point that any L1 signalling is not required, and type 2 has some similarity with LTE UL SPS at least on the point that L1 signalling is used for activation/deactivation. RAN1 also agree for both types to support HARQ operations with repetitions, while further study is needed whether to configure multiple resources, and also to decide a timing reference for type 2. In addition, it is agreed in RAN1 that

•	NR supports more than 1 HARQ process for UL transmission without grant

In this paper and paper [2], we discuss MAC operation for common functionalities related with HARQ operation. More specifally, this paper deals with a reliability enhancement feedback mode for NR SPS, while general HARQ aspects of NR SPS is provided in [2]. Some further SPS enhancements are in [3] and our views on grant-free are in [4].

# Discussion

For URLLC data both latency and reliability needs to be considered. Error cases that can be tolerated for eMBB operation can be severe for URLLC services. For example, RLC re-transmissions can reach reliability requirement, but the latency is too large for URLLC. Optimizations to resolve some error cases is therefore motivated.

SPS UL is highly relevant for URLLC service. The main reason is that very low UL latency can be enabled when the SR step is skipped. Supporting also reliable retransmissions and automatic repetitions may provide higher reliability on top of the low latency.

## Reliability-enhancement feedback mode

In the companion paper [2], we propose that if no retransmission UL-grant is received within the configurable time T, then UE considers the transmission successful and data buffer can be overwritten by a new transmission.

Note that it has been agreed that when the UE is configured with SPS, the UE should always skip UL transmission if there is no data. Hence, gNB does not expect a transmission in every resource specified by the SPS UL grant, and therefore the absence of a retransmission UL grant (i.e., no feedback) from the network can also imply that the transmission was not detected at all in gNB, or that UE missed the PDCCH.  See the following example.

Figure 1 Error cases in default HARQ feedback mode

Such error cases might be acceptable for eMBB data, since it can be recovered by higher layers at the cost of increased latency. But it is undesirable for data with high reliability requirement, such as URLLC data.

One mitigation mechanism is to enforce the transmission of a PDDCH message that indicates positive ACK feedback for each UL data transmission. Consequently, if no such PDDCH is received within the configurable feedback time T, then UE considers that the transmission is not successful and UE should retransmit the packet. Note that this does not preclude retransmission based on the UL grant (both dynamic UL grant and SPS grant) as proposed in paper [2].

This mode of feedback operation can be configured by the network. As one example, the UL grant with NDI not-toggled can be used to indicate positive ACK feedback on PDCCH.

The retransmission of the packet can be on the SPS UL resources. Since the frequency resources, MCS etc. are already configured the retransmission is done according to the SPS grant and is thereby non-adaptive. Moreover, since the initial transmission might be missed by the gNB we should keep the same redundant version for the retransmission. This is aligned with SPS UL operation in LTE.

A reliability-enhanced feedback mode is configurable for SPS UL

If the higher reliability is configured for the UE:

If no PDDCH indicate ACK or no UL grant for retransmisison is received within the time T, UE retransmits the packet on the first available SPS resource with the same RV as the initial transmission; if a PDDCH indicating ACK is received within the time T, then UE considers the transmission successful

The following two figures show an example of the above proposal with a retransmission scenario and a successful transmission, respectively.

Figure 2 Retransmisison scenario. Neither PDCCH-ACK nor UL-grant is received within time T

Figure 3 Succesful transmission scearnio. PDCCH-ACK is received within time T

This mode of feedback for SPS UL may lead to increased PDCCH load. For instance, ACK feedback would need to be sent for most of the UL transmissions since the BLER is low. However, this might be needed for packets that requires a very high reliability. In addition, as agreed in RAN2#98, “skipping SPS grant is mandated in NR regardless of SPS periodicity”. Thus, the total impact may be low if the traffic rate with this service requirement is low in general.

## HARQ for SPS with repetition

In the incoming LS [2], RAN1 also agree to support HARQ operations with repetitions. The main use case is data with ultra-low latency and high-reliabilty requirement. It can also potentially be used for coverage enhancement where the number of K is large.

The feedback mechanisms discussed above for SPS UL should be possible to configure also with repetition. Also the maximum feedback time T should function in the same way.

- Observation 1	A reliability-enhanced feedback mode including a maximum feedback time, should be configurable for SPS UL together with repetions.

Regarding the number of PDCCH messages to send as feedback, either the network sends one per repetition copy, or one for the entire sequence. In either case the HARQ process and the resource should be clear for the UE. This should be up to the network to decide.

- Observation 2	The network can send feedback for every repetition or per sequence.

# Conclusion

In section 2 we made the following observations:

Observation 1	A reliability-enhanced feedback mode including a maximum feedback time, should be configurable for SPS UL together with repetions.

Observation 2	The network can send feedback for every repetition or per sequence.

We made the following proposals:

Proposal 1	A reliability-enhanced feedback mode is configurable for SPS UL

Proposal 2	If no PDDCH indicate ACK or no UL grant for retransmisison is received within the time T, UE retransmits the packet on the first available SPS resource with the same RV as the initial transmission; if a PDDCH indicating ACK is received within the time T, then UE considers the transmission successful

# References

RAN2 Adhoc 2017 June, Qingdao, Chairman notes

R1-1711686, LS on NR UL SPS / UL transmission without UL grant

R2-1708352, General HARQ aspects of SPS UL

R2-1708351, SPS enhancements in NR

R2-1708350, Grant Free and Semi-Persistent Scheduling in NR