3GPP TSG-RAN WG2 Meeting #99bis		R2-1710803

Prague, Czech Republic, 9th - 13th October 2017

Title:	DRAFT Reply LS on NR Paging Occasion

Response to:	R1-1716918

Release:	Rel-15

Work Item:	NR_newRAT

Source:	MediaTek [to be RAN WG2]

To:	RAN WG1

Cc:

# Contact Person:

# Name:	             Li-Chuan TSENG

E-mail Address:	li-chuan (dot) tseng (at) mediatek (dot) com

1. Overall Description:

Q: Per RAN2 definition on Paging Occasion, are the Paging DCI and Paging Message sent in the same Paging Occasion or they can be sent in the different Paging Occasions.

The following agreement has been made in RAN2 NR#1 meeting:

- UE monitors one paging occasion per DRX cycle. Paging occasion is the time interval over which a paging message is transmitted by gNB.

Paging message is scheduled by Paging DCI carried by NR-PDCCH and is transmitted in the associated NR-PDSCH. Paging DCI and Paging Message should be sent in the same PO. Otherwise, the UE would need to monitor two POs (one for DCI and another for paging message) per DRX cycle, which violates the RAN2 agreement. Notice that this does not imply that Paging DCI and Paging Message must be consecutive.

# 2. Actions:

# To RAN WG1

# RAN2 would like ask RAN1 to take the above into account

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting 100	27 Nov – 1 Dec 2017 	Reno, USA

TSG RAN WG2 Meeting NR#2	22 Jan – 26 Jan 2018	USA