3GPP TSG-RAN WG2 #99bis	R2-1711133

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.4.1.5.3

Source:	Ericsson

Title:	Text proposal for TS 38.331 on SCG RLF

Document for:	Discussion, Decision

# Introduction

This document is a text proposal capturing the RAN2 agreements so far regarding the handling of SCG failure.

# Text proposal

============================== Begin Text Proposal ============================

#### 5.x.x	T304 expiry (synchronized SCG reconfiguration failure)

The UE shall:

1>	if T304 expires and it was started upon synchronized SCG reconfiguration initiation:

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicatedSCG, is not available for use by the UE anymore.

2>	initiate the SCG failure information procedure as specified in 5.x.x.x. to report SCG reconfiguration failure;

============================== End Text Proposal ============================

============================== Begin Text Proposal ============================

### 5x.x.	SCG failure information

#### 5.x.x.1	General

Figure 5.6.13.1-1: SCG failure information

The purpose of this procedure is to inform the NR network about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure.

#### 5.x.x..2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with 5.x.x; or

--FFS: RLF detection in NR

1>	upon SCG change failure, in accordance with 5.x.xa; or

1>	upon stopping uplink transmission towards the PSCell due to exceeding the maximum uplink transmission timing difference when powerControlMode is configured to 1, in accordance with subclause 7.x.x of TS 38.313 [xx].

Upon initiating the procedure, the UE shall:

1>	suspend all SCG transmission for all DRBs and SRBs;

1>	reset SCG-MAC;

1>	stop T304;

1>	initiate transmission of the SCGFailureInformation message in accordance with 5.x.x.3;

#### 5.x.x.3	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message to provide SCG radio link failure information:

2>	include failureType and set it to the trigger for detecting SCG radio link failure;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide SCG change failure information:

2>	include failureType and set it to scg-ChangeFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to exceeding maximum uplink transmission timing difference:

2>	include failureType and set it to maxUL-TimingDiff;

1> if the UE is operating in EN-DC:

2>	set the measResultServFreqList to include for each SCG cell that is configured by the SN to be measured, if any, within measResultSCell the quantities of the concerned SCell, if available, according to performance requirements in [xx];

2>	for each SCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

2>	set the measResultNeighCells to include the best measured cells on non-serving NR frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	if the UE was configured to perform measurements by the SN for one or more non-serving NR frequencies and measurement results are available, include the measResultListNR;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

2>	initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3 of TS 36.331

1>else:

2>	set the measResultServFreqList to include for each SCG cell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in [xx];

2>	for each SCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

2>	set the measResultNeighCells to include the best measured cells on non-serving NR frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	if the UE was configured to perform measurements for one or more non-serving NR frequencies and measurement results are available, include the measResultListNR;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

2> The UE shall submit the SCGFailureInformation message to lower layers for transmission

============================== End Text Proposal ============================

============================== Begin Text Proposal ============================

### 6.2.2	Message definitions

#### –	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to NR

SCGFailureInformation message

-- ASN1START

SCGFailureInformation ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformation-r15			SCGFailureInformation-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformation-r15-IEs ::=	SEQUENCE {

failureReportSCG-r15				FailureReportSCG-r15 			OPTIONAL,

nonCriticalExtension				SEQUENCE {}	OPTIONAL

}

-- Regular non-critical extensions:

FailureReportSCG-r15 ::= 			SEQUENCE {

failureType-r15						ENUMERATED {t313-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure, maxUL-TimingDiff  },

measResultServFreqList-r15			MeasResultServFreqList-r15		OPTIONAL,

measResultNeighCells-r15			MeasResultList2NR-r15			OPTIONAL,

...,

}

MeasResultList2NR-r15 ::=				SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR-r15

MeasResult2NR-r15 ::=				SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR,

measResultList-r15					MeasResultListNR

}

-- ASN1STOP

============================== End Text Proposal ============================

============================== Begin Text Proposal ============================

### 6.3.5	Measurement information elements

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency and inter- RAT mobility.

MeasResults information element

-- ASN1START

MeasResults ::=						SEQUENCE {

measId								MeasId,

measResultServingFreqList			MeasResultServFreqList,

measResultNeighCells				CHOICE {

measResultListNR					MeasResultListNR

}																	OPTIONAL

}

MeasResultServFreqList ::=	SEQUENCE (SIZE (1..maxServCell)) OF MeasResultServFreq

MeasResultServFreq ::=				SEQUENCE {

servFreqId							ServCellIndex		OPTIONAL,

measResultServingCell				MeasResultNR,

measResultBestNeighServingCell		MeasResultNR		OPTIONAL

}

MeasResultListNR ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=	SEQUENCE {

physCellId							PhysCellId							OPTIONAL,

--FFS: Details of cgi info

cgi-Info							TYPE_FFS!

measResult							SEQUENCE {

cellResults			SEQUENCE{

resultsSSBCell						ResultsSSBCell					OPTIONAL,

resultsCSI-RSCell 					ResultsCSI-RSCell				OPTIONAL

},

rsIndexResults		SEQUENCE{

resultsSSBIndexes					ResultsPerSSBIndexList				OPTIONAL,

resultsCSI-RSIndexes				ResultsPerCSI-RSIndexList 			OPTIONAL			}																	OPTIONAL

}

}

ResultsSSBCell ::= SEQUENCE {

ssb-Cellrsrp		RSRP-Range	OPTIONAL,

ssb-Cellrsrq		RSRQ-Range	OPTIONAL,

ssb-Cellsinr		SINR-Range	OPTIONAL

}

ResultsCSI-RSCell ::= SEQUENCE {

csi-rs-Cellrsrp		RSRP-Range	OPTIONAL,

csi-rs-Cellrsrq		RSRQ-Range	OPTIONAL,

csi-rs-Cellsinr		SINR-Range	OPTIONAL

}

ResultsPerSSBIndexList::= SEQUENCE (SIZE (1..maxNroSSBs)) OF ResultsPerSSBIndex

ResultsPerSSBIndex ::= SEQUENCE {

ssbIndex			SSBIndex,

ss-rsrp				RSRP-Range	OPTIONAL,

ss-rsrq				RSRQ-Range	OPTIONAL,

ss-sinr				SINR-Range	OPTIONAL

}

resultsPerCSI-RSIndexList::= SEQUENCE (SIZE (1..maxNroCSI-RS)) OF ResultsPerCSI-RSIndex

ResultsPerCSI-RSIndex ::= SEQUENCE {

csi-rsIndex				CSI-RSIndex,

csi-rsrp				RSRP-Range	OPTIONAL,

csi-rsrq				RSRQ-Range	OPTIONAL,

csi-sinr				SINR-Range	OPTIONAL

}

-- ASN1STOP

MeasResults field descriptions

csi-rs-Cellrsrp Measured RSRP result per NR cell based on CSI-RSRP value(s) from the L1 filter(s).

csi-rs-Cellrsrq Measured RSRQ result per NR cell based on CSI-RSRQ value(s) from the L1 filter(s).

csi-rs-Cellsinr Measured SINR result per NR cell based on CSI-SINR value(s) from the L1 filter(s).

csi-rsIndex CSI-RS resource index associated to the measurement information to be reported.

csi-rsrp L3 filtered CSI-RSRP measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-RSRP is defined in TS 38.215 [x].

csi-rsrq L3 filtered CSI-RSRQ measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-RSRQ is defined in TS 38.215 [x].

csi-sinr L3 filtered CSI-SINR measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-SINR is defined in TS 38.215 [x].

measId Identifies the measurement identity for which the reporting is being performed.

measResult Measured results of an NR cell.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultServingFreqList Measured results of the serving frequencies including measurement results of PCell, configured SCell(s) and best neighbouring cell on each serving frequency.

resultsCSI-RSIndexes List of measurement information per CSI-RS resource index of an NR cell.

resultsSSBIndexes List of measurement information per SS/PBCH index of an NR cell.

resultsCSI-RSCell Cell level measurement results (e.g. RSRP, RSRQ, SINR) to be reported derived from CSI-RS measurements.

resultSSBCell Cell level measurement results (e.g. RSRP, RSRQ, SINR) to be reported derived on SS/PBCH block measurements.

ssb-Cellrsrp Measured RSRP result per NR cell based on SS-RSRP value(s) from the L1 filter(s).

ssb-Cellrsrq Measured RSRQ result of an NR Cell based on SS-RSRP value(s) from the L1 filter(s).

ssb-Cellsinr Measured SS-SINR result of an NR Cell based on SS-SINR value(s) from the L1 filter(s)..

ssbIndex SS/PBCH block index associated to the measurement information to be reported.

ss-rsrp L3 filtered SS-RSRP measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-RSRP is defined in TS 38.215 [x].

ss-rsrq L3 filtered SS-RSRQ measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-RSRQ is defined in TS 38.215 [x].

ss-sinr L3 filtered SS-SINR measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-SINR is defined in TS 38.215 [x].

============================== End Text Proposal ============================