3GPP TSG-RAN WG2 NR #99bis Meeting                                            	R2-1711188

Prague, Czech, 9st-13th October 2017

Agenda item:	10.2.3

Source:	Samsung

Title:	Signaling to support bandwidth part

Document for:	Discussion & Decision

# 1. Introduction

RAN#75 in March approved a 5G WID [1] on New Radio access technology, which targets a unified framework for traffic with diverse QoS requirements e.g. enhanced mobile broadband (eMBB), massive machine-type-communications (mMTC), ultra-reliable and low latency communications (URLLC).

During NR SI/WI phases for Rel-15, the following agreements were achieved regarding bandwidth part (BWP) for NR

RAN1 Agreement (#88bis)

- One or multiple bandwidth part configurations for each component carrier can be semi-statically signalled to a UE

  - A bandwidth part consists of a group of contiguous PRBs

    - Reserved resources can be configured within the bandwidth part

  - The bandwidth of a bandwidth part equals to or is smaller than the maximal bandwidth capability supported by a UE

  - The bandwidth of a bandwidth part is at least as large as the SS block bandwidth

    - The bandwidth part may or may not contain the SS block

  - Configuration of a bandwidth part may include the following properties

    - Numerology

    - Frequency location (e.g. center frequency)

    - Bandwidth (e.g. number of PRBs)

  - Note that it is for RRC connected mode UE

  - FFS how to indicate to the UE which bandwidth part configuration (if multiple) should be assumed for resource allocation at a given time

  - FFS neighbour cell RRM

8.1 Numerologies and frame structure from TR 38.912

At least for single carrier operation, NR should allow a UE to operate in a way where it receives at least downlink control information in a first RF bandwidth and where the UE is not expected to receive downlink control information or data in a second RF bandwidth that is larger than the first RF bandwidth within less than X µs.

RAN1 Agreements (#89)

- Confirm the WA of RAN1#88bis.

- Each bandwidth part is associated with a specific numerology (sub-carrier spacing, CP type)

  - FFS: slot duration indication if RAN1 decides to not to downselect between 7 symbol and 14 symbols for NR slot duration

- UE expects at least one DL bandwidth part and one UL bandwidth part being active among the set of configured bandwidth parts for a given time instant.

  - A UE is only assumed to receive/transmit within active DL/UL bandwidth part(s) using the associated numerology

    - At least PDSCH and/or PDCCH for DL and PUCCH and/or PUSCH for UL

      - FFS: down selection of combinations

  - FFS if multiple bandwidth parts with same or different numerologies can be active for a UE simultaneously

    - It does not imply that it is required for UE to support different numerologies at the same instance.

    - FFS: TB to bandwidth part mapping

- The active DL/UL bandwidth part is not assumed to span a frequency range larger than the DL/UL bandwidth capability of the UE in a component carrier.

- Specify necessary mechanism to enable UE RF retuning for bandwidth part switching

- In case of one active DL BWP for a given time instant,

  - Configuration of a DL bandwidth part includes at least one CORESET.

  - A UE can assume that PDSCH and corresponding PDCCH (PDCCH carrying scheduling assignment for the PDSCH) are transmitted within the same BWP if PDSCH transmission starts no later than K symbols after the end of the PDCCH transmission.

  - In case of PDSCH transmission starting more than K symbols after the end of the corresponding PDCCH, PDCCH and PDSCH may be transmitted in different BWPs

  - FFS: Value of K (may depend on numerology, possibly reported UE retuning time, etc)

RAN1 Agreements (AH#2)

- At least one of configured DL BWPs includes one CORESET with common search space at least in primary component carrier

- Each configured DL BWP includes at least one CORESET with UE-specific search space for the case of single active BWP at a given time

  - In case of single active BWP at a given time, if active DL BWP does not include common search space, then UE is not required to monitor the common search space

- Activation/deactivation of DL and UL bandwidth parts can be

  - by means of dedicated RRC signaling

    - Possibility to activate in the bandwidth part configuration

  - by means of DCI (explicitly and/or implicitly) or MAC CE [one to be selected]

    - by means of DCI could mean

      - Explicit: Indication in DCI (FFS: scheduling assignment/grant or a separate DCI) triggers activation/deactivation

        - Separate DCI means DCI not carrying scheduling assignment/grant

      - Implicit: Presence of DCI (scheduling assignment/grant) in itself triggers activation/deactivation

      - This does not imply that all these alternatives are to be supported.

  - FFS: by means of timer

  - FFS: according to configured time pattern

RAN1 Agreements (#90)

- There is an initial active DL/UL bandwidth part pair to be valid for a UE until the UE is explicitly (re)configured with bandwidth part(s) during or after RRC connection is established

  - The initial active DL/UL bandwidth part is confined within the UE minimum bandwidth for the given frequency band

  - FFS: details of initial active DL/UL bandwidth part are discussed in initial access agenda

- Support activation/deactivation of DL and UL bandwidth part by explicit indication at least in (FFS: scheduling) DCI

  - FFS: In addition, MAC CE based approach is supported

- Support activation/deactivation of DL bandwidth part by means of timer for a UE to switch its active DL bandwidth part to a default DL bandwidth part

  - The default DL bandwidth part can be the initial active DL bandwidth part defined above

  - FFS: The default DL bandwidth part can be reconfigured by the network

  - FFS: detailed mechanism of timer-based solution (e.g. introducing a new timer or reusing DRX timer)

  - FFS: other conditions to switch to default DL bandwidth part

- When a UE performs measurement or transmit SRS outside of its active BWP, it is considered as a measurement gap

  - FFS: details of measurement gap configuration

  - During the measurement gap, UE is not expected to monitor CORESET

This contribution investigates design considerations and framework to support wider BW in NR; especially about flexible BW support based on bandwidth part.

# 2. 	Design Issues for Bandwidth Part

  - Procedures/signaling to support BWP

RAN1 agreed on BWP which can be configured to a UE semi-statically for RRC connected mode UE. BWP is a group of contiguous PRBs and UE is not required to receive any DL signal outside of the active BWP. The motivation of BWP is to support wider BW compared to maximum BW capability of UE. The configured BWP is regarded as the operating BW of the UE at the instance. Network can configure multiple different BWPs to RRC connected UEs to utilize wider BW efficiently. From UE side, at least one BWP should be configured and multiple BWPs can be configured by network decision based on UE BW capability, and measurement result. Multiple BWPs configuration may be useful for BW adaptation to save UE power consumption or for flexible scheduling mechanism in single CC.

Figure 1: The concept of bandwidth part

According to the configuration of BWP, the UE understands the PHY channel structure and other related parameters based on indication in the BWP configuration. For clarity, it is better to use separate terms as DL BWP and UL BWP. RAN1 agreed on initial active DL/UL BWP pair which is valid before explicit (re)configuration of BWP during or after RRC connection establishment procedure. Unfortunately RAN1 didn’t make any agreement no FFS part on details of initial active DL/UL BWP. So I would like to RAN2 leads this discussion.

- There is an initial active DL/UL bandwidth part pair to be valid for a UE until the UE is explicitly (re)configured with bandwidth part(s) during or after RRC connection is established - The initial active DL/UL bandwidth part is confined within the UE minimum bandwidth for the given frequency band - FFS: details of initial active DL/UL bandwidth part are discussed in initial access agenda

UE performs idle mode operation such as RMSI reception, and RA procedure via the initial active DL/UL BWP pair. Even though there is a flexibility to have different frequency resources for RMSI and each message in RA procedure, we prefer to have a common initial DL/UL BWP pair during initial access. The frequency resources for RMSI and RA should be confined in the initial DL/UL BWP pair. The initial DL/UL BWP pair can be configured via MSI.

Proposal 1: The initial active DL/UL BWP configuration is signaled to UE in MSI for initial access.

Network can determine whether or not to keep the initial DL/UL BWP as one of configured BWPs for Connected mode UE. During RRC connection establishment procedure, network can configure at least a pair of DL/UL BWP which may be same or not as the initial DL/UL BWP pair. If new DL/UL BWP pair is not configured explicitly in RRC connection establishment procedure, the initial DL/UL BWP pair is inherited implicitly as a pair of DL/UL BWP for Connected mode. If needed, network can reconfigure DL or UL BWP individually.

Observation 1: Initial DL/UL BWP pair for initial access can be inherited or reconfigured to a DL/UL BWP pair for Connected mode up to network decision.

After UE establishes SRB1 via RRC connection establishment procedure, UE may deliver UECapabilityInformation with BWP-related RF information to network upon UECapabilityEnquery for initial attach. Once the BWP-related information RF information is stored in MME, network may configure one or more BWPs without enquiry. The configuration information for dedicated BWP can be included in rrcConnectionReconfiguration like message, which is sent after UE capability information is received by gNB. UE can be configured with multiple BWPs (for the purpose of load balancing or adaptation between BWPs).

Proposal 2: rrcConnectionReconfiguration like message may contain configuration information for multiple BWPs.

BWP is typically configured to PCell, but may be configured to SCell as well, since SCell can have a wide CC for the purpose of data offloading. BWP configuration may be contained in radioResourceConfigDedicated IE for PCell or in radioResourceConfigDedicatedSCell IE for SCell. These IEs may be contained in newly defined bwptoAddModList IE.

Proposal 3: For BWP configuration, bwp-toAddModList IE is defined.

Proposal 4: For BWP configuration in PCell, RAN2 assumes radioResourceConfigDedicated IE in bwpToAddModList as baseline.

Proposal 5: For initial BWP configuration in SCell, RAN2 assumes  radioResourceConfigDedicatedSCell in sCelltoAddModList as baseline.

Proposal 6: For BWP configuration in SCell, RAN2 assumes radioResourceConfigDedicated IE in bwpToAddModList as baseline.

UE can be configured with multiple BWPs. However, additional activation by RRC signaling is not needed at least in Rel-15, since there is at most one active DL BWP and at most one UL BWP for a UE at a given time for a serving cell. So all configured BWPs for serving cell except the initial active DL/UL BWP are deactivated at initial state.

Proposal 7: All configured BWPs for serving cell except initial active DL/UL BWP is deactivated at initial state.

For handover operation, an active DL/UL BWP pair should be configured to UE for target cell, since UE does not acquire SI from the target cell before handover complete.

Proposal 8: An active DL/UL BWP is configured to UE in rrcConnectionReconfiguration like message for handover.

Figure 4: Bandwidth part-based scheduling

Scheduling is one of time-stringent functions since it performs every scheduling opportunity and is sensitive to latency from the capacity and service requirement perspectives. In case of scheduling, as shown in Figure 2, the indication of BWP change could be realized by indication via control channel at one BWP for data transmission at another BWP. From the analogy to cross-carrier scheduling, we call indication of BWP change for scheduling by cross-bandwidth part scheduling or shortly cross-BWP scheduling. Cross-BWP scheduling may be performed in the same RF BW or different RF BWs. For proper scheduling by network, UE should provide UE capability information including feature and number of RF BW. For details on resource indication for self/cross-BWP scheduling, please refer to the companion paper [2].

On the other hand, some UE may not be capable to operate with low latency during RF retuning. If such limited capable UE is scheduled across BWP, the scheduling performance will be severely degraded due to higher overhead for every indication for scheduling. So it seems natural to support self-bandwidth part scheduling. Since both self- and cross- BWP scheduling should be supported, the identification of BWP is needed to indicate in which BWP, data is delivered corresponding to scheduling assignment. For the indication, DCI contains at least the index of BWP. In addition, symbol/slot index for indicate TB may be contained in DCI.

Observation 2: RAN1 agreed on both self-BWP and cross-BWP scheduling.

Proposal 9: BWP is configured with BWPIndex by RRC so that the index is contained in DCI or RRC message to support cross-BWP scheduling and reconfiguration of BWP.

According to the following RAN1 agreements,

- At least one of configured DL BWPs includes one CORESET with common search space at least in primary component carrier - Each configured DL BWP includes at least one CORESET with UE-specific search space for the case of single active BWP at a given time - In case of single active BWP at a given time, if active DL BWP does not include common search space, then UE is not required to monitor the common search space - One or multiple bandwidth part configurations for each component carrier can be semi-statically signalled to a UE - A bandwidth part consists of a group of contiguous PRBs - Reserved resources can be configured within the bandwidth part - The bandwidth of a bandwidth part equals to or is smaller than the maximal bandwidth capability supported by a UE - The bandwidth of a bandwidth part is at least as large as the SS block bandwidth - The bandwidth part may or may not contain the SS block - Configuration of a bandwidth part may include the following properties - Numerology - Frequency location (e.g. center frequency) - Bandwidth (e.g. number of PRBs) - Note that it is for RRC connected mode UE - FFS how to indicate to the UE which bandwidth part configuration (if multiple) should be assumed for resource allocation at a given time - FFS neighbour cell RRM

In addition to BWPIndex, the configuration information of BWP shall include numerology, PRB offset, bandwidth (number of PRBs) and CORESET configuration. In addition, information of direction i.e. DL or UL shall be included since RAN1 agreed that DL BWP and UL BWP are individually configured and be distinguished when UE receives cross-BWP scheduling DCI or RRC Connection Reconfiguration message.

Proposal 10: The configuration information of a BWP includes at least the followings:

- BWPIndex

- numerology,

- PRB offset,

- number of PRBs,

- CORESET configuration

- link direction

# 3. Summary

Based on the discussion and identified observations in this contribution, we propose followings:

Observation 1: Initial DL/UL BWP pair for initial access can be inherited or reconfigured to a DL/UL BWP pair for Connected mode up to network decision.

Observation 2: RAN1 agreed on both self-BWP and cross-BWP scheduling.

Proposal 1: The initial active DL/UL BWP configuration is signaled to UE in MSI for initial access.

Proposal 2: rrcConnectionReconfiguration like message may contain configuration information for multiple BWPs.

Proposal 3: For BWP configuration, bwp-toAddModList IE is defined.

Proposal 4: For BWP configuration in PCell, RAN2 assumes radioResourceConfigDedicated IE in bwpToAddModList as baseline.

Proposal 5: For initial BWP configuration in SCell, RAN2 assumes  radioResourceConfigDedicatedSCell in sCelltoAddModList as baseline.

Proposal 6: For BWP configuration in SCell, RAN2 assumes radioResourceConfigDedicated IE in bwpToAddModList as baseline.

Proposal 7: All configured BWPs for serving cell except initial active DL/UL BWP is deactivated at initial state.

Proposal 8: An active DL/UL BWP is configured to UE in rrcConnectionReconfiguration like message for handover.

Proposal 9: BWP is configured with BWPIndex by RRC so that the index is contained in DCI or RRC message to support cross-BWP scheduling and reconfiguration of BWP.

Proposal 10: The configuration information of a BWP includes at least the followings:

- BWPIndex

- numerology,

- PRB offset,

- number of PRBs,

- CORESET configuration

- link direction

References

[1] RP-170823, New WID Proposal: Work Item on New Radio (NR) Access Technology

[2] R1-1717675, “On Bandwidth Part Operation”, Samsung, October 2017