3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711253

Prague, Czech Republic, 9th – 13th October 2017	update of R2-1708352

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Remaining issues on SPS UL

Document for:	Discussion, Decision

# Introduction

In RAN2-99, the following agreements were made in the chairman’s notes for SPS [1]:

As in LTE SPS UL, retransmission for SPS UL transmission are based only on UL dynamic grant

Further, RAN1 has sent an LS [2] on the progress of the UL-grant free transmissions and the latest ongoing discussion in RAN1 is summarized in [6]. It is common understanding that type-2 grant-free corresponds to SPS. Given the slow progress in RAN1 discussion, in this paper and paper [3], we discuss MAC operation for common functionalities. This paper deals with remaining issues of NR SPS UL, while a reliability enhancement feedback mode for NR SPS UL is provided in [3]. Discussions of SPS on Scell are in [4] and our views on grant-free versus SPS (i.e., type 1 versus type 2) are in [5].

# Discussion

## Baseline operations

In the LS from RAN1,

- At least when an UL grant is used for retransmissions of Type 1 UL transmission without UL grant, different RNTI from the RNTI for UL transmission with grant is needed. - FFS how to determine the RNTI. - For Type 2 UL transmission without UL grant, different RNTI from the RNTI for UL transmission with grant is needed for activation/deactivation and at least for re-transmission. - FFS how to determine the RNTI.

In addition, we have agreed that the retransmission can be based on UL-grant.

As in LTE-SPS, retransmissions for SPS UL transmission are based an uplink grant received on SPS C-RNTI.

In the LS from RAN1,

- Support using MAC CE as an acknowledgement for L1 signalling for activation/deactivation of Type 2 UL transmission without grant (similar/same behaviour as in LTE SPS).

Since MAC CE is defined in RAN2, we propose that

As in LTE-SPS, use MAC CE for confirmation of UL activation/deactivation

## HARQ feedback

The following HARQ feedback options are mentioned in the LS from RAN1.

- If HARQ feedback is supported, to indicate HARQ feedback of UL transmission without grant, following options and related UE behavior should be further studied. - Option 1: Based on UL grant to indicate “ACK” - Option 2: Group-common DCI - 2-1: Only ACK - 2-2: ACK and NACK - Option 3: Define a Timer, UE assumes following, when the Timer expires - 3-1: ACK if an NACK is not received after the K repetitions - 3-2: NACK if an ACK is not received - FFS: Option 1, Option 2 and Option 3-2 can be used during and after the K repetition - Note: UL grant for the same TB initially transmitted without grant can indicate “NACK”

Here we provide our views from RAN2, that indepent from the discussion outcome in RAN1, at least option 3-1 and 3-2 should be supported. Other detailed discussions on what DCI format to carry ACK or NACK are left to RAN1.

In LTE, a fixed timing (n+4) is configured so that UE knows implicitly when the feedback on PHICH will be sent and proper actions are taken depending on the feedback in PHICH. If ACK is sent on PHICH, then the UE can overwrite the buffer and transmit new data on SPS UL. In NR, unless e.g. a PDCCH indicating ACK is sent (both option 1 and option 2-1 in the LS from RAN1), UE keeps data in the buffer and no new data can be sent in SPS UL with the same HARQ process ID. Even with option 2-2 where both ACK and NACK can be sent, there should be a decisive moment when the UE should assume that nothing is going to be received in the group common DCI. In summary, no matter what oucome will be in RAN1, a PDCCH-based feedback has been agreed if HARQ feedback is supported. But, it becomes uncertain when such a feedback is expected.

From RAN1 LS, HARQ feedback (if supported) is PDCCH-based, but it is uncertain when feedback will be transmitted from gNB.

It is stated that the UL grant for the same TB initially transmitted without grant can indicate “NACK”, but it is also unclear when such an UL-grant will be transmitted from gNB.

It is uncertain when an UL grant for re-retransmission will be transmitted by gNB.

In NR SPS UL, as one straightforward solution, UE can keep the data in the buffer after the transmission and use other HARQ process IDs for new data transmsion. Once this HARQ process is configured to be transmitted again and all other HARQ process IDs are in-use, a default interpretation of the previous transmission result is assumed when neither PDCCH-based HARQ feedback nor UL-grant for re-transmission has been received.

For example, if the UE is configured to receive only UL-grant for re-transmission, then the data buffer for this HARQ process is flushed for new data when all HARQ process IDs are exhausted. However, this puts a limitation that the UL-grant for retransmission needs to be sent before all HARQ process IDs are exhausted. One example is shown in the following figure. There is a time limit before gNB is too late to send the UL-grant. This time is roughly:

# HARQ process x SPS periodicity – fastest UE processing time.

A proper time margin should be given to gNB to process, and this works fine as long as the SPS periodicity is long or the number of the HARQ process is large. But, such a limitation is not desired as SPS can be configured for URLLC with very short peridocity in time by mini-slot and higher numerology and with a small number of HARQ processes. As one example, we can configure only one HARQ process with a small periodicity for the UE. This is to target the use case with infrequent and small data transmission in URLLC. In other example with dynamic TDD, in a pattern with UL-frame in most of the time, the allocated HARQ processes can quickly run-out.

It is not flexible to regard the previous UL transmission successful if no UL-grant is received before the next configured grant appears.

Since the following scheme relies on a large number of SPS processes and long SPS periodicity, it is not flexbile:

    - Consider transmission successful if no retransmission UL grant is received before next configured UL grant for the same HARQ process.

As a more flexbile and forward-compatible solution, a timer for HARQ feedback or UL-grant can be set after a transmission or retransmission, counting down from the time T. The detailed feedback to monitor during the time T can wait for RAN1 discussion outcome.

One option is that a maximum time T is explicitly configured for the UE to wait just for an UL grant for retransmission. This means that the UE keeps the data in the buffer, and waits for a retransmission UL grant during a maximum time T. If no UL grant is received with the same HARQ process ID when the timer expires, the UE assumes that the data is successfully delivered and it can reuse the HARQ process for new data.

This option is in-align with the UL dynamic scheduling procedure, does not require HARQ feedback and has low PDCCH load.

In SPS UL, a time T after an UL transmission on a HARQ process is configured to wait for a retransmission UL-grant for the same HARQ process.

If no retransmissions UL grant is received within time T, then UE considers the transmission successful and the HARQ process is overwritten by new data.

This proposal implies that if the UE receives a UL grant while timer T is running, it shall perform a retransmission. This corresponds to option 3-1 in the RAN1 LS. See the following figure as an example.

Figure 1  Illustration of time T

In addition, we can specify to start monitoring after the HARQ RTT timer fire offs.

In SPS UL, UE start to wait for a retransmission UL-grant after HARQ RTT timer.

In LTE, "drx-ULRetransmissionTimer (one per asynchronous UL HARQ process)" specifies the maximum number of consecutive PDCCH-subframe(s) until a grant for UL retransmission is received. After the timeout, UE goes to DRX state, and the consideration is more on the trade-off between gNB scheduling flexibility and UE energy consumption, but latency is not considered. For SPS UL, we have to consider the latency of MAC PDU associated with current HARQ process, and thus it is more convenient to have a different value.

“DRX-ULRetransmissionTimer” in LTE also specifies the time to monitor UL-grant, but it specifies a different behavior after timeout and a different timer value is needed.

Lastly, a maximum time T can also be configured to wait for the positive-feedback (if HARQ-feedback is supported in option 1, 2-1 and 2-2). In this case, if no PDDCH indicate ACK or no UL grant for retransmisison is received within the time T, UE retransmits the packet on the first available SPS resource with the same RV as the initial transmission. Details are presented in the companion paper [3], and this corresponds to option 3-2 in the RAN1 LS.

## Repetition

It has been agreed in RAN1 to support HARQ operations with repetitions. However, it has not been clear which type of repetition is supported. This is still under-discussion in RAN1, see proposal 6 in [6].

One example is the following, where the total time duration of the repetition lasts shorter than the period of the SPS. This is useful for mMTC service, and we can re-use as much as the schemes developed there. In this example, three repetitions can be configured within one SPS period.

However, the above approach is not latency efficient. It can only be used for periodic traffic, such as 5 ms periodic traffic with URLLC requirement (i.e., 1 ms latency budget). In addition, we need a perfect alignment of offset and periodicity.

The same TB can be repeated on multiple SPS resorucse within one period.

Another repetition scheme is where the repetition can be performed on the next periodic appearing SPS resources, see the following figure as an example. Such a scheme is possible, since in NR we can configure SPS with very short period.

This is more delay-efficient. As one example illustrated in the below figure, we can start at any time to reduce alignment delay.

The same TB can be repeated on SPS resources across multiple periods.

Nevertheless, this is more complicated to support HARQ Process ID and redudancy version configurations, see our contributions in the last meeting [7].

Because the discussions are still on-going in RAN1, we propose that

Wait for RAN1’s progress on repetition.

# Conclusion

For remaining and general HARQ aspects of SPS UL in NR, we made the following observation:

Observation 1	From RAN1 LS, HARQ feedback (if supported) is PDCCH-based, but it is uncertain when feedback will be transmitted from gNB.

Observation 2	It is uncertain when an UL grant for re-retransmission will be transmitted by gNB.

Observation 3	Since the following scheme relies on a large number of SPS processes and long SPS periodicity, it is not flexbile:

i.	Consider transmission successful if no retransmission UL grant is received before next configured UL grant for the same HARQ process.

Observation 4	In SPS UL, UE start to wait for a retransmission UL-grant after HARQ RTT timer.

Observation 5	“DRX-ULRetransmissionTimer” in LTE also specifies the time to monitor UL-grant, but it specifies a different behavior after timeout and a different timer value is needed.

Observation 6	The same TB can be repeated on multiple SPS resorucse within one period.

Observation 7	The same TB can be repeated on SPS resources across multiple periods.

We propose the following:

Proposal 1	As in LTE-SPS, retransmissions for SPS UL transmission are based an uplink grant received on SPS C-RNTI.

Proposal 2	As in LTE-SPS, use MAC CE for confirmation of UL activation/deactivation

Proposal 3	In SPS UL, a time T after an UL transmission on a HARQ process is configured to wait for a retransmission UL-grant for the same HARQ process.

Proposal 4	If no retransmissions UL grant is received within time T, then UE considers the transmission successful and the HARQ process is overwritten by new data.

Proposal 5	Wait for RAN1’s progress on repetition.

# References

RAN2-99, 2017 Aug, Berlin, Chairman notes

R1-1714995, LS on NR UL transmission without UL grant

R2-1711254, Enhanced HARQ feedback mode in SPS, Ericsson

R2-1711252, SPS for Scell, Ericsson

R2-1711251, Modelling of SPS/Grant Free Scheme in NR, Ericsson

R1-1716851, Offline summary for UL transmission procedures, NTT DoCoMo

R2-1708352, General HARQ aspects of SPS UL, Ericsson