3GPP TSG-RAN WG2 Meeting #99bis 	R2-1711285

Prague, Czech Republic, 9 - 13 October 2017

Agenda item:	10.2.15

Source:	Nokia, Nokia Shanghai Bell

Title:	Control of the frequency on which the UE camps

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

Network Slicing technical realization with UE assistance has been recognized by SA2, RAN3 and RAN2. In this context, RAN2 #99 meeting agreed the following:

Agreements:

Working assumption: For needs of slicing, appropriate configuration of the dedicated priorities provided from the gNB can be used to control the frequency on which the UE camps. (i.e. reuse of same mechanism as in LTE). (To be checked whether the gNB has knowledge of all the slices to which the UE is registered)

FFS: Whether any additional mechanisms are to be introduced for control of the frequency on which the UE camps

This contribution provides an overview of latest status and conclusions related to the procedure of selecting frequencies the UE camps on in Network Slicing and proposes to progress the RAN2 work.

# 2	Discussion

## 2.1	Architectural assumptions on Network Slicing

TS23.501 v1.4.0 [1] defines:

A single UE can simultaneously be served by one or more Network Slice instances via a 5G-AN. (…)

The selection of a Network Slice instance(s) serving a UE and the Core Network Control Plane and user plane Network Functions corresponding to the Network Slice instance is the responsibility of 5GC.

The (R)AN may use Requested NSSAI in access stratum signalling to handle the UE Control Plane connection before the 5GC informs the (R)AN of the Allowed NSSAI. The Requested NSSAI is not used by the RAN for routing when the UE provides also a Temporary User ID.

When a UE is successfully registered, the CN informs the (R)AN by providing the whole Allowed NSSAI for the Control Plane aspects.

Furthermore, more detailed implementation of the signalling between 5GC and 5G-AN defined in the latest 38.413 [2] reads:

For each PDU session for which the S-NSSAI IE is included in the PDU SESSION RESOURCE SETUP REQUEST message the NG-RAN node shall store the Network Slice Selection Assistance Information corresponding to the concerned PDU session and use it as specified in TS 38.300 [8].

Observation 1: gNB has the knowledge of the slices the UE is registered to.

## 2.2	What information is needed for dedicated priority configuration

RAN2 working assumption made in RAN2#99 relies on the function that the UE receives in RRCConnectionRelease message a dedicated configuration (e.g idleModeMobilityControlInfo or similar). The dedicated configuration defines the dedicated priorities provided from the gNB that can be used to control the frequency on which the UE camps.

The last serving gNB knows the slices of the UE based on:

- Slice assistance information which the UE has used in the last serving gNB

- Slice assistance information to which the UE has registered to (could be received in the UE context).

  - These slices are supported also in the last serving gNB.

- Slice assistance information which to which the UE has subscription (could be received in the UE context from AMF).

  - These slices could be also slices not supported in the last serving gNB.

UE may receive:

- priority information with a carrier,

- priority information with S-NSSAI information mapped to a carrier(s)

Observation 2: It is feasible for gNB to use RRCConnectionRelease message for a slice configuration.

# There are various requirements towards handling of slice configuration. I.e. The slice configuration (i.e. set of available slices) for a UE remains the same within the Registration Area. NG-RAN needs to also support a differentiated handling of traffic for different network slices which have been pre-configured. At the same time the UE respects priority based cell reselection according to the agreed framework in 38.300 [3] on Support for UE associating with multiple network slices simultaneously.

How NG-RAN supports the slice enabling in terms of NG-RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent [3].

Observation 3: It is agreed framework that dedicated priorities can be used to control the frequency on which the UE camps.

Proposal 1: gNB uses RRCConnectionRelease message to set dedicated priorities to control the frequency on which the UE camps.

# 3	Conclusion

We conclude the contribution with the following observations and proposals to be agreed:

Observation 1: gNB has the knowledge of the slices the UE is registered to.

Observation 2: It is feasible for gNB to use RRCConnectionRelease message for a slice configuration.

Observation 3: It is agreed framework that dedicated priorities can be used to control the frequency on which the UE camps.

Proposal 1: gNB uses RRCConnectionRelease message to set dedicated priorities to control the frequency on which the UE camps.

# Annex:Text Proposal to 38.300

Modified Subclause

## 7.3	Network Slicing

### 17.3.1	General Principles and Requirements

In this sub clause, the general principles and requirements related to the realization of network slicing in the NG-RAN for NR connected to 5GC are given.

A network slice always consists of a RAN part and a CN part. The support of network slicing relies on the principle that traffic for different slices is handled by different PDU sessions. Network can realise the different network slices by scheduling and also by providing different L1/L2 configurations. UE should be able to provide assistance information for network slice selection in RRC message, if it has been provided by NAS. While the network can support large number of slices (hundreds), the UE need not support more than 8 slices in parallel.

Network Slicing is a concept to allow differentiated treatment depending on each customer requirements. With slicing, it is possible for Mobile Network Operators (MNO) to consider customers as belonging to different tenant types with each having different service requirements that govern in terms of what slice types each tenant is eligible to use based on Service Level Agreement (SLA) and subscriptions.

NSSAI (Network Slice Selection Assistance Information) includes one or more S-NSSAIs (Single NSSAI). Each network slice is uniquely identified by a S-NSSAI, as defined in TS 23.501 [3].

The following key principles apply for support of Network Slicing in NG-RAN:

# RAN awareness of slices

-	NG-RAN supports a differentiated handling of traffic for different network slices which have been pre-configured. How NG-RAN supports the slice enabling in terms of NG-RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent.

# Selection of RAN part of the network slice

-	NG-RAN supports the selection of the RAN part of the network slice, by assistance information provided by the UE or the 5GC which unambiguously identifies one or more of the pre-configured network slices in the PLMN.

# Resource management between slices

-	NG-RAN supports policy enforcement between slices as per service level agreements. It should be possible for a single NG-RAN node to support multiple slices. The NG-RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.

# Support of QoS

-	NG-RAN supports QoS differentiation within a slice.

# RAN selection of CN entity

-	For initial attach, the UE may provide assistance information to support the selection of an AMF. If available, NG-RAN uses this information for routing the initial NAS to an AMF. If the NG-RAN is unable to select an AMF using this information or the UE does not provide any such information the NG-RAN sends the NAS signalling to a default AMF.

-	For subsequent accesses, the UE provides a Temp ID, which is assigned to the UE by the 5GC, to enable the NG-RAN to route the NAS message to the appropriate AMF as long as the Temp ID is valid (NG-RAN is aware of and can reach the AMF which is associated with the Temp ID). Otherwise, the methods for initial attach applies.

# Resource isolation between slices

-	the NG-RAN supports resource isolation between slices. NG-RAN resource isolation may be achieved by means of RRM policies and protection mechanisms that should avoid that shortage of shared resources in one slice breaks the service level agreement for another slice. It should be possible to fully dedicate NG-RAN resources to a certain slice. How NG-RAN supports resource isolation is implementation dependent.

# Slice Availability

-	Some slices may be available only in part of the network. Awareness in the NG-RAN of the slices supported in the cells of its neighbours may be beneficial for inter-frequency mobility in connected mode. It is assumed that the slice configuration does not change within the UE’s registration area.

-	The NG-RAN and the 5GC are responsible to handle a service request for a slice that may or may not be available in a given area. Admission or rejection of access to a slice may depend by factors such as support for the slice, availability of resources, support of the requested service by other slices.

# Support for UE associating with multiple network slices simultaneously

-	In case a UE is associated with multiple slices simultaneously, only one signalling connection is maintained and for intra-frequency cell reselection, the UE always tries to camp on the best cell. For inter-frequency cell reselection, dedicated priorities can be used to control the frequency on which the UE camps by means of RRC Connection Release message.

# Granularity of slice awareness

-	Slice awareness in NG-RAN is introduced at PDU session level, by indicating the S-NSSAI corresponding to the PDU Session, in all signalling containing PDU session resource information.

# Validation of the UE rights to access a network slice

-	It is the responsibility of the 5GC to validate that the UE has the rights to access a network slice.  Prior to receiving the Initial Context Setup Request message, the NG-RAN may be allowed to apply some provisional/local policies, based on awareness of which slice the UE is requesting access to. During the initial context setup, the NG-RAN is informed for all network slices for which resources are being requested.

### 17.3.2	CN Instance and NW Slice Selection

#### 17.3.2.1	   CN-RAN interaction and internal RAN aspects

NG-RAN selects AMF based on a Temp ID or assistance information provided by the UE over RRC. The mechanisms used in the RRC protocol are described in the next sub clause.

Table 17.3.2.1-1 AMF selection based on Temp ID and assistance information

| Temp ID | Assistance Info | AMF Selection by NG-RAN |
| --- | --- | --- |
| not available or invalid | not available | Default AMF is selected |
| not available or invalid | present | Selects AMF which supports UE requested slices |
| valid | not available, or present | Selects AMF per CN identity information in Temp ID |

#### 17.3.2.2	Radio Interface Aspects

### 17.3.3    Resource Isolation and Management

Resource isolation enables specialized customization and avoids one slice affecting another slice.

Hardware/software resource isolation is up to implementation. Each slice may be assigned with either shared or dedicated radio resource up to RRM implementation and SLA.

To enable differentiated handling of traffic for network slices with different SLA:

-	NG-RAN is configured with a set of different configurations for different network slices;

-	To select the appropriate configuration for the traffic for each network slice, NG-RAN receives relevant information indicating which of the configurations applies for this specific network slice.

### 17.3.4    Signalling Aspects

It is FFS whether signalling procedures will be kept in this section or moved to other sections describing procedures.

#### 17.3.4.1	   General

In this sub clause, signalling flows related to the realization of network slicing in the NG-RAN are given.

#### 17.3.4.2	   CN Instance and NW Slice Selection

RAN selects AMF based on a Temp ID or assistance information provided by the UE.

Figure 17.3.4.1-1: AMF instance selection

In case a Temp ID is not available, the RAN uses the assistance information provided by the UE at RRC connection establishment to select the appropriate AMF instance (the information is provided after MSG3 of the random access procedure). If such information is also not available, the RAN routes the UE to a default AMF instance.

End of Modified Subclause

# References

- 3GPP TS 23.501, System Architecture for the 5G System (Release 15), v.1.4.0, SA2

- 3GPP TS 38.413, NG-RAN; NG Application Protocol (NGAP)  (Release 15), v.0.3.0, RAN3

- 3GPP TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2 (Release 15), v.1.0.0, RAN2