3GPP TSG-RAN WG2 NR Ad Hoc					            R2-1700103

Spokane, USA, 17th – 19th January 2017

Agenda item: 3.2.1.4

Source: National Instruments

Title:	Considerations on the Random-Access Procedure in Massive MIMO NR

Document for: Discussion & Decision

# Introduction

The 5G Study Item regarding the “Study on New Radio Access Technology” was approved at the 3GPP TSG RAN#71 meeting. It has the objective to be able to operate from sub 1 GHz to 100 GHz in a large variety of deployment scenarios in a single technical framework [1]. Massive MIMO has been proposed as a promising candidate to meet the spectral efficiency targets for New Radio (NR) [2]. Within RAN2, there has been discussions and agreements about improvements to the LTE random-access procedures, such as a 2-step version that is expected to improve latency and changes required to accommodate higher frequency NR that requires TX/RX beamforming. In this contribution, we discuss aspects that can improve the efficiency of initial access in Massive MIMO NR while maintaining as much commonality as possible across all use cases.

# Discussion

## Reciprocity-based Massive MIMO NR

A canonical Massive MIMO system [2] has the following defining characteristics

- Large number of antenna ports at the gNB and conventional MIMO UEs

- TDD Operation

- DL Precoding based on UL Channel estimation and Channel Reciprocity

Since reciprocity is leveraged, no explicit feedback is required from the UE to gNB to determine the DL MU-MIMO precoding. As shown in Figure 1, during each coherence time, UL channel sounding is performed before UL data and DL data transmission based on that channel instance. The massive antenna array size enables the gNB to resolve a point in space to focus the energy. This is an important differentiator from conventional beamforming where the beams are steered in a specific direction. NR protocols should be designed to consider these differences of Massive MIMO NR.

Figure 1: Reciprocity-based DL in Massive MIMO

## Random-Access Procedures and Massive MIMO NR

Figure 2: Random-access procedures under study in NR

Figure 2 shows the random-access procedures being discussed in NR. In 3GPP TSG RAN2#96, the group has agreed that the 4-step contention-based and the contention-free procedures should be supported and that these will follow the steps in LTE. [3] observed that these LTE-like procedures are compatible with Massive MIMO NR since they provide scope for reciprocity training. In addition to the LTE-like procedures, a new 2-step procedure can be optionally configured by the network in NR. For the same reasons listed in [3], this new procedure is also compatible with the reciprocity-based Massive MIMO NR.

Observation 1: The 2-step random-access procedure is compatible with the reciprocity-based random-access procedure required in Massive MIMO NR

## NR Use Case-Specific Information and Random-Access

In some NR use cases, the performance and/or efficiency may be enhanced if gNB can obtain additional information regarding the UE, in addition to the information currently exchanged in LTE, as part of the procedure. An example of this is beam index information in high frequency NR that can help decrease the time duration required to complete the random-access procedure.

Observation 2: In some NR use cases, ability to exchange additional use case-specific information during the random-access procedure can improve performance and/or efficiency.

Proposal 1: Study procedural enhancements that enable the UE and gNB to exchange additional information specific to each NR use case during the random-access procedure.

### UE MIMO Capability Information in Massive MIMO NR

For Massive MIMO NR, UE MIMO capability is a parameter of interest. Since the Massive MIMO NR depends on reciprocity and DL beamforming, the system efficiency improves if we can utilize all the available degrees of freedom in the channel. In other words, enabling MIMO channel sounding in the UL and, thereby, the estimation of the optimal DL precoder by the gNB is highly desirable. Ideally, this information can be conveyed during the random-access procedure so that the system can take advantage of this as early as possible [4], [5].

Observation 3: It is desirable to communicate UE MIMO capability information during the random-access procedure in Massive MIMO NR.

Proposal 2: Study procedural enhancements that enable the gNB to obtain the MIMO capability of the UE during the random-access procedure in Massive MIMO NR.

### Example Techniques

The following paragraphs list techniques that can be used to convey additional information during the random-access procedure. A contention-based 4-step process and Massive MIMO NR is assumed in the following discussion as the primary use case. As agreed in 3GPP TSG RAN2#96, the group strives for as much commonality of the procedure among the different use cases. Hence, applicability of the proposed approaches to other NR use cases is also considered.

Option 1: In MSG3

The MIMO capability information is sent during MSG3. In this case, the random-access procedure itself cannot take advantage of the enhancement. Subsequent message exchanges between UE and gNB can utilize it.

This option is flexible and can be extended to other NR use cases. LTE already defines flexible MSG3 size. This can be leveraged by different NR use cases to define different use case-specific MSG3 formats. For example, Massive MIMO NR can add formats that contain UE MIMO capability and high frequency NR can add formats that contain beam index information.

Proposal 3: Study flexible MSG3 format options that can carry additional information specific to each NR use case, such as UE MIMO capability in Massive MIMO NR, to the gNB during the random-access procedure.

Option 2: RACH subsets

The available RACH preambles and resources are separated into subsets where each subset corresponds to a specific UE MIMO capability. The UE selects the RACH preamble and resource from a subset based on its MIMO capability. Then, the UE and gNB can do UL MIMO channel sounding starting from MSG3 exchange onwards, i.e. after the first two steps in the 4-step random-access procedure.

This option is applicable to other NR use cases where other parameters need to be exchanged and to the 2-step procedure.

Option 3: UL MIMO channel sounding during RACH preamble

The available RACH preambles are separated into subsets where each subset corresponds to a specific UE MIMO capability. Assume that the UE can support 2 spatial streams at the transmitter. The UE then selects 2 RACH preambles from the subset corresponding to 2 spatial stream capability. The 2 preambles in this subset may be paired such that the pairs are always selected together. The UE then sends the selected pair of RACH preambles during the first step of the contention-based random-access procedure. The gNB will be able to determine the MIMO capability information, estimate the UL MIMO channel and compute the DL precoder. In this option, all steps in the random-access procedure utilize the MIMO capability of the UE.

### Applicability to 2-Step Random-Access Procedure

The methods listed in the previous section are also applicable to 2-step procedure. In the case of option 1, the use case-specific information can be carried by the data signal that accompanies the RACH preamble from UE, instead of MSG3 as in the 4-step process.

The specific events and use cases where 2-step procedure can be configured is agreed as an FFS item in 3GPP TSG RAN2#96 and will also involve further discussion with RAN1 regarding the outcome of their study on the same topic. Whether it is desirable to exchange additional NR use case-specific information during the 2-step procedure depends on the selected events and use cases.

Proposal 4: Determine if NR use case-specific information embedded in the RACH preamble and its associated data signal in the 2-step random-access procedure is desirable in the events and use cases where the 2-step procedure is enabled.

Proposal 5: Depending on the outcome of proposal 4, study methods to embed NR use case-specific information in the initial RACH preamble and its associated data signal in the 2-step random-access procedure.

# Conclusion

In this contribution, we provide some considerations on Massive MIMO NR and the random-access procedure and propose the following:-

Proposal 1: Study procedural enhancements that enable the UE and gNB to exchange additional information specific to each NR use case during the random-access procedure.

Proposal 2: Study procedural enhancements that enable the gNB to obtain the MIMO capability of the UE during the random-access procedure in Massive MIMO NR.

Proposal 3: Study flexible MSG3 format options that can carry additional information specific to each NR use case, such as UE MIMO capability in Massive MIMO NR, to the gNB during the random-access procedure.

Proposal 4: Determine if NR use case-specific information embedded in the RACH preamble and its associated data signal in the 2-step random-access procedure is desirable in the events and use cases where the 2-step procedure is enabled.

Proposal 5: Depending on the outcome of proposal 4, study methods to embed NR use case-specific information in the initial RACH preamble and its associated data signal in the 2-step random-access procedure.

# References

[1] RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DoCoMo

[2] R1-164117, “Overview of Massive MIMO for NR”, National Instruments, Nanjing, May 2016.

[3] R2-167850, “Considerations on the Random-Access Procedure in Massive MIMO NR”, National Instruments, Reno, Nov 2016.

[4] R1-1609722, “NR MIMO Initial Access Procedure”, National Instruments, Lisbon, Oct 2016.

[5] R1-1612609, “Physical Layer Considerations for Random-Access Procedure in Massive MIMO NR”, National Instruments, Reno, Nov 2016.