3GPP TSG-RAN WG2 #AH	Tdoc R2-1700428

# Spokane, Washington, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.2

Source:	Ericsson

Title:	Data duplication and link selection for URLLC in NR

Document for:	Discussion, Decision

# Introduction

Transmission reliability and latency enhancement are two key aspects for URLLC, defined as Ultra-Reliable and Low-Latency Communications in TR 22.862 and TR 38.913. At present, RAN2 has the following agreements:

- 	Multi-connectivity (e.g. with packet duplication, link selection) should be studied for achieving the reliability requirements for URLLC.”

# Discussion

In a general, multi-connectivity (MC) conceptually can be defined to include all the radio schemes involving more than one radio link to serve a UE for both uplink (UL) and downlink (DL). In NR, MC can include dual-connectivity (DC), and carrier aggregation (CA) from a system architecture perspective.

## Multiple connectivity solutions for URLLC

MC can contribute to URLLC in at least two aspects: i) boosting reliability without sacrificing latency performance; ii) removing the interruption time otherwise caused by mobility. The philosophy to emphasize is that MC as a diversity scheme can obtain a large magnitude of gains in reliability and latency that any link-level diversity scheme by its own cannot achieve [2].

For URLLC, two or more (if considering future enhancement) links might be needed for some extreme-reliability cases such as error rate of 10-5~10-9 within a latency bound [1]. Both DL and UL MC may be of special interests to URLLC, though UL MC may have limiting factors, such as UE’s power.

Below some definitions are provided:

- Packet duplication: duplicating Packet Data Units (PDUs) and transmitting those over multiple connections, or over a single connection dynamically selected from multiple available.

- Link selection/switching: For a multiple connected UE, the link on which to transmit is dynamically decided. Furthermore, the triggers for link selection/switching can be made reactively and/or pro-actively. For instance:

- Reactive link selection: A link selection switching to another connection is triggered after failure(s) of transmission(s) at one connection are detected;

- Pro-active link selection: A selection on another connection is triggered before a failure(s) of transmission(s) in one connection is occurring, e.g. based on channel status information.

Note that for Reactive link selection retransmissions on the other link may be needed, which come with additional delay thus potentially making it unsuitable for URLLC.

### Transmission of packet duplicates

Duplicated data transmission can be implemented in different layers in the protocol stack:

- Data transmission path duplication / diversity can be implemented at PDCP layer: as PDCP supports detection of duplicated PDUs and in-order delivery/reordering, this solution is quite straightforward. Also, it is applicable to both uplink and downlink. In this way, packet duplication and link selection could be realized flexibly on top of existing protocol functionality of RLC and MAC, e.g. independent of underlying scheduling functionality. PDCP based duplication/link-selection can be useful in a deployment that builds on dual connectivity, including LTE-NR interworking.

Data duplication at PDCP layer across cell group (e.g., MCG / SCG) under network control is supported for URLLC.

- Data transmission path duplication / diversity could be provided at lower layers as well: Compared with DC, carrier-aggregation (CA) in LTE operates with a common MAC. As the MAC layer has more timely information on instant radio channel quality measurements (such as CSI report, A/N feedback), diversity at MAC layer has potentially more advantages to improve resource efficiency as compared to the PDCP layer. This may also be more preferably when the latency requirements are very tight compared to service that uses PDCP/backhaul duplication. The protocol impact and solution would need to be studied, but could entail duplication at RLC or MAC; for which MAC transmit in separate Transport Blocks or performs link selection. Here, some layer interaction is needed between MAC and RLC to efficiently use duplication or link selection across component carriers. It is to note that a single HARQ process per carrier is preferable for lower scheduling complexity.

Data duplication at lower layers across component carriers should be investigated for URLLC.

## Trade-off between latency and resource efficiency

Pro-active link selection would be good for both latency and spectrum efficiency. Yet it is not always achievable due to uncertainty in e.g. channel status information; where duplication should be used to avoid long latency due to re-active link selection. Table 3 briefly outline the comparison for duplication and link-selection.

Table 3 latency and spectrum efficiency comparison between duplication and link-selection

|  | Duplication | Link selection |
| --- | --- | --- |
| Latency | Good | Good if ideal or pro-active triggering. But, Bad if rudimentary re-active link-selection is used. |
| Spectrum efficiency | Bad | Good |

To balance between latency and data transmission duplication vs resource efficiency (for which e.g. link selection is performed), it is necessary to also study how to remove unnecessary redundancy caused by buffering at one path while delivered over other paths.

Regarding DC, the backhaul capability is one key decisive factor. DC may be suitable for both relaxed and non-ideal backhaul while CA requires a very short backhaul latency. The latter may though have a better potential to maximize the transmission diversity gain as it may have a shorter time granularity to catch up with fast changes of link status.

Packet duplication might be costly in terms of spectrum resources while link-selection is relatively efficient in a general sense.

Packet duplication is advantageous at latency performance while link selection method may extend the latency as proactive way is not always achievable.

Combined scheme of packet duplication and link selection is supported.

# Conclusion

In section 2 , we made the following observations:

Observation 1	Packet duplication might be costly in terms of spectrum resources while link-selection is relatively efficient in a general sense.

Observation 2	Packet duplication is advantageous at latency performance while link selection method may extend the latency as proactive way is not always achievable.

Based on the discussion in section 2 we propose the following:

Proposal 1	Data duplication at PDCP layer across cell group (e.g., MCG / SCG) under network control is supported for URLLC.

Proposal 2	Data duplication at lower layers across component carriers should be investigated for URLLC.

Proposal 3	Combined scheme of packet duplication and link selection is supported.

# References

R1-167061, “On URLLC design principles”, Ericsson, TSG-RAN WG1 #86，Gothenburg, Sweden, August 22–26, 2016

R2-168629, “NR UP Design for URLLC”, Ericsson, 3GPP TSG-RAN WG2 #96，Reno, Nevada, USA, 14th – 18th November 2016

3GPP TR 36.912 V13.0.0 (2015-12), Technical Report,3GPP, Technical Specification Group Radio Access Network, “Feasibility study for Further Advancements for E-UTRA (LTE-Advanced),(Release 13)”

R2-1700436 – “URLLC aspects for contention based UL transmission in NR”, 3GPP TSG-RAN WG2 NR, Spokane, USA, 16th – 20th January 2017