3GPP TSG-RAN WG2 NR Ad Hoc	Tdoc R2-1700431

Spokane, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.2

Source:	Ericsson

Title:	Aspects for contention based UL transmission in NR

Document for:	Discussion, Decision

# Introduction

The “autonomous/grant-free/contention based” UL transmission for NR has been discussed in RAN1. In the last meeting RAN1#87, the following agreement was made

Agreements:

- At least an UL transmission scheme without grant is supported for URLLC

  - Resource may or may not be shared among one or more users

    - FFS: resource configuration details

  - FFS other details of design

Based on RAN1 discussions, this contribution discusses some RAN2 aspects concerning “autonomous/grant-free/contention based” UL transmission.

# Discussion

## Background

Autonomous or grant-free transmissions refer to the case when no explicit grant needs to be signalled using downlink control channels for every UL transmission, i.e. opposite to LTE dynamic scheduling via PDCCH. Instead UEs are assigned resources that can be used for UL transmissions whenever the UEs receive UL data in their buffers. The resources could further be assigned in a one-to-one mapping between UE and resource such as in the SPS mechanism in LTE or a many-to-one mapping. In the case of many-to-one mapping between UE and resource, the transmissions will be subject to contention implying that there is a risk of collision if more than one UE would use the same time and frequency resource.

## Aspects in grant-free and contention based UL transmission

### Use cases

Low latency is one of the key requirements in many 5G use cases such as critical machine-type communication, which is referred to by ITU as ultra-reliable and low latency communication (URLLC) and possibly mMTC (massive machine type communication). As one candidate technology to meet requirements related to such devices, grant-free and contention based UL transmissions are being discussed in RAN1. RAN2 should thus study the feasibility and solutions to apply grant-free transmissions considering both

- The case where a UE is allocated grant-free dedicated resources, i.e. there is a one-to-one mapping between the UE and resource. We call this case grant-free UL transmissions.

- The case where several UEs are allocated a grant-free but shared resource, i.e. there is a many-to-one mapping between the UEs and resource.  We call this case grant-free contention based UL transmissions.

Study the feasibility to apply grant-free UL transmission for use cases with critical latency requirements.

Study the feasibility to apply grant-free contention based UL transmission for use cases with critical latency requirements.

### RRC states

The basic use case for grant-free UL transmissions is when the UE is in the RRC connected state. In this case the grant-free contention based UL transmission and the related contention resolution mechanisms will be a complement to dynamic scheduling to reduce the latency due to grant allocation.

On the other hand, a UE in inactive ‘state’ may also benefit from a grant-free contention based channel to reduce the control plane latency. This can be achieved by e.g. transmitting a request to resume connection over a contention based channel. Furthermore, in the special case of small and infrequent data transmissions, a contention based mechanism could also be used to transmit data. This is described in more detail in [4], where we expand more on mechanisms to multiplex small data with a resume message over a contention-based channel using a two-step random access.

Grant-free contention based UL transmissions should be supported to complement dynamic scheduling to reduce latency caused by grant allocation in connected state.

Grant-free contention based transmissions to send e.g. a RRC “Resume” message, possibly multiplexed with small data when UE is in RRC Inactive ‘state’ should be supported.

### Resource Allocation

In LTE, an SPS like mechanism allows the transmission of data packets without scheduling requests, which is beneficial to reduce latency. The UE is pre-configured by the eNB with an SPS-RNTI and a SPS periodicity. First the network activates SPS resources via PDCCH.  Then, during every SPS period, the UE can transmit using the configured grant. Compared to dynamic scheduling, pre-scheduling with SPS may result in less efficient resource utilization due to its semi-static nature, for example, PRB positions and MCS setting are rather semi-static, and thus has the issue of resource over-provisioning; especially when there are a large number of UEs with SPS resources at the same time.

SPS like resource allocation, i.e., explicitly pre-allocated resources to UEs have a risk of resource over-provisioning

In LTE, contention based PUSCH transmission has been studied in SI LATRED [1]. According to this study, for contention based PUSCH transmission, multiple UEs may share the same PUSCH resource and this may allow more efficient PUSCH resource utilization compared to the existing pre-scheduling scheme. Collision will happen if two or more UEs that share the same PUSCH resource perform a transmission at the same time, and in this case the eNB may not be able to successfully decode all the PUSCH transmissions. Because of collision, potential retransmissions can result in increased latency for colliding UEs. It should be noted that it was proposed to include an indication of the DMRS cyclic shift as part of the SPS configuration, which would allow the network to identify the colliding UEs.

Allowing multiple UEs to share the same time and frequency resources may result in more efficient resource utilization but is sensitive to collisions

SPS like transmission is beneficial for latency reduction but require that a large amount of resources is set aside for each UE. This is doable when the system load is low, while contention based transmission will be a better alternative from resource efficiency when the load is high. Hence, both mechanism should be supported to complement each other.

NR support both grant-free UL transmissions and grant-free contention based UL transmissions

### Contention resolution

Since collisions will happen in the contention based case, efficient mechanisms for contention resolution also need to be studied. Since the gNB needs to know which UE has transmitted on the shared resource, some form of UE identity must be linked to the transmission. Several approaches are possible for the UE to indicate its identity. For connected state, one possibility would be for the UE to attach its C-RNTI in the transmission on the shared resource. However, this approach would lead to a high unnecessary overhead as the C-RNTI is two bytes. Another approach would be to scramble the CRC of the data with the C-RNTI.

Another more efficient approach would be as proposed in R1-1612955, to overlay the UE identity on orthogonal DMRS patterns. The gNB would then deduce which UE has transmitted based on the received DMRS. This kind of approach was also discussed in LTE SI LATRED [1]. Successful contention resolution relies on that both the UE identity and the data have been successfully decoded. In case only the UE identity is decoded, the UE should have the opportunity to retransmit the data on a contention free resource. In case neither the data nor the UE id has been successfully decoded, an efficient method for retransmissions should be designed.

Contention resolution mechanisms for grant-free contention based UL transmissions shall be studied

If the UE is in inactive state, it has no defined or active DMRS pattern. In inactive state the obvious UE identification would be from its Resume ID. In this case the overhead is unavoidable as the gNB anyway needs to receive the Resume ID including both UE and network part to fetch the right RRC context to be able to decrypt the user data.

NR should use Resume ID in RRC inactive state for grant-free contention based UL transmission

# Conclusion

In section 2 we made the following observations:

Observation 1	SPS like resource allocation, i.e., explicitly pre-allocated resources to UEs have a risk of resource over-provisioning

Observation 2	Allowing multiple UEs to share the same time and frequency resources may result in more efficient resource utilization but is sensitive to collisions

Based on the discussion in Section 2 we propose the following:

Proposal 1	Study the feasibility to apply grant-free UL transmission for use cases with critical latency requirements.

Proposal 2	Study the feasibility to apply grant-free contention based UL transmission for use cases with critical latency requirements.

Proposal 3	Grant-free contention based UL transmissions should be supported to complement dynamic scheduling to reduce latency caused by grant allocation in connected state.

Proposal 4	Grant-free contention based transmissions to send e.g. a RRC “Resume” message, possibly multiplexed with small data when UE is in RRC Inactive ‘state’ should be supported.

Proposal 5	NR support both grant-free UL transmissions and grant-free contention based UL transmissions

Proposal 6	Contention resolution mechanisms for grant-free contention based UL transmissions shall be studied

Proposal 7	NR should use Resume ID in RRC inactive state for grant-free contention based UL transmission

# References

TR 36.881, Study on latency reduction techniques for LTE, v0.6.0

R2-166799, uplink dynamic scheduling in NR

R2-166922 , Small data transmission for inactive Ues