3GPP TSG-RAN WG2 NR AH#2	Tdoc R2-1706625

# Qingdao, China, 27th – 29th June 2017                      (Revision of R21704434)

Agenda Item:	10.2.8

Source:	Ericsson

Title:	0 ms interruption support during handover procedure in NR

Document for:	Discussion, Decision

# Introduction

In RAN2#97, the following has been agreed.

Agreements

1	We will aim to define HO for NR with an interruption as close to zero as possible while only having single Tx/Rx in the UE, and 0ms interruption at least for the case that the UE supports simultaneous Tx/Rx with source cell and target cell during HO

It has been further agreed in RAN2#97bis that:

=>	We will progress the basic HO mechanism for NR (not including LTE Rel-14-like mobility enhancements) and when stable we can discuss potential optimisations to target close to 0ms or 0ms interruption.

=>	We will progress handover with 0ms interruption with dual tx/rx targeting to define a single solution. Discussion of this can start when basic DC operation is more stable

That is, we need to find solution to support 0 ms interruption during handover from source to target cell at least when the UE supports dual Tx/Rx.

In this contribution, we will discuss how the target of 0ms interruption time can be fulfilled. The paper analyses two solutions: i) handover with some enhancement, ii) DC (Dual connectivity) with role switch.

# Discussion

The definition of 0 ms interruption is from 38.913 which is listed below:

7.7	Mobility interruption time

Mobility interruption time means the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The target for mobility interruption time should be 0ms.

According to this requirement, it means UE need to at least transmit/receive PDCP packet with source at subframe N, and then transmit/receive PDCP packet with target at subframe N+1, otherwise 0ms interruption is not satisfied. Below we analyse two solutions to achieve 0 ms interruption:

- Normal handover with some enhancement;

- Dual Connectivity with role switch;

## 0ms interruption support with enhanced handover procedure

For simplicity, we first consider inter-frequency case where UE is assumed with dual Tx/Rx chain.

To support 0ms interruption during handover, both source cell and UE can continue to communicate with each other even after the source node sends HO command to the UE. As a comparison, a feature called make-before-break was just standardized in LTE Rel-14 which is used to reduce handover interruption delay.

As make-before-break, is for single Tx/Rx UE, the UE will release the connection to source when the following conditions are met:

- Transmits the RACH preamble;

- Transmits the HO Complete message on the PUSCH (if RACH-less HO with UL grant received in HO Command message); or

- Monitors the PDCCH in the target cell (if RACH-less HO without UL grant in HO Command message. The UE instead receives UL grant through the PDCCH).

Or in other word, UE release connection with source before its connection with target is ready for packet transmission/reception. So make-before-break feature as it was defined in Rel-14 cannot meet 0 ms interruption requirement. For 0 ms interruption, the connection between the source and the UE can be released only after UE can receive PDCP packets from the target cell/target gNB.

Therefore, to support 0 ms interruption, after receiving HO command, UE should keep its connection with the source cell until it is able to receive packets from the target cell. Correspondingly after sending HO command to the UE, source cell should continue serve UE in parallel to forward data to target cell so that target cell is ready to serve UE when UE connect to target cell.

The whole signalling procedure to support 0ms interruption is illustrated in Figure 1 below.

Figure 1 Flow chart during handover to support 0ms interruption

Step 1 to 5, same as those step in LTE, so no further explanation.

Step 6: Source send to the UE a handover command, (i.e. RRCConnectionReconfiguration with MobilityControlInfo), containing an indicator to inform UE to do 0ms interruption handover.

Note 1: Source can start to forward PDCP packet to target

Step 7: UE then start to synchronize with target cell, but keep its connection with target

Step 8: UE synchronized with target cell

Step 9: Target cell starts to schedule UE in uplink

Step 10: UE sends RRCConnectionReconfigurationComplete message to target

Note 2: Now target can start to send UE PDCP packet. At the same time, source may continue send UE PDCP packet

Step 11: UE needs to distinguish PDCP packet from source and target cell

Note 3: The PDCP packet received by UE from target may be a duplication from source. But this still meet 0ms interruption requirement. Avoiding duplication over the air can be left to further NW implementation

Step 12: UE detach from source cell

Step 13: Source cell send SN status transfer message to target cell

Step 14: Target inform source to release UE context

Step 15: Source release resource for UE

As can be seen in the figure 1, there could be some overlap duration in which UE needs to simultaneously receive PDCP packets from source cell and the PDCP packet received from target cell. This is because it is quite challenging to know the right timing when UE should stop to receive from source and start to receive from target without interruption.

The protocol stack at UE during this HO transition period is illustrated in Figure 2 below.

PDCP

RLC

MAC

PHY

Source

PDCP

RLC_S

RLC_T

MAC_S

MAC_T

PHY_S

PHY_T

PDCP

RLC

MAC

PHY

Target

PDCP packet

SN=X,X+1,X+2

UE

SN=X,X+1

SN=X+1,X+2

Figure 2 UE internal protocol stack to support 0 ms interruption

That is, two RLC/MAC entities are setup during this transition period with one corresponds to data transmission/reception with source and the other corresponds to data transmission/reception with the target. Both connect to one PDCP entity at UE.

From UE protocol stack perspective, this looks similar to dual connectivity 3C. But they are different. UE need to receive PDCP packet generated by both source and target gNB in this solution, instead of just from MgNB in DC 3C.

During HO, there are two sub-cases to consider, security KEYs are different between source and target cell, and security KEY is the same for source and target cell.

If different security KEYs are used at the two cells, since only one PDCP entity is used to handle these packets during handover at UE, UE need to identify which PDCP packet is from which cell so that it can decipher packets correctly. This is not an issue according to figure 2 as PDCP entity can know where a PDCP packet is from according to which RLC entity delivers that packet to it.

If security KEY does not change between source and target cell, then PDCP entity does not need to even check from where PDCP PDU comes from.

Based on the above analysis, we have the following proposals.

- It is proposed to adopt a handover procedure with some enhancement to meet 0ms interruption. Precisely, after receiving HO command, UE do the followings:

- Keep connection with source cell. Continue to transmit/receive with the source cell;

- Initiate random access to target cell, with new MAC and RLC entity established for target cell;

- Release connection with source cell after receives/transmits PDCP packet from target cell

- How to distinguish PDCP packets ciphered with different KEYs by source and target is an implementation issue in UE.

Besides UE, the source cell also needs to know when it does not need to serve this UE anymore. The source cell can know such information via legacy LTE procedure, i.e. when the target cell requests the source cell to release UE context after the target cell gets Path Switch Request Ack from the core network. If such delay is regarded as a bit long, source can also sense that UE has left it by checking the HARQ/RLC feedback from UE. That is, it is up to source node implementation, as was also the approach selected for the Rel-14 solution.

RAN2 does not need to specify the solution about how source node know when not to schedule UE anymore

Another impact at the source cell to support 0ms interruption is when/how to transfer SN status transfer message. In normal handover, the source cell will send to the target cell a SN status transfer message once it sends HO command to the UE. As the source cell will not serve the UE after it has sent HO command to the UE, the SN in the SN status transfer message can be determined. To support 0ms interruption, the source needs to continue serve the UE after send HO command. Then it is not possible to determine which SN it will not use for the PDCP packet.

Source gNB cannot determine the SN to be assigned to PDCP SDU for target cell when it send HO command to the UE as it need to continue serve UE.

As in LTE, the timing of the SN status transfer message can be left for network implementation. The source cell will not send status transfer message to the target cell after it send a HO command to the UE. The source cell just forwards PDCP SDU with SN assigned by it to the target cell. Source cell sends SN status transfer message to target cell after it know it does not need to serve UE anymore. This is of course still need RAN3 to decide.

Transmission timing of the SN status transfer message is left for network implementation, as in Rel-14.

## 0ms interruption support using DC procedure with role switch

Another approach to meet 0ms interruption is to adopt DC procedure. In this approach, target node will be first added as a SgNB, and then source MgNB initiate a role switch procedure between MgNB and SgNB so that SgNB become new MgNB and MgNB become new SgNB, finally, new SgNB is released. The whole signalling procedure is shown in Figure 3 below.

Figure 3 Signalling procedure for a UE to support 0 ms interruption during HO using DC procedure

Compare Figure 1 to Figure 3, it can be seen clearly that the signalling procedure of DC is complicated compared to HO procedure to meet 0ms interruption requirement.

- To meet 0 ms interruption, it is not necessary to first add target gNB as SgNB and then executing role switch procedure between MgNB and SgNB.

## 0ms interruption in intra-frequency mobility scenario

For intra-frequency mobility, RAN2 is not clear whether simultaneous transmission/reception with both source and target is feasible or not. Therefore, RAN2 send a LS to RAN4/RAN1.

The following are some observation from RAN4 [1].

O2 : Dual RX chains or dual TX chains allow independent gain setting but this seems not to be very useful in the context of simultaneous reception or simultaneous transmission.

O3 : For asynchronous scenarios, dual FFT/iFFT would be needed to account for different RX or TX timing

That is, for asynchronous network, dual Tx/Rx Chain is needed, but it seems not realistic to assume there is always a spare FFT for intra-frequency HO purpose.

For UE with single Tx/Rx, TDM solution between two legs seems a reasonable solution. But it may require good synchronization between source and target node.

Whatever UE is with single or dual Tx/Rx, from whole signalling procedure perspective, we do not think there are differences between inter-frequency and intra-frequency to support 0 ms interruption during handover. The signalling procedure discussed in 2.1 for inter-frequency can be applied to intra-frequency as well if simultaneous transmission/reception to/from source and target cell in intra-frequency is feasible

- If intra-frequency simultaneous transmission/reception to/from source and target is feasible, the signalling procedure for inter-frequency can be applied as well to support 0 ms interruption for intra-frequency handover.

# Conclusion

Based on the discussion in section 2, we have the following observation and proposals.

Observation 1	RAN2 does not need to specify the solution about how source node know when not to schedule UE anymore

Observation 2	Source gNB cannot determine the SN to be assigned to PDCP SDU for target cell when it send HO command to the UE as it need to continue serve UE.

- It is proposed to adopt a handover procedure with some enhancement to meet 0ms interruption. Precisely, after receiving HO command, UE do the followings:

- Keep connection with source cell. Continue to transmit/receive with the source cell;

- Initiate random access to target cell, with new MAC and RLC entity established for target cell;

- Release connection with source cell after receives/transmits PDCP packet from target cell

- How to distinguish PDCP packets ciphered with different KEYs by source and target is an implementation issue in UE.

Transmission timing of the SN status transfer message is left for network implementation, as in Rel-14.

To meet 0 ms interruption, it is not necessary to first add target gNB as SgNB and then executing role switch procedure between MgNB and SgNB.

If intra-frequency simultaneous transmission/reception to/from source and target is feasible, the signalling procedure for inter-frequency can be applied as well to support 0 ms interruption for intra-frequency handover.

# Reference

[1] R4-1704805 Considerations on the feasibility of DC-related mobility enhancements in NR, RAN4#83, HangZhou, China