3GPP TSG RAN WG2#NR_AdHoc#2	                                  R2-1706656

Qingdao, China, 27 - 29 June, 2017                                      (Resubmission of R2-1704651)

Agenda Item:	10.2.4

Source: 	ZTE Corporation

Title:  	Consideration on the counter check procedure in LTE/NR tight interworking

Document for:	Discussion and decision

Introduction

In the last meeting a LS(R2-1706161) was planned and sent to SA3 to ask whether the Counter Check procedure is needed if integrity protection is supported for DRB. Even if the answer from SA3 is No, considering that the function of integrity protection is optionally enabled, the Counter Check procedure is still needed for the case that integrity protection of whole UP data is disabled or for the DRBs which are not configured with integrity protection.

Observation1: Considering that integrity protection is optionally enabled, the Counter Check procedure is needed for the case that integrity protection of whole UP data is disabled or for the DRBs which are not configured with integrity protection.

Counter check procedure is used optionally by E-UTRAN to periodically perform a local authentication, i.e. the UE is requested to check if, for each DRB, the most significant bits of the COUNT match with the values indicated by E-UTRAN.

In LTE DC, SeNB initiated counter check procedure is used to perform the verification of the value of the PDCP COUNTs associated with SCG bearers established in the SeNB. The procedure is as below[1]:

Figure 1: Legacy LTE DC Counter Check Procedure

For LTE-NR tight interworking (option3a/3x), assuming the same counter check procedure of LTE-DC is adopted in EN-DC (note that how to perform the counter check in NR is not decided yet.), i.e. SgNB triggers counter check and sends the PDCP COUNT values of all SCG DRBs to MeNB, MeNB probably be hard to understand the COUNT CHECK REQUEST message from SgNB since the value range of DRB ID and PDCP COUNT of SCG bearers might be different with MeNB. Considering the case that MeNB offload the majority part or even total of the data throughput to SgNB, counter check procedure may be seldom triggered by MeNB itself during the whole connection time of UE, thus security risks exists if MeNB cannot get any usable counter check information of SgNB bearers.

Observation2: There’s problem if simply adopting the counter check mechanism of LTE-DC to EN-DC, since the value range definition of DRB ID and PDCP COUNT of NR SCG bearers might be incompatible with LTE MeNB.

In this paper we discuss how to solve above issue.

Discussion

For EN-DC, direct SCG SRB was agreed to be introduced in RAN2#97 :

- UE can be configured with an SCG SRB to allow SN RRC messages to be sent directly between UE and SN.

- For SN RRC reconfigurations not requiring any coordination with MN then SN RRC messages can be transported directly to the UE (or eNB implementation can be deliver it embedded within a MN RRC message)

Based above agreements, technically it can be supported that the SgNB independently perform the counter check procedure with UE. For example, similar counter check procedure as LTE could be adopted, i.e. SgNB triggers counter check procedure ( if the PDCP COUNT value of any SCG DRBs reaches a pre-defined value) and sends counter check message including all PDCP COUNT values of all SCG DRBs to UE, UE feedback to SgNB directly with the counter check response message including the PDCP COUNT checking results, then SgNB transfer the counter check results of SCG bearers to MeNB with a understandable format when necessary, e.g. counter check failure is detected.

An example of independent SN initiated counter check procedure is shown in figure 2.

Figure 2: Example of independent SN initiated Counter Check Procedure of LTE-NR tight interworking

With the figure-2 procedure, a complete counter check between the network and UE could be accomplished in current phase.

Based on above analysis, we propose the counter check procedure should be initiated by MN and SN independently in EN-DC, and SN initiated counter check message and the response message from UE should be allowed to transfer on SCG SRB.

Proposal 1: In LTE-NR tight interworking, the counter check procedure should be initiated by MN and SN independently.

Proposal 2: The transmission of Counter Check related messages through SCG SRB should be allowed.

During the procedure in figure 2, the SgNB only needs to report the Count Check results of SN bearers to the MN if the Counter Check Response message from UE contains one or more PDCP COUNT values, i.e. PDCP COUNT failure is found, otherwise SgNB doesn't have to send anything to MeNB.

Proposal 3: SN doesn't need to transfer any Counter Check results related info to MN if there's no Counter Check failure detected.

As mentioned above, it's not decided yet whether same value ranges of DRB ID and PDCP COUNT of LTE are adopted in NR standalone, thus some simplifications on the Counter Check results needs to be considered to design an understandable message to MeNB. Two potential alternatives are listed below:

- Alt-1: A SN initiated SN release message with a cause as Counter Check failure.

- Alt-2: One indication to inform the Counter Check failure of SCG bearers, e.g. one-bit of TRUE/FALSE indication could be considered for simplification, or more bits for further detailed information about the failure.

In current phase, SN initiated SN release is meant to be supported as a essential procedure in LTE/NR tight-interworking, which means alt-1 can be easily supported with minor impacts to the specification. From the new cause ' Counter Check failure ' MN can get the counter check result thus the whole counter check procedure could work.

Proposal 4: Once counter check failure is detected in SN, one SN release request could be sent to MN with the cause as counter check failure.

But from SA3's view point, MN should be the final decision maker for the Counter Check failure handling, i.e. SN release might not be what MN actually wants which might cause MN rejecting the SN release request. A direct information of Counter Check result of SN is still needed for MN to make more decision chooses, e.g. release the whole connection of UE or just report to MME/O&M to track the attacker. Based this requirement Alt-2 seems also necessary.

In current phase, the design of the indication could be left FFS. It's proposed to firstly consider introducing the indication on Xn interface to inform the Counter Check failure of SN to MN, thus the procedure of SN initiated counter check for LTE-NR tight inter-working could be completed and speeding the stage2 specification progress.

Proposal 5: Once counter check failure is detected in SN, one counter check failure indication should be sent to MN.

Noted that Alt-1 and 2 does not conflicts to each other, they can be applied both.

Conclusion

Observation1: Considering that integrity protection is optionally enabled, the Counter Check procedure is needed for the case that integrity protection of whole UP data is disabled or for the DRBs which are not configured with integrity protection.

Observation2: There’s problem if simply adopting the counter check mechanism of LTE-DC to EN-DC, since the value range definition of DRB ID and PDCP COUNT of NR SCG bearers might be incompatible with LTE MeNB.

Proposal 1: In LTE-NR tight interworking, the counter check procedure should be initiated by MN and SN independently.

Proposal 2: The transmission of Counter Check related messages through SCG SRB should be allowed.

Proposal 3: SN doesn't need to transfer any Counter Check results related info to MN if there's no Counter Check failure detected.

Proposal 4: Once counter check failure is detected in SN, one SN release request should be sent to MN with the cause as counter check failure.

Proposal 5: Once counter check failure is detected in SN, one counter check failure indication should be sent to MN.

Reference

[1] TS36.300

[2] TS36.331

[3] TS33.401

[4] TS36.423