3GPP TSG-RAN WG2 NR-Adhoc	R2-1707480

Qingdao, China, 27th – 29th June 2017

Agenda item:	10.4.1.4.1

Source:	Samsung

Title:	TP on RRM Model to 38.300

Document for:	Approval

## ************** AGREED TEXT PROPOSAL TO TS 38.300 *****************

## X.X	Measurement Model

NOTE: K beams correspond to the measurements on NR-SS block or CSI-RS resources configured for L3 mobility by gNB and detected by UE at L1.

-	A: measurements (beam specific samples) internal to the physical layer.

-	Layer 1 filtering: Internal layer 1 filtering of the inputs measured at point A. Exact filtering is implementation dependent. How the measurements are actually executed in the physical layer by an implementation (inputs A and Layer 1 filtering) in not constrained by the standard.

-	A1: Measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering.

-	Beam Consolidation/Selection: Beam specific measurements are consolidated to derive cell quality if N > 1, else when N = 1 the best beam measurement is selected to derive cell quality. The behaviour of the Beam consolidation/selection is standardised and the configuration of this module is provided by RRC signalling. Reporting period at B equals one measurement period at A1.

-	B: A measurement (i.e. cell quality) derived from beam-specific measurements reported to layer 3 after beam consolidation/selection.

-	Layer 3 filtering for cell quality: Filtering performed on the measurements provided at point B. The behaviour of the Layer 3 filters are standardised and the configuration of the layer 3 filters is provided by RRC signalling. Filtering reporting period at C equals one measurement period at B.

-	C: A measurement after processing in the layer 3 filter. The reporting rate is identical to the reporting rate at point B. This measurement is used as input for one or more evaluation of reporting criteria.

-	Evaluation of reporting criteria: This checks whether actual measurement reporting is necessary at point D. The evaluation can be based on more than one flow of measurements at reference point C e.g. to compare between different measurements. This is illustrated by input C and C1. The UE shall evaluate the reporting criteria at least every time a new measurement result is reported at point C, C1. The reporting criteria are standardised and the configuration is provided by RRC signalling (UE measurements).

-	D: Measurement report information (message) sent on the radio interface.

-  L3 Beam filtering: Filtering performed on the measurements (i.e. beam specific measurements) provided at point A1. The behaviour of the beam filters are standardised and the configuration of the beam filters is provided by RRC signalling. Filtering reporting period at E equals one measurement period at A1.

- 	E: A measurement (i.e. beam-specific measurement) after processing in the beam filter. The reporting rate is identical to the reporting rate at point A1. This measurement is used as input for selecting the X measurements to be reported.

-	Beam Selection for beam reporting: This selects the X measurements from the measurements provided at point E. The behaviour of the beam selection is standardised and the configuration of this module is provided by RRC signalling.

-	F: Beam measurement information included in measurement report (sent) on the radio interface.

Layer 1 filtering will introduce a certain level of measurement averaging. How and when the UE exactly performs the required measurements will be implementation specific to the point that the output at B fulfils the performance requirements set in [RAN4 Ref]. Layer 3 filtering and parameters used is specified in [38.331] and does not introduce any delay in the sample availability between B and C. Measurement at point C, C1 is the input used in the event evaluation. L3 Beam filtering and parameters used is specified in [38.331] and does not introduce any delay in the sample availability between E and F.

## *********************** END OF TEXT PROPOSAL **************************