3GPP TSG-RAN WG2 Meeting AH-1801                                                          R2-1800660

Vancouver, Canada, 22nd – 26th January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for beam failure recovery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | yyyy-MM-dd |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel- 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Fix the spec misalignment between RAN1 and RAN2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the parameters configured for beam failure recovery transmission through dedicated PRACH resource - Beam failure recovery request is transmitted through dedicated PRACH resource with the maximum transmission number PreambleTransMax-BFR. - Candidate beam is selected based on L1-RSRP within candidateBeam-Rs-List comparing with the beamFailureCandidateBeamThreshold and Power-offset between SSB and CSI-RS - Preamble transmission power for beam failure recovery transmission is calculated using the set of parameters configured for beam recovery - If maximum transmission number of beam failure recovery request over dedicated PRACH resource is reached, UE considers it as beam recovery failure. No indication to RRC is sent. - MAC layer detects beam failure by counting a number of consecutive detected beam failure instances, which is indicated from PHY layer. - Beam failure recovery transmission is triggered when both beam failure is detected and candidate beam is identified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistence exists between MAC and PHY |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2,5.1.3, 5.1.4, 5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 38.331 CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.1	Random Access procedure

### 5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell other than PSCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

RRC configures the following parameters for the Random Access procedure:

-	prach-ConfigIndex: the available set of PRACH resources for the transmission of the Random Access Preamble;

-	ra-PreambleInitialReceivedTargetPower: initial preamble power;

-	ra-PreambleInitialReceivedTargetPower-bfr: initial preamble power for beam failure recovery request transmission;

-	rsrp-ThresholdSSB, csirs-dedicatedRACH-Threshold, and sul-RSRP-Threshold: an RSRP threshold for the selection of the SS block and corresponding PRACH resource;

-	ra-PreamblePowerRampingStep: the power-ramping factor;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-PreambleIndex-bfr: Random Access Preamble for beam failure recovery request transmission;

-	ra-PreambleTx-Max: the maximum number of preamble transmission;

-	ra-PreambleTx-Max-bfr: the maximum number of preamble transmission for beam failure recovery request transmission;

-	if SSBs are mapped to preambles:

-	startIndexRA-PreambleGroupA, numberOfRA-Preambles, and numberOfRA-PreamblesGroupA for each SSB in each group (SpCell only).

-	else:

-	startIndexRA-PreambleGroupA, numberOfRA-Preambles, and numberOfRA-PreamblesGroupA in each group (SpCell only).

-	If numberOfRA-PreamblesGroupA is equal to numberOfRA-Preambles, there is no Random Access Preambles group B.

-	The preambles in Random Access Preamble group A are the preambles startIndexRA-PreambleGroupA to startIndexRA-PreambleGroupA + numberOfRA-PreamblesGroupA – 1;

-	The preambles in Random Access Preamble group B, if exists, are the preambles startIndexRA-PreambleGroupA + numberOfRA-PreamblesGroupA to startIndexRA-PreambleGroupA + numberOfRA-Preambles – 1.

NOTE 2:	if random Access Preambles group B is supported by the cell and SSBs are mapped to preambles, random access preambles group B is included in each SSB.

-	if Random Access Preambles group B exists:

-	ra-Msg3SizeGroupA (per cell): the threshold to determine the groups of Random Access Preambles.

-	the set of Random Access Preambles for SI request and corresponding PRACH resource(s), if any;

-	the set of Random Access Preambles for beam failure recovery request and corresponding PRACH resource(s), if any;

-	ra-ResponseWindow: the time window to monitor RA response(s);

-	bfr-ResponseWindow: the time window to monitor response(s) on beam failure recovery request;

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B exists:

-	if the MAC Entity is configured with supplementaryUplink, and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,c_SUL: the configured UE transmitted power of the SUL carrier.

-	else:

-	PCMAX,c: the configured UE transmitted power of the Serving Cell performing the Random Access Procedure.

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	TEMPORARY_C-RNTI.

When the Random Access procedure is initiated, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	if the carrier to use for the Random Access procedure is explicitly signalled:

2>	select the signalled carrier for performing Random Access procedure.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the cell for the Random Access procedure is configured with supplementaryUplink; and

1>	if the RSRP of the downlink pathloss reference is less than sul-RSRP-Threshold:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,c_SUL.

1>	else:

2>	select the normal carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,c.

1>	perform the Random Access Resource selection procedure (see subclause 5.1.2).

### 5.1.2	Random Access Resource selection

The MAC entity shall:

1>	if the Random Access procedure was initiated by a beam failure indication from lower layer; and

1>	if the contention free PRACH resources for beam failure recovery request and the associated SS blocks and/or CSI-RSs in candidateBeam-RS-List have been explicitly provided by RRC; and

1>	if at least one of the SS blocks with L1-RSRP above beamFailureCandidateBeamThreshold +Power-offsetamongst the associated SS blocks or the CSI-RSs with L1-RSRP above beamFailureCandidateBeamThreshold amongst the associated CSI-RSs is available:

2>	select an SS block with L1-RSRP above beamFailureCandidateBeamThreshold +Power-offsetamongst the associated SS blocks or a CSI-RS with L1-RSRP above beamFailureCandidateBeamThreshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex-bfr corresponding to the selected SS block or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by either PDCCH or RRC; and

1>	if the ra-PreambleIndex is not 0b000000; and

1>	if contention free PRACH resource associated with SS blocks or CSI-RS have not been explicitly provided by RRC:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex.

1>	else if the contention free PRACH resources associated with SS blocks have been explicitly provided by RRC and at least one SS block with SS-RSRP above rsrp-ThresholdSSB amongst the associated SS blocks is available:

2>	select an SS block with SS-RSRP above rsrp-ThresholdSSB amongst the associated SS blocks;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SS block.

1>	else if the contention free PRACH resources associated with CSI-RSs have been explicitly provided by RRC and at least one CSI-RS with CSI-RSRP above csirs-dedicatedRACH-Threshold amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above csirs-dedicatedRACH-Threshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else:

2>	select a SS block with SS-RSRP above rsrp-ThresholdSSB;

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B exists; and

3>	if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – ra-PreambleInitialReceivedTargetPower:

4>	select the Random Access Preambles group B.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the preamble transmission attempt corresponding to the first transmission of Msg3.

2>	if the association between Random Access Preambles and SS blocks is configured:

3>	select a ra-PreambleIndex randomly with equal probability from the random access preambles associated with the selected SS block and the selected group.

2>	else:

3>	select a ra-PreambleIndex randomly with equal probability from the random access preambles within the selected group.

2>	set the PREAMBLE_INDEX to the selected ra-PreambleIndex.

1>	if an SS block is selected above and an association between PRACH occasions and SS blocks is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SS block.

1>	else if a CSI-RS is selected above and an association between PRACH occasions and CSI-RSs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected CSI-RS.

1>	else:

2>	determine the next available PRACH occasion.

1>	perform the Random Access Preamble transmission procedure (see subclause 5.1.3).

### 5.1.3	Random Access Preamble transmission

The MAC entity shall, for each preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if SS block selected is not changed (i.e. same as the previous random access preamble transmission):

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

- if the Random Access Preamble is for beam failure recovery request:

2> set PREAMBLE_RECEIVED_TARGET_POWER to ra-PreambleInitialReceivedTargetPower -bfr+ DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) * powerRampingStep-bfr;

1>  else:

2>	set PREAMBLE_RECEIVED_TARGET_POWER to ra-PreambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) * powerRampingStep;

1>	except for contention free preamble for beam failure recovery request, compute the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the preamble using the selected PRACH, corresponding RA-RNTI (if available), PREAMBLE_INDEX and PREAMBLE_RECEIVED_TARGET_POWER.

The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI= 1 + s_id + 14*t_id + 14*X*f_id + 14*X*Y*ul_carrier_id

where s_id is the index of the first OFDM symbol of the specified PRACH (0 ≤ s_id < 14), t_id is the index of the first slot of the specified PRACH in a system frame (0 ≤ t_id < X), f_id is the index of the specified PRACH in the frequency domain (0 ≤ f_id < Y), and ul_carrier_id is the UL carrier used for Msg1 transmission (0 for normal carrier, and 1 for SUL carrier). The values X and Y are specified in TS 38.213 [6].

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if 'multiple preamble transmission' has been signalled:

2>	start the ra-ResponseWindow at the start of the first PDCCH occasion after a fixed duration of X symbols (specified in TS 38.213 [6]) from the end of the first preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI(s) while ra-ResponseWindow is running.

1>	else if the contention free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	start the bfr-ResponseWindow at the start of the first PDCCH occasion after a fixed duration of X symbols (specified in TS 38.213 [6]) from the end of the preamble transmission;

2>	monitor the PDCCH of the SpCell for response to beam failure recovery request identified by the C-RNTI while bfr-ResponseWindow is running.

1>	else:

2>	start the ra-ResponseWindow at the start of the first PDCCH occasion after a fixed duration of X symbols (specified in TS 38.213 [6]) from the end of the preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a Backoff Indicator subheader:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the Backoff Indicator subheader using Table 7.2-1.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see subclause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for the SI request to upper layers.

3>	else:

4>	if 'multiple preamble transmission' has been signalled:

5>	stop transmitting remaining preambles, if any.

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see subclause 5.2);

5>	indicate the ra-PreambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) * powerRampingStep);

5>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the common PRACH preambles:

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

1>	if ra-ResponseWindow expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received; or

1>	if bfr-ResponseWindow expires and if the PDCCH addressed to the C-RNTI has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = ra-PreambleTx-Max-bfr+ 1:

3>	consider the random access procedure unsuccessfully completed:

2>	if PREAMBLE_TRANSMISSION_COUNTER = ra-PreambleTx-Max + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers.

3>	else if the Random Access Preamble is transmitted on a SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if in this Random Access procedure, the Random Access Preamble was selected by MAC among the common PRACH preambles:

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	delay the subsequent Random Access Preamble transmission by the backoff time.

2>	perform the Random Access Resource selection procedure (see subclause 5.1.2).

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response transmission.

## 5.17	Beam Failure Recovery Request procedure

The beam failure recovery request procedure is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by the lower layers and indicated to the MAC entity.

RRC configures the following parameters for the beam failure recovery request procedure:

- beamFailureRecoveryTimer

- beamFailureInstanceMaxCount

- beamFailureCandidateBeamThreshold

- candidateBeam-RS-List

A beam recovery failure request shall be triggered if beam failure is detected and there are candidate beam available.

The MAC entity considers beam failure as detected if consecutive number of beam failure instance indicated from lower layer reaches beamFailureInstanceMaxCount.

The MAC entity considers there are candidate beam available if the L1-RSRP of any reference signal in candidate-Beam-RS-List is above beamFailureCandidateBeamThreshold.

The MAC entity shall:

1>	if beam failure recovery request is triggered:

2>	start beamFailureRecoveryTimer;

2>	initiate a Random Access procedure (see subclause 5.1) on the SpCell.

1>	if the beamFailureRecoveryTimer expires:

2>	indicate beam failure recovery request failure to upper layers.

1>	if downlink assignment or uplink grant on the PDCCH addressed for the C-RNTI has been received:

2>	stop and reset beamFailureRecoveryTimer;

2>	consider the Beam Failure Recovery Request procedure successfully completed.