3GPP TSG-RAN WG2 NR AH#1801	Tdoc R2-1800707

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.3.2.1

Source:	Ericsson

Title:	t-Reassembly in RLC

Document for:	Discussion, Decision

# Introduction

In NR variables for RLC UM and AM have been defined to keep track of the upper and lower sides of the window. I.e. when a packet is received, the RLC SDU SN value is compared against the variables which control the reassembly and receiving window.

In RLC, the timer t-Reassembly is defined to update receive window status and the associated variables for RLC SDUs. This action depends on PDUs placed in the reception buffer and the absence SDU segments. A missing RLC SDU segment involves the start of a timer t-Reassembly for handling receive window operation and the procedures for SDU segment Sequence Number book keeping.

In this contribution an error in the procedure for evaluation starting or (re)starting the timer t-Reassembly is explained and includes a suggested correction.

# Problem description

Below is an outline of the issue described below. In this example, RLC UM is used. However, the same error exists in the procedures for RLC AM.

In RLC UM operation SDUs (n+1) and (n+m) are missing segments in the reassembly window (fig 1):

38.322, sec. 5.2.2.2

Below procedures leading to this state is outlined.

1) SN(n+1) have been placed in the reception buffer, segments missing.

t-Reassembly not running:

…

else if x falls outside of the reassembly window:

-	update RX_Next_Highest to x + 1;

-	discard any UMD PDUs with SN that falls outside of the reassembly window;

-	if RX_Next_Reassembly falls outside of the reassembly window:

-	set RX_Next_Reassembly to the SN of the first SN >= (RX_Next_Highest – UM_Window_Size) that has not been reassembled and delivered to upper layer.

RX_Next_Highest = SN(n+2), RX_Next_Reassembly = SN(n+1)

…

-	if t-Reassembly is not running (includes the case when t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest > RX_Next_Reassembly + 1; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = x before the last byte of all received segments of this RLC SDU:

-	start t-Reassembly;

-	set RX_Timer_Trigger to RX_Next_Highest.

RX_Timer_Trigger = SN(n+2), t-Reassembly started

2) SN(n+m) have been placed in the reception buffer, segments missing, t-Reassembly running:

…

-	else if x falls outside of the reassembly window:

-	update RX_Next_Highest to x + 1;

-	discard any UMD PDUs with SN that falls outside of the reassembly window;

-	if RX_Next_Reassembly falls outside of the reassembly window:

-	set RX_Next_Reassembly to the SN of the first SN >= (RX_Next_Highest – UM_Window_Size) that has not been reassembled and delivered to upper layer.

RX_Next_Highest = SN(n+m)+1, RX_Next_Reassembly = SN(n+1), t-Reassembly running

…

-	if t-Reassembly is running:

-	if RX_Timer_Trigger <= RX_Next_Reassembly; or

-	if RX_Timer_Trigger falls outside of the reassembly window and RX_Timer_Trigger is not equal to RX_Next_Highest; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is no missing byte segment of the RLC SDU associated with SN = RX_Next_Reassembly before the last byte of all received segments of this RLC SDU:

-	stop and reset t-Reassembly.

No action

3) All segments of RLC SDU with SN = n+1 (= RX_Next_Reassembly) are received

…

-	if all byte segments with SN = x are received:

-	reassemble the RLC SDU from all byte segments with SN = x, remove RLC headers and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Next_Reassembly:

-	update RX_Next_Reassembly to the SN of the first SN > current RX_Next_Reassembly that has not been reassembled and delivered to upper layer.

…

RX_Next_Reassembly = SN(n+m), t-Reassembly is running

…

if t-Reassembly is running:

-	if RX_Timer_Trigger <= RX_Next_Reassembly; or

-	if RX_Timer_Trigger falls outside of the reassembly window and RX_Timer_Trigger is not equal to RX_Next_Highest; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is no missing byte segment of the RLC SDU associated with SN = RX_Next_Reassembly before the last byte of all received segments of this RLC SDU:

-	stop and reset t-Reassembly.

…

RX_Timer_Trigger < RX_Next_Reassembly, stop and reset t-Reassembly.

…

if t-Reassembly is not running (includes the case when t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest > RX_Next_Reassembly + 1; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = x before the last byte of all received segments of this RLC SDU:

-	start t-Reassembly;

RX_Next_Highest = RX_Next_Reassembly + 1= SN(n+m)+1. All segments of RLC SDU with "SN = x = n+1 NOT equal to current RX_Next_Reassembly = SN(n+m) have been received => t-Reassembly not started.

Figure 1

As can be seen above, although segments for RLC SDU with SN(n+m) are missing, t-Reassembly is not started due to that the criteria in “if RX_Next_Highest = RX_Next_Reassembly + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = x before the last byte of all received segments of this RLC SDU:” acts on SN=x.

Evaluating toward the missing byte segments of the RLC SDU associated with SN= RX_Next_Reassembly would however solve this as this would render the condition TRUE and as a result start t-Reassembly.

An accompanying CR can be found in R2-1800706

Correct the reference of the associated RLC SDU from SN=x to RX_Next_Reassembly.

# Conclusion

Based on the discussion in section 2 we propose the following:

Correct the reference of the associated RLC SDU from SN=x to RX_Next_Reassembly.

# References

3GPP TS 38.322, v 15.0.0