3GPP TSG-RAN WG2 NR Ad hoc 1801	R2-1800967

Vancouver, Canada, 22 – 26 January 2018	Revision of R2-1707989

Agenda item:	10.3.2.3

Source:	Nokia, Mediatek, Nokia Shanghai Bell

Title:	RLC handling upon duplication deactivation

WID/SID:	NR_newRAT - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, the RLC behaviour upon deactivation of duplication is briefly discussed.

# 2	Deactivation of Duplication

When duplication is deactivated and PDCP stops feeding the RLC entity, comes the question what to do with the data possibly buffered at RLC and awaiting transmission. As suggested in a companion contribution [R2-1707990], PDCP should issue a discard notification to lower layers. Such a solution heavily relies on implementation and discarding the corresponding RLC SDUs may not always be possible (CU/DU split on the network side, pre-processing on the UE side) and other means to minimise the overhead needs to be considered. For instance:

1.	Transmit the RLC PDU header fields without the associated SDUs to synchronise the RLC entities; or

2.	Flushing all buffered data at RLC and reset of Tx variables in transmitter and Rx variables in receiver.

NOTE:	The corresponding Rx and Tx variables are not reset if duplication is still active in the reverse link.

Both alternatives rely RLC being notified that something needs to be done. Furthermore, they ensure that when the duplication is reactivated, possible HFN desync in the receiving PDCP entity could be avoided. For instance, this could be caused by a possible tail segment of a PDCP PDU waiting for transmission once duplication is deactivated.

Proposal 1: discuss the possible means of reducing overhead when duplication is deactivated.

# 3	Conclusion

This contribution has discussed the impacts of duplication to RLC and suggests the following:

Proposal 1: discuss the possible means of reducing overhead when duplication is deactivated.