3GPP TSG-RAN WG2 AH1801 	R2-1801174

# Vancouver, Canada, 22th Jan – 26th Jan 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | I031 Measurement gap configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-01-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To introduce measurement gap configuration in 38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The measuremnet gap configuration procedure, addressing EN-DC, is introduced in 38.331. The needed IEs are also added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement gap configuration for NR FR2 frequencies, in case UE supports indpendent measurement gaps per FR, will not be available |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.9, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1>	if measGapConfigFR2 is set to setup:

2>	if a measurement gap configuration for FR2, measGapConfigFR2, is already setup, release the measurement gap configuration for FR2;

2>	setup the measurement gap configuration for FR2 indicated by the measGapConfigFR2 in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition (SFN and subframe of SCG cells on FR2):

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [x];

1>	else if measGapConfigFR2 is set to release:

2>	release the measurement gap configuration for FR2, measGapConfigFR2;

NOTE 1:	If the UE is operating in EN-DC, the UE applies the same measurent gap pattern configured by the E-UTRAN for NR FR1 frequencies.

NOTE 2: If the UE is opeating in EN-DC and measGapConfigFR2 is not configured, the UE applies the same measurement gap pattern configured by the E-UTRAN for both FR1 and FR2.

END OF CHANGES

START OF CHANGES

### 6.3.2	Radio resource control information elements

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEAS-CONFIG-START

MeasConfig ::=							SEQUENCE {

-- Measurement objects

measObjectToRemoveList					MeasObjectToRemoveList											OPTIONAL,

measObjectToAddModList					MeasObjectToAddModList											OPTIONAL,

-- Reporting configurations

reportConfigToRemoveList				ReportConfigToRemoveList										OPTIONAL,

reportConfigToAddModList				ReportConfigToAddModList										OPTIONAL,

-- Measurement identities

measIdToRemoveList						MeasIdToRemoveList												OPTIONAL,

measIdToAddModList						MeasIdToAddModList												OPTIONAL,

-- Other parameters

-- s-Measure config

s-MeasureConfig							CHOICE {

ssb-rsrp								RSRP-Range,

csi-rsrp								RSRP-Range

} 																										OPTIONAL,

quantityConfig						QuantityConfig														OPTIONAL,

-- Placehold for measGapConfig

measGapConfigFR1							MeasGapConfig 				OPTIONAL,

measGapConfigFR2							MeasGapConfig 				OPTIONAL

}

MeasObjectToRemoveList ::=				SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=					SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=			SEQUENCE (SIZE (1..maxNrofReportConfigId)) OF ReportConfigId

-- TAG-MEAS-CONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

measGapConfig Used to setup and release measurement gaps. For EN-DC, only measGapConfigFR2 can be configured by NR, if the UE is capable of per FR measurement gap configuration.

measIdToAddModList List of measurement identities.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for PCell or PSCell (when UE is in EN-DC) RSRP measurement controlling when the UE is required to perform measurements associated to neighbouring cells. Choice of ssb-rsrp corresponds to cell RSRP based on SS/PBCH block and choice of csi-rsrp corresponds to cell RSRP of CSI-RS.

END OF CHANGES

START OF CHANGES

### 6.3.2	Radio resource control information elements

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/ release of measurement gaps.

MeasGapConfig information element

MeasGapConfig ::=                 CHOICE {

release                          NULL,

setup                            SEQUENCE {

gapOffset                         CHOICE {

--MGL = 6ms

gp0-r15                           INTEGER (0..39),

gp1-r15                           INTEGER (0..79),

--MGL = 3ms

gp2-r15                           INTEGER (0..39),

gp3-r15                           INTEGER (0..79),

--MGL = 6ms

gp4-r15                           INTEGER (0..39),

gp5-r15                           INTEGER (0..79),

--MGL = 4ms

gp6-r15                           INTEGER (0..19),

gp7-r15                           INTEGER (0..39),

gp8-r15                           INTEGER (0..79),

gp9-r15                           INTEGER (0..159),

-- MGL = 3ms

gp10-r15                          INTEGER (0..20),

gp11-r15                          INTEGER (0..159),

--MGL = 5.5ms

gp12-r15                          INTEGER (0..19),

gp13-r15                          INTEGER (0..39),

gp14-r15                          INTEGER (0..79),

gp15-r15                          INTEGER (0..159),

--MGL = 3.5ms

gp16-r15                          INTEGER (0..19),

gp17-r15                          INTEGER (0..39),

gp18-r15                          INTEGER (0..79),

gp19-r15                          INTEGER (0..159),

--MGL = 1.5ms

gp20-r15                          INTEGER (0..19),

gp21-r15                          INTEGER (0..39),

gp22-r15                          INTEGER (0..79),

gp23-r15                          INTEGER (0..159)

}

}

}

MeasGapConfig field descriptions

gapOffset Value gapOffset of gp0 corresponds to gap offset of Gap Pattern Id “0” with MGRP = 40ms, gapOffset of gp1 corresponds to gap offset of Gap Pattern Id “1” with MGRP = 80ms, gapOffset of gp2 corresponds to gap offset of Gap Pattern Id “2” with MGRP = 40ms and MGL = 3ms, gapOffset of gp3 corresponds to Gap Pattern Id “3” with MGRP = 80ms and MGL = 3ms, gapOffset of gp4 corresponds to gap offset of Gap Pattern Id “4” with MGRP = 40ms and MGL = 6ms, gapOffset of gp5 corresponds to Gap Pattern Id “5” with MGRP = 80ms and MGL = 6ms, gapOffset of gp6 corresponds to gap offset of Gap Pattern Id “6” with MGRP = 20ms and MGL = 4ms, gapOffset of gp7 corresponds to Gap Pattern Id “7” with MGRP = 40ms and MGL = 4ms, gapOffset of gp8 corresponds to gap offset of Gap Pattern Id “8” with MGRP = 80ms and MGL = 4ms, gapOffset of gp9 corresponds to Gap Pattern Id “9” with MGRP = 160ms and MGL = 4ms, gapOffset of gp10 corresponds to gap offset of Gap Pattern Id “10” with MGRP = 20ms and MGL = 3ms, gapOffset of gp11 corresponds to Gap Pattern Id “11” with MGRP = 160ms and MGL = 3ms, gapOffset of gp12 corresponds to gap offset of Gap Pattern Id “12” with MGRP = 20ms and MGL = 5.5ms, gapOffset of gp13 corresponds to Gap Pattern Id “13” with MGRP = 40ms and MGL = 5.5ms, gapOffset of gp14 corresponds to gap offset of Gap Pattern Id “14” with MGRP = 80ms and MGL = 5.5ms, gapOffset of gp15 corresponds to Gap Pattern Id “15” with MGRP = 160ms and MGL = 5.5ms, gapOffset of gp16 corresponds to gap offset of Gap Pattern Id “16” with MGRP = 20ms and MGL = 3.5ms, gapOffset of g1p7 corresponds to Gap Pattern Id “17” with MGRP = 40ms and MGL = 3.5ms, gapOffset of gp18 corresponds to gap offset of Gap Pattern Id “18” with MGRP = 80ms and MGL = 3.5ms, gapOffset of gp19 corresponds to Gap Pattern Id “19” with MGRP = 160ms and MGL = 3.5ms, gapOffset of gp20 corresponds to gap offset of Gap Pattern Id “20” with MGRP = 20ms and MGL = 1.5ms, gapOffset of gp21 corresponds to Gap Pattern Id “21” with MGRP = 40ms and MGL = 1.5ms, gapOffset of gp22 corresponds to gap offset of Gap Pattern Id “22” with MGRP = 80ms and MGL = 1.5ms, gapOffset of gp23 corresponds to Gap Pattern Id “23” with MGRP = 160ms and MGL = 1.5ms,

END OF CHANGES