3GPP TSG-RAN WG2 AH-1801	R2-1801325

Vancouver, Canada, 22nd – 26th January 2018

Title:	Draft LS to RAN4 on reportAmount and reportInterval

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson [to be changed to RAN2]

To:	RAN4

Contact Person:

#### Name:	Icaro Da Silva

E-mail Address:	icaro.leonardo.da.silva@ericsson.com

1. Overall Description

The parameters reportAmount and reportInterval determine the number of reports and the interval between each measurement report for both event triggered and periodical reports, as specified in reportConfig IE. In LTE, it is possible to configure the UE with 8 different reportAmount values, as {r1, r2, r4, r8, r16, r32, r64, infinity}. Moreover, reportInterval can take the following values {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60, spare3, spare2, spare1}.

In NR, reportAmount and reportInterval fields in reportConfigNR are yet marked as FFS. We believe that the number of reports seem to be sufficient in LTE and hence we propose to use the same values for reportAmount in NR as well. However, for reportInterval, there can be some further refinements as for instance intervals of more than 30 minutes between measurement reports seems unrealistic in many scenarios, and on the other hand there are not any values for intervals between 10 ms and 1 minute. Hence, we propose to use {120 ms, 240 ms, 480 ms, 640 ms, 1024 ms, 2048 ms, 5120 ms, 10240 ms, 20480 ms, 40960 ms, 1 min, 5 min, 10 min, 30 min, spare2, spare1} range for reportInterval instead

Therefore, we solicit RAN4’s view on the above parameters in relation to NR and EN-DC.

2. Actions:

ACTION: 	RAN2 respectfully asks RAN4 to consider the following proposals:

- Defining the same values for reportAmount field as in LTE, i.e, {r1, r2, r4, r8, r16, r32, r64, infinity}

- Defining the following values for reportInterval in NR {120 ms, 240 ms, 480 ms, 640 ms, 1024 ms, 2048 ms, 5120 ms, 10240 ms, 20480 ms, 40960 ms, 1 min, 5 min, 10 min, 30 min, spare2, spare1}.

3. Date of Next RAN2 Meetings:

RAN2#

RAN2#