3GPP TSG-RAN WG2 NR Ad Hoc #3		        R2-1801409

Vancouver, Canada, 22 – 26 January, 2018

Agenda Item	10.3.3.5(NR_newRAT-Core)

Source 	LG Electronics Inc.

Title 	PDCP duplication for SRBs

Document for 	Discussion and Decision

Introduction

This document discusses the applicability of packet duplication to SRBs.

Discussion

RAN2 agreed that the packet duplication is also applied to SRBs. However, it is not clear whether the packet duplication is applied to all types of SRBs or only specific type of SRBs.

Currently, there are four types of SRBs defined in NR, i.e. SRB0, SRB1, SRB2, and SRB3. For each type of SRBs, the applicability of packet duplication is provided below.

- SRB0

  - The packet duplication is not applicable because the SRB0 does not use PDCP.

- SRB1

  - Most RRC messages are transmitted via SRB1, and the main target for SRB duplication is SRB1. Thus, packet duplication should be applied to SRB1.

- SRB2

  - The SRB2 is used to transport low priority NAS data. It is possible to apply packet duplication for SRB2, but the improving reliability for SRB2 is not critical as it transports “low priority” data.

- SRB3

  - The SRB3 is used to transport SCG RRC message directly to the UE. The reason for introducing the SRB3 is to improve the latency of SCG RRC message delivery rather than to improve the reliability. For this reason, RAN2 has not introduced SCG SRB split bearer. Therefore, packet duplication is not applicable to SRB3.

Based on the above reasoning, the packet duplication is not applicable to SRB0 and SRB3, but applicable to SRB1 and SRB2. For SRB2, however, we think packet duplication is not so essential as explained above.

Thus, we propose that the packet duplication is applied only to SRB1.

Proposal: SRB packet duplication is applied only to SRB1.

Proposal

For SRB packet duplication, we have following proposal:

Proposal: SRB packet duplication is applied only to SRB1.