# 3GPP TSG RAN WG2 Meeting  Ad hoc1801                                               R2-1801534

# Vancouver, Canada, 22nd– 26th January 2018		    revision of R2-1800949

Agenda item:		10.4.3.2

Source:			Intel Corporation

Title:		I081: NR BPC enhancements for handling EN-DC and NR SA

Document for:	 	Discussion and decision

# Introduction

In this contribution, we discuss the potential issue with sharing the NR BPC for both NR SA and for EN-DC/NE- DC, and provide solutions for discussion.

# Discussion

The BPC table definition as part of the UE EN-DC capabilities in TS38.331 is defined with the intention that the same tables is to be used for transferring the BPC capabilities for both EN/NE-DC and NR SA (including potentially NR-NR DC BPC capabilities).

This unified BPC table approach can bring up some issues.

It is still FFS whether the RF EN-DC BCs will have indexed links to the unified BPC table. Similarly it is still FFS if the NR SA RF BCs will have links to the unified BPC table.

Due to potential sharing of UE resources between LTE and NR while in EN-DC, the baseband capabilities could differ between EN-DC and NR SA operation, even when the set of NR carriers are the same for the EN-DC and NR SA configurations. While the capabilities of BPC entries meant for EN-DC are usually lower compared to the capabilities that are supported when in NR SA mode, the other way is not precluded.

In the case where the RF BCs do not have explicit links to the BPC entries, it is assumed that the MN and SN can find the NR BPC capabilities based on the number of NR carriers and other NR parameters. This is the case even if the LTE part of the BPC has explicit links to the NR part of the BPC (which is the unified NR BPC).

Agreements 1 Define in 38.331 that a given BPC is applicable in CA band combinations (for SA operation) which consist of bands 1) with equal bandwidth class, and 2) equal or higher MIMO layer capability. This applicability rule also applies to fallback combinations which may not be signalled.

Also the above RAN2 agreement on having the UE skip reporting fallback entries for BPC can mean that the UE may not be able to report the NR BPC entries meant for EN-DC if a corresponding NR SA entry can cover this as fallback. This can happen when the UE has to report both the EN-DC and NR SA capabilities.

If the same unified BPC is used for both EN-DC and NR SA, then for a NR SA gNB to be able to find the BPC entries that are meant for NR SA operation:

- The NR SA gNB has to comprehend the EN-DC related structures and possibly parse through the LTE BPC part to find the BPC entries meant for EN-DC operation.

We cannot preclude the possibility of having NR SA deployments without the necessity to have the EN-DC configuration deployed, and hence cannot mandate the NR SA gNBs to implement the EN-DC features.

In such a case:

- The RF BCs for NR SA should have explicit links to the NR BPC entries meant for NR SA operation

- Or add a field in each of the unified NR BPC entries indicating the applicability of the BPC entry to EN-DC or NR SA or possibly other future configuration.

We believe the latter option is a simple solution that has the potential for future extensibility.

Proposal 1: The UE is allowed to include the fallback BPCs of a higher capability BPC in cases where the fallback BPC is needed to be used for EN-DC purposes.

Proposal 2: Each of the NR BPC entries defined by the IE BasebandProcessingCombination to include a field which provides the information on whether the entry can be used for EN-DC or NR SA or for both.

# Conclusion

Proposal 1: The UE is allowed to include the fallback BPCs of a higher capability BPC in cases where the fallback BPC is needed to be used for EN-DC purposes.

Proposal 2: Each of the NR BPC entries defined by the IE BasebandProcessingCombination to include a field which provides the information on whether the entry can be used for EN-DC or NR SA or for both.

Corresponding TPs are shown in section 4, we are happy to provide CRs if needed.

# Text proposals

TS38.331

/*** skip unrelated part***/

## 5.6	UE capabilities

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

#### Editor’s Note: Targeted for completion in June 2018.5.6.1.2	Initiation

Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.3	Reception of the UECapabilityEnquiry by the UE

Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.4	Compilation of band combinations supported by the UE

The UE shall:

1>	if includes requestedFreqBandList:

2>	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message,  only consisting of bands included in requestedFreqBandList, and prioritized in the order of requestedFreBandList, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

2>	for each band combination included in the candidate list:

3>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of candidates as specified in TS 38.306 [xx]:

4>	remove the band combination from the list of candidates;

2>	include all band combinations in the candidate list into supportedBandCombination;

1>	else:

2> include all band combinations supported by the UE into supportedBandCombination, excluding fallback band combinations with the same capabilities of another band combination included in the list of band combinations supported by the UE.

#### 5.6.1.5	Compilation of baseband processing combinations supported by the UE

The UE shall:

1>	for each band combination included in supportedBandCombination:

2>	include the baseband processing combination supported for the band combination into supportedBasebandProcessingCombination, unless it is already included;

2> include for this baseband processing combination, all the scenarios where this capability is applicable using the IE applicabilityOfBPC where bit0 is set if this capability applies to EN-DC scenario, where bit1 is set if this capability applies to NE-DC scenario, where bit2 is set if this capability applies to NR SA scenario.

2>	if there are the fallback baseband processing combinations of this baseband processing combination as specified in TS 38.306 [xx] for which supported baseband capabilities are different from this baseband processing combination:

3>	include the fallback baseband processing combinations into supportedBasebandProcessingCombination while adding the corresponding information of the fallback baseband processing capability applicability scenarios.

### 6.3.3	UE capability information elements

/*** skip unrelated part***/

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [yy].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=	SEQUENCE {

measParameters-MRDC			MeasParameters-MRDC,

rf-Parameters-MRDC			RF-Parameters-MRDC,

phyLayerParameters-MRDC		PhyLayerParameters-MRDC

-- FFS on other parameters

}

RF-Parameters-MRDC ::= SEQUENCE {

supportedBandCombination	BandCombinationList

-- FFS on other parameters

}

PhyLayerParameters-MRDC ::= SEQUENCE {

supportedBasebandProcessingCombination-MRDC		BasebandProcessingCombination-MRDC

-- FFS if supportedBasebandProcessingCombination-MRDC is included here or BandCombinationList

-- FFS on other parameters

}

BasebandProcessingCombination-MRDC ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF LinkedBasebandProcessingCombination

LinkedBasebandProcessingCombination ::= SEQUENCE {

basebandProcessingCombinationIndex			BasebandProcessingCombinationIndex,

basebandProcessingCombinationLinkedIndex	SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationIndex

}

BasebandProcessingCombinationIndex ::= INTEGER (1..maxBasebandProcComb)

MeasParameters-MRDC ::= SEQUENCE {

intraCarrierConcurrentMeas	ENUMERATED {supported}	OPTIONAL,

independentGapConfig		ENUMERATED {supported}	OPTIONAL,

sstd-MeasType1				ENUMERATED {supported}	OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [yy].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters,						-- FFS OPTIONAL

mac-Parameters					MAC-Parameters,						-- FFS OPTIONAL

phyLayerParameters				PhyLayerParameters,

rf-Parameters					RF-Parameters,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PhyLayerParameters ::=	SEQUENCE {

supportedBasebandProcessingCombination		SupportedBasebandProcessingCombination

-- FFS on other parameters

}

RF-Parameters ::= SEQUENCE {

supportedBandListNR					SupportedBandListNR,

supportedBandCombination			BandCombinationList,

intraBandAsyncFDD					ENUMERATED {supported}	OPTIONAL

-- FFS Whether intraBandAsyncFDD is included per UE or per band combination

}

SupportedBandListNR ::=	SEQUENCE (SIZE (1..maxBands)) OF BandNR

SupportedBandCombination ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

SupportedBasebandProcessingCombination ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombination

BasebandProcessingCombination ::= SEQUENCE {

applicabilityOfBPC				BIT STRING (SIZE (maxApplicableBPCEntries)),

basebandParametersPerBand		SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BasebandParametersPerBand

-- FFS on other parameters

}

BasebandParametersPerBand ::= SEQUENCE {

ca-BandwidthClassDL			CA-BandwidthClass,

ca-BandwidthClassUL			CA-BandwidthClass,

basebandParametersPerCC		SEQUENCE (SIZE (1..maxNrofSCells)) OF BasebandParametersPerCC,

supportedBW-PerCC				BW-PerCC

-- FFS on the need (e.g. if ca-BandwidthClass is sufficient to cover BW-PerCC)

-- FFS on other parameters

}

BasebandParametersPerCC ::= SEQUENCE {

supportedMIMO-CapabilityDL		MIMO-Capability						OPTIONAL,

supportedMIMO-CapabilityUL		MIMO-Capability						OPTIONAL,

modulationOrder					ModulationOrder,

subCarrierSpacing				SubCarrierSpacing

-- FFS if modulationOrder and subCarrierSpacing are included per Band or per CC

-- FFS on other parameters

}

BandNR ::=	SEQUENCE {

bandNR							FreqBandIndicatorNR,

supportedMIMO-CapabilityDL		MIMO-Capability						OPTIONAL,

supportedMIMO-CapabilityUL		MIMO-Capability						OPTIONAL

-- FFS on other parameters

}

CA-BandwidthClass ::= ENUMERATED {a, b, c, d, e, f, ...}

MIMO-Capability ::= SEQUENCE {

-- FFS on the parameters

}

ModulationOrder ::= SEQUENCE {

-- FFS on the parameters

}

SubCarrierSpacing ::= SEQUENCE {

-- FFS on the parameters

}

PDCP-Parameters ::= SEQUENCE {

dataRateDRB-IP		ENUMERATED {kbps64, spare6, spare5, spare4, spare3, spare2, spare1, spare0}		OPTIONAL,

supportedROHC-Profiles	SEQUENCE {

profile0x0000		BOOLEAN,

profile0x0001		BOOLEAN,

profile0x0002		BOOLEAN,

profile0x0003		BOOLEAN,

profile0x0004		BOOLEAN,

profile0x0006		BOOLEAN,

profile0x0101		BOOLEAN,

profile0x0102		BOOLEAN,

profile0x0103		BOOLEAN,

profile0x0104		BOOLEAN

},

maxNumberROHC-ContextSessions	ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64, cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles			ENUMERATED {supported}	OPTIONAL,

continueROHC-Context			ENUMERATED {supported}	OPTIONAL,

outOfOrderDelivery				ENUMERATED {supported}	OPTIONAL,

shortSN							ENUMERATED {supported} 	OPTIONAL,

volteOverNR-PDCP				ENUMERATED {supported}	OPTIONAL

}

RLC-Parameters ::= SEQUENCE {

amWithShortSN					ENUMERATED {supported}	OPTIONAL,

umWithShortSN					ENUMERATED {supported}	OPTIONAL,

umWIthLongSN					ENUMERATED {supported}	OPTIONAL

}

MAC-Parameters ::= SEQUENCE {

lcp-Restriction					ENUMERATED {supported}	OPTIONAL,

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

numberOfSR-Configurations		ENUMERATED {n2, n3, n4, ...}	OPTIONAL, -- FFS value range

numberOfConfiguredGrantConfigurations	ENUMERATED {n2, n3, n4, ...}	OPTIONAL -- FFS value range

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxApplicableBPCEntries					INTEGER ::= ffsValue		-- Maximum number of BPC types

maxBandComb								INTEGER ::=	ffsValue		-- Maximum number of DL band combinations

maxBasebandProcComb						INTEGER ::=	ffsValue		-- Maximum number of base band processing combinations

maxNrofServingCells						INTEGER ::=	16		-- Max number of serving serving cells (SpCell + SCells) per cell group

maxNrofSCells							INTEGER ::=	15		-- Max number of secondary serving cells per cell group

maxNrofCellMeas							INTEGER ::=	ffsValue		-- Maximum number of entries in each of the cell lists in a measurement object

maxNrofSS-BlocksToAverage				INTEGER ::= ffsValue		-- Max number for the (max) number of SS blocks to average to determine cell measurement

maxNrofCSI-RS-ResourcesToAverage			INTEGER ::= ffsValue		-- Max number for the (max) number of CSI-RS to average to determine cell measurement

maxNrofSR-CongigPerCellGroup			INTEGER ::= 8		-- Maximum number of SR configurations per cell group

maxLCG-ID								INTEGER ::= 7		-- Maximum value of LCG ID

macLC-ID								INTEGER ::= ffsValue		-- Maximum value of Logical Channel ID

maxNrofTAGs								INTEGER ::=	4		-- Maximum number of Timing Advance Groups

maxNrofTAGs-1							INTEGER ::=	3		-- Maximum number of Timing Advance Groups minus 1

maxNrofBandwidthParts					INTEGER ::= 4		-- Maximum number of BWPs per serving cell

maxNrofBandwidthParts-1					INTEGER ::= 3		-- Maximum number of BWPs per serving cell minus 1

maxSymbolIndex							INTEGER ::= 13		-- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofPhysicalResourceBlocks			INTEGER ::= 275		-- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1			INTEGER ::= 274		-- Maximum number of PRBs

maxNrofPhysicalResourceBlocksTimes4		INTEGER ::= ffsValue	-- Maximum number of PRBs (used to reference PRBs in another subcarrier spacing)

maxNrofControlResourceSets 				INTEGER ::= ffsValue 	-- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1			INTEGER ::= 12  	-- Max number of CoReSets configurable on a serving cell minus 1

maxCoReSetStartSymbol					INTEGER ::= ffsValue		-- Highest possible start symbol for a control resource set

maxCoReSetDuration						INTEGER ::= 3		-- Max number of OFDM symbols in a control resource set

maxNrofSearchSpacesPerCoReSet			INTEGER ::= ffsValue		-- Max number of search spaces configurable per Control Resource Set

maxSFI-DCI-PayloadSize					INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1				INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxINT-DCI-PayloadSize					INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1				INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns				INTEGER ::= 4		-- Max number of rate matching patterns that may be configured

maxRateMatchPatternId				INTEGER ::= 7		-- Max allowed rate matching pattern ID

maxNrofCSI-Reports						INTEGER ::= ffsValue 	-- Maximum number of report configurations

maxNrofCSI-Reports-1	 				INTEGER ::= ffsValue 	-- Maximum number of report configurations minus 1

maxNrofReportConfigIdsPerTrigger		INTEGER ::= 16	 	-- Maximum number of report configurations per reportTrigger

maxNrofCSI-ResourceConfigurations		INTEGER ::= ffsValue		-- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1		INTEGER ::= ffsValue		-- Maximum number of resource configurations minus 1

maxNrofCSI-ResourceSets					INTEGER ::= ffsValue		-- Maximum number of resource sets per resource configuration

maxNrofCSI-ResourceSets-1				INTEGER ::= ffsValue		-- Maximum number of resource sets per resource configuration minus 1

maxNrofNZP-CSI-RS-Resources				INTEGER ::= ffsValue		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1			INTEGER ::= ffsValue		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-Resources				INTEGER ::= ffsValue		-- Maximum number of Zero-Power (NZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1			INTEGER ::= ffsValue		-- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1

maxNrofCSI-IM-Resources					INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1				INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-ResourcesPerSet			INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax in 38.214

maxNrofCSI-IM-ResourcesPerSet-1			INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources per set minus 1. See CSI-IM-ResourcePerSetMax

maxNrofSSB-Resources					INTEGER ::= 64		-- Maximum number of SSB resources in a resource set

maxNrofSSB-Resources-1					INTEGER ::= 63		-- Maximum number of SSB resources in a resource set minus 1

maxNrofCSI-RS-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of CSI-RS resources per resource set

maxNrofCSI-MeasId						INTEGER ::= ffsValue		-- Maximum number of link configurations

maxNrofCSI-MeasId-1						INTEGER ::= ffsValue		-- Maximum number of link configurations minus 1

maxNrofCSI-RS-ResourcesRRM				INTEGER ::= ffsValue		-- Maximum number of CSI-RS resources for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1			INTEGER ::= ffsValue		-- Maximum number of CSI-RS resources for an RRM measurement object minus 1

maxNrofObjectId							INTEGER ::= ffsValue		-- Maximum number of configured measurement objects

maxNrOfRA-PreamblesPerSSB				INTEGER ::=	ffsValue		-- Maximum number of Random Access Preamble value per SSB

maxNrofReportConfigId					INTEGER ::= ffsValue		-- Maximum number of reporting configurations

maxNrofMeasId							INTEGER ::= ffsValue		-- Maximum number of configured measurements

maxNroQuantityConfig					INTEGER	::= 2		-- Maximum number of quantity configurations

maxNrofSRS-ResourceSets					INTEGER ::= ffsValue		-- Maximum number of SRS resource sets.

maxNrofSRS-ResourceSets-1				INTEGER ::= ffsValue		-- Maximum number of SRS resource sets minus 1.

maxNrofSRS-Resources					INTEGER ::= ffsValue		-- Maximum number of SRS resources in an SRS resource set.

maxNrofSRS-Resources-1					INTEGER ::= ffsValue		-- Maximum number of SRS resources in an SRS resource set minus 1.

maxRA-PreambleIndex						INTEGER ::= ffsValue		-- Maxximum value of Random Access Preamble Index

maxRAT-CapabilityContainers				INTEGER ::= ffsValue		-- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands					INTEGER ::= ffsValue		-- Maximum number of simultaneously aggregated bands

maxNrofSlotFormatCombinationsPerSet		INTEGER ::= ffsValue		-- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1	INTEGER ::= ffsValue		-- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofPUCCH-ResourceSets				INTEGER ::= 4		-- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1				INTEGER ::= 3		-- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-ResourcesPerSet-1			INTEGER ::= 7		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet minus 1.

maxNrofPUCCH-P0-PerSet					INTEGER ::= 8		-- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReference-RSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReference-RSs-1	INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1.

maxNrofP0-PUSCH-AlphaSets				INTEGER ::= 30		-- Maximum number of P0-pusch-alpha-sets (see 38,213, section 7.1)

maxNrofP0-PUSCH-AlphaSets-1				INTEGER ::= 29		-- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, section 7.1)

maxNrofPUSCH-PathlossReference-RSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReference-RSs-1	INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1.

maxEARFCN								INTEGER ::= 262143	-- Highest value of extended E-UTRA EARFCN range

ffsmax		 							INTEGER ::= ffsValue

ffsRange		 						INTEGER ::= ffsValue

maxBands 								INTEGER ::= ffsValue

maxCellPrep		 						INTEGER ::= ffsValue

maxCellReport		 					INTEGER ::= ffsValue

maxCellSCG		 						INTEGER ::= ffsValue

maxDCIpayload		 					INTEGER ::= ffsValue

maxDRB		 							INTEGER ::= ffsValue

maxFreq		 							INTEGER ::= ffsValue

maxLCH									INTEGER ::= ffsValue

maxLCid 								INTEGER ::= ffsValue

maxNrofCSI-RS	 						INTEGER ::= ffsValue

maxNrofAggregatedCellsPerCellGroup 		INTEGER ::= ffsValue

maxNrofCandidateBeams 					INTEGER ::= ffsValue

maxNrofCSI-ReportConfig-1 				INTEGER ::= ffsValue

maxNrofCSI-ResrouceConfigurations 		INTEGER ::= ffsValue

maxNrofPCIsPerSMTC 						INTEGER ::= ffsValue

maxNrofPUCCH-PathlossReferenceRS-1 		INTEGER ::= ffsValue

maxNrofPUSCH-PathlossReferenceRS-1 		INTEGER ::= ffsValue

maxNrofQFIs 							INTEGER ::= ffsValue

maxNrofSchedulingRequestResoruces 		INTEGER ::= ffsValue

maxNrofSearchSpaces 					INTEGER ::= ffsValue

maxNrofSlotFormatCombinations 			INTEGER ::= ffsValue

maxNrofSlotFormatCombinations-1 		INTEGER ::= ffsValue

maxNrofSlotFormatsPerCombination 		INTEGER ::= ffsValue

maxNrofSpatialRelationInfos 			INTEGER ::= ffsValue

maxNrofSR-ConfigPerCellGroup 			INTEGER ::= ffsValue

maxNrofSRS-ResourcesPerSet 				INTEGER ::= ffsValue

maxNrofSRSTriggerStates 				INTEGER ::= ffsValue

maxNrofIndexesToReport 					INTEGER ::= ffsValue

maxNrofSSBs 							INTEGER ::= ffsValue

maxNrofTCI-StatesPDCCH 					INTEGER ::= ffsValue

maxNrof-TCI-RS-Sets						INTEGER ::= ffsValue

maxQFI 									INTEGER ::= ffsValue

maxQuantityConfigId 					INTEGER ::= ffsValue

maxRAcsirsResources 					INTEGER ::= ffsValue

maxRAssbResources 						INTEGER ::= ffsValue

maxReportConfigId 						INTEGER ::= ffsValue

maxRS-IndexReport 						INTEGER ::= ffsValue

maxSCellGroups 							INTEGER ::= ffsValue

ffsValue								INTEGER ::= 64

-- IE definitions introduced to not get warning at ASN.1 syntax check

AdditionalReestabInfoList ::=	ENUMERATED {ffsTypeAndValue}

AdditionalSpectrumEmission 	::=	ENUMERATED {ffsTypeAndValue}

ARFCN-ValueNR 				::=	ENUMERATED {ffsTypeAndValue}

BSR-Configuration ::=			ENUMERATED {ffsTypeAndValue}

CandidateRS-IndexInfoList ::=	ENUMERATED {ffsTypeAndValue}

CellIdentity ::=				ENUMERATED {ffsTypeAndValue}

CSI-RSIndex ::=					ENUMERATED {ffsTypeAndValue}

FilterCoefficient ::=			ENUMERATED {ffsTypeAndValue}

Hysteresis ::=					ENUMERATED {ffsTypeAndValue}

MeasGapConfig ::=				ENUMERATED {ffsTypeAndValue}

MeasObjectEUTRA ::=				ENUMERATED {ffsTypeAndValue}

MeasResultListEUTRA ::=			ENUMERATED {ffsTypeAndValue}

MeasResultSSTD ::=				ENUMERATED {ffsTypeAndValue}

PDUsessionID ::=				ENUMERATED {ffsTypeAndValue}

PhyCellNR ::=					ENUMERATED {ffsTypeAndValue}

PhysCellIdEUTRA ::=				ENUMERATED {ffsTypeAndValue}

PhysCellIdRange ::=				ENUMERATED {ffsTypeAndValue}

PhysicalCellId ::=				ENUMERATED {ffsTypeAndValue}

P-Max ::=						ENUMERATED {ffsTypeAndValue}

RadioBearerConfiguration ::=	ENUMERATED {ffsTypeAndValue}

RA-Resources ::=				ENUMERATED {ffsTypeAndValue}

ReportConfigEUTRA ::=			ENUMERATED {ffsTypeAndValue}

ReportInterval ::=				ENUMERATED {ffsTypeAndValue}

RRC-TransactionIdentifier ::=	ENUMERATED {ffsTypeAndValue}

RSRP-Range ::=					ENUMERATED {ffsTypeAndValue}

RSRQ-Range ::=					ENUMERATED {ffsTypeAndValue}

SchedulingRequestId ::=			ENUMERATED {ffsTypeAndValue}

ShortMAC-I ::=					ENUMERATED {ffsTypeAndValue}

SINR-Range ::=					ENUMERATED {ffsTypeAndValue}

SSB-Id ::=						ENUMERATED {ffsTypeAndValue}

TAG-Configuration ::=			ENUMERATED {ffsTypeAndValue}

TimeToTrigger ::=				ENUMERATED {ffsTypeAndValue}

UECapabilityInformation ::=	ENUMERATED {ffsTypeAndValue}

BW-PerCC ::=		ENUMERATED {ffsTypeAndValue}

CellsTriggeredList ::=		ENUMERATED {ffsTypeAndValue}

CellToSFI ::=		ENUMERATED {ffsTypeAndValue}

FFS_Value ::=		ENUMERATED {ffsTypeAndValue}

FreqBandIndicatorNR ::=		ENUMERATED {ffsTypeAndValue}

MBSFN-SubframeConfigList ::=		ENUMERATED {ffsTypeAndValue}

NumberOfRA-Preambles ::=		ENUMERATED {ffsTypeAndValue}

NZP-CSI-RS-ResourceConfigId ::=		ENUMERATED {ffsTypeAndValue}

PUCCH-resource-config-PF0 ::=		ENUMERATED {ffsTypeAndValue}

PUCCH-resource-config-PF1 ::=		ENUMERATED {ffsTypeAndValue}

SchedulingRequestResource-Config ::=		ENUMERATED {ffsTypeAndValue}

SlotFormatIndicator ::=		ENUMERATED {ffsTypeAndValue}

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

### –	End of NR-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

TS38.306

/*** skip unrelated part***/

### 4.7	Physical layer parameters

#### applicabilityOfBPC

This field provides information on how the capability of the BPC entry is to be used for. Bitstring where bit0 if set, applies to EN-DC operation, bit1 if set applies to NE-DC operation, and bit2 if set, applies to NR SA operation. Multiple bits can be set indicating that the same capability of the BPC entry is applicable to multiple scenarios.

#### -	modulationOrder

This field defines the supported modulation scheme by the UE.

#### -	subCarrierSpacing

This field defines the supported sub-carrier spacing by the UE.

#### -	supportedBasebandProcessingCombination

This field defines the supported CA baseband processing combinations by the UE. Baseband combinations associated to supportedBandCombination are only included. A baseband combination is applicable to the band combination which consist of bands with equal ca-BandwidthClassDL, ca-BandwidthClassUL, and equal or higher supportedMIMO-CapabilityDL and supportedMIMO-Capability.

#### -	supportedBasebandProcessingCombination-MRDC

This field defines the supported MN and SN baseband processing combinations for MR-DC by the UE.

Editor’s Note: How to map the supported baseband combinations to band combinations for MR-DC is FFS.

Editor’s Note: Whether it is defined as optional or mandatory with IOT indication will be further discussed. Other parameters will be added later based on RAN1/4 inputs.