3GPP TSG-RAN WG2 NR AH#3	Tdoc DRAFT R2-1801646

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.4.3.2 ASN.1 issue documents

Source:	Ericsson

Title:	DRAFT Update to Flag for Beam ID reporting (Issue S005)

Document for:	Discussion, Decision

# Introduction

In RAN2AH2018-1 we had an offline discussion about the need for the parameter onlyReportBeamIds and the possibility of defining some implicit rule instead. That was raised during the ASN.1 review, reflected by the following issue:

| S005 | Field onlyReportBeamIds is redundant (was commented serveral times before). Also related procedures should be removed |
| --- | --- |

The offline discussion has concluded that the parameter is needed. In addition, it has also proposed how to sort beam measurement information and derive identifiers.

# Discussion

In the current version 38.331, the following parameters in reportConfig are used to indicate the UE what exactly to report in terms of beam measurement information.

-- RS index reporting configuration

reportQuantityRsIndexes						MeasReportQuantity													OPTIONAL,

maxNrofIndexesToReport							INTEGER (1..maxNrofIndexesToReport) 								OPTIONAL,

onlyReportBeamIds							BOOLEAN																OPTIONAL,

where

MeasReportQuantity ::=						SEQUENCE {

rsrp										BOOLEAN,

rsrq										BOOLEAN,

sinr										BOOLEAN

}

In RAN2#Ad-Hoc 2018-1 the following has been agreed concerning beam sorting:

Agreements

1:  The UE uses RSRP if present, otherwise RSRQ, for beam sort in ReportConfigNR if multiple quantities are configured to report.

2:  The UE would use the report quantity for beam report to sort the beams in the MR if only one measurement quantity is configured to report.

The rule can be translated in by the following table:

| reportQuantityRsIndexes in the reportConfig includes following beam level reporting quantity. | Measurement quantity to be used for the sorting of beam level measurements and to derive beam identifiers |
| --- | --- |
| RSRP | RSRP |
| RSRQ | RSRQ |
| SINR | SINR |
| RSRP and SINR | RSRP |
| RSRP and RSRQ | RSRP |
| RSRQ and SINR | RSRQ |
| RSRP and RSRQ and SINR | RSRP |

Beam reporting can include measurement results or only the beam identifiers. However, yet an additional problem discussed was how to indicate the UE that only beam identifiers should be included i.e. that the UE shall not include the measurement results.

Current solution captured in the specifications uses a flag. Then, if the flag onlyReportBeamIds is set to TRUE the UE uses the information provided in reportQuantityRsIndexes. If a single flag is set, then it seems quite easy to assume that this is the flag used for deriving the beam IDs. An alternative for the case od multiple flags would be to use the same rule defined for the sorting quantity. In other words, sorting quantity would be used for defining the quantity to be used to derive the beam IDs (and sort them).

That seemed to be agreed offline among most companies although during online session, two other alternatives were highlighted.

For beam ID only reporting, we suggest RAN2 to discuss:

- Option 1/ Keep current text in specification (including onlyReportBeamIds)

  - As agreed offline.

- Option 2/ Always report measurements when configured (i.e. no need for only ID flag)

  - This could make the spec quite simple, if RAN2 is able to take that step.

- Option 3/ Define an additional rule so the UE can omit measurements without the need for a flag e.g. if all quantities are FALSE, omit measurements and use RSRP to derive IDs for periodical report type and use trigger quantity for event triggered report type.

  - Limitation is that network cannot select quantity to derive beam IDs. Somewhat limiting compared to option 2/.

If option 1 is agreed, a minor updated has been suggested to improve readability, considering the way the logic is written. Hence, we suggest discussing the following:

Rename onlyReportBeamIds to includeBeamMeasurements.

# Text Proposal

#### 5.5.5.2	Reporting of beam measurement information

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered:

2>	consider the trigger quantity as the sorting quantity;

1>	if reportType is set to periodical:

2>	 if a single reporting quantity is set to TRUE in reportQuantityRsIndexes;

3>	 consider the configured single quantity as the sorting quantity;

2> 	else:

3>	 if rsrp is set to TRUE;

4>	 consider RSRP as the sorting quantity;

3>	 else:

4>	 consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRsIndexesToReport beam indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting quantity and the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if onlyReportBeamIds is not configured, include the SS/PBCH based measurement results for the quantities in reportQuantityRsIndexes set to TRUE for each beam index;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RSIndexes the index associated to the best beam for that CSI-RS sorting quantity and the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if onlyReportBeamIds is not configured, include the CSI-RS based measurement results for the quantities in reportQuantityRsIndexes set to TRUE for each beam index;

Editor’s Note: FFS Measurement relation actions during procedures such as handover and re-establishment