3GPP TSG-RAN WG2 Meeting #95bis                                                         R2-166652

Kaohsiung, Taiwan, 10-14 October 2016

Agenda item:	9.2.2.4

Source:	Samsung

Title:	Initial access to support multiple numerologies in NR

Document for:	Discussion & Decision

# 1. Introduction

RAN#71 in March approved a 5G SID [1] on New Radio (NR) access technology, which targets a unified framework for traffic with diverse QoS requirements e.g. enhanced mobile broadband (eMBB), massive machine-type-communications (mMTC), ultra-reliable and low latency communications (URLLC).

In RAN1 [2] using different sets of numerology and frame structures for different service verticals is being considered. From RAN1#86/RAN2#95 meetings, the followings were agreed:

RAN1 Agreements

•       For subcarrier spacing of each synchronization signal (e.g. NR PSS,SSS) in a NR carrier, the following alternatives should be studied

•       Alt 1: Subcarrier spacing is predefined in the specification for a given frequency range

•       Ex: 15kHz for sub-6GHz, 60kHz for over-6GHz

•       Note that there are more than one frequency ranges

•       Alt 2: Subcarrier spacing is selected by NR BS

•       FFS: Details on the set of possible numerologies

•       Note: Blind detection of multiple numerologies can be considered

•       Alt 3: Single subcarrier spacing is predefined in the specification for all frequency ranges

•       Other alternatives are not precluded

RAN2 Agreements

Agreements

1: 	RAN2 to agree on proposals 1, 4, 5, 6 and 7 from [1] to be captured as guidelines for SI design in RAN2 TR with rephrasing of some proposals if needed.

2: Other mechanisms than periodic broadcast of system information should be studied during study item.

3: Agree on the terminology of Minimum SI (at least for purpose of the SI discussions).

4: Minimum SI needs to be broadcasted periodically.

5: Contents and format of minimum SI are FFS. Content will at least include information to support cell selection, for acquiring other SI, for accessing the cell.

FFS Whether all "cells"/TRPs periodically broadcast the minimum SI.

6: Agree on the terminology of Other SI where other SI comprises everything not broadcasted in minimum SI.

FFS Whether ETWS/CMAS like information would be considered as Other SI or Minimum SI

7: Both network triggered and UE initiated mechanisms for Other SI delivery shall be further studied.

8: It is network decision whether other SI is broadcasted or delivered through UE-specific signalling.

This contribution investigates design considerations for NR initial access to support multiple numerologies; especially about configuration of numerology during initial access procedures.

# 2. 	Design Considerations for NR Initial Access

  - LTE Initial Access Procedures

Initial access in LTE has a sequence comprising frequency acquisition, time & frequency synchronization, cell selection, MIB/SIB acquisition, and random access. A UE which is successful for random access can establish connection for setting up SRB (Signalling Radio Bearer).

LTE can be regarded as single numerology system except supporting normal/extended CP and RACH preamble format. CP/symbol length information is implicitly indicated by format of synchronization signal (PSS/SSS). After synchronization, there is no separation of procedure for DL signalling due to change of numerology. RACH preamble format including CP length and preamble length is separately indicated by SIB2. Meanwhile the subcarrier spacing for RACH preamble is given to UE and the subframe length is same as the numerology for DL. We therefore assume that LTE RACH preamble configuration is a part of the default numerology and is additionally informed only for UL transmission of RACH preamble.

  - Possible NR Initial Access Procedures

At RAN2#93bis meeting, we discussed on common initial access procedure [3] which has the commonality across DL synchronization, cell search, minimum (or essential) SI acquisition, and random access. The conclusion from the discussion was that common initial procedure is beneficial from the UE complexity and the forward compatibility perspectives. Since remaining SI information can be requested by UE, minimum SI is simple and common regardless of the condition of UE.

  - Issues to Support Multiple Numerologies

According to on-going discussion in RAN1, we could define that default numerology is same as the numerology for (successful) reception of synchronization signals (i.e. PSS/SSS). By the way, like LTE RACH, there is possibly a sub-numerology which could be derived from the default numerology. Therefore in this paper we will use the terminology - default numerology set which comprises single or multiple default numerologies which can be derived by the numerology for detecting synchronization signals without assuming additional configuration to the UE. On the other hand, we also define the dedicated (or alternative) numerology set as single or multiple numerologies for UE’s operation in resource region where UE is dedicated for the specific service. Procedures and issues to configure the dedicated numerology set are investigated here. The case of same numerology for the default and dedicated numerology set is valid when a numerology set is fixed. In other cases, the dedicated numerology set is indicated by network via channels in the default numerology.

Observation 1: If single or multiple numerologies are derived by the numerology for detecting synchronization signals, we can call those numerologies as a default numerology set. If alternatively other numerologies are indicated by network, we can call those additional numerologies as a dedicated numerology set.

Figure 1: Options for transition of numerology set

Issue 1: In which procedures does UE require to operate via the default/dedicated numerology?

The possible options when to indicate the dedicated numerology set is depicted in Figure 1. Option 1 shows that the dedicated numerology is indicated via synchronization signal. If the LTE-like PCI (physical cell ID) indication via synchronization signal concept is preserved also in NR, numerology indication needs more bits in addition to PCI indication. This could results in lower reception performance or higher UE complexity due to a burden for synchronization signal. Therefore we will not discuss further issues based on Option 1 in this paper.

According to RAN1/2 agreements [4][5] for NR, minimum (or essential) SI is being considered to provide information for at least initial access procedures. The minimum SI is naturally transmitted via default numerology set if synchronization signal does not indicate the dedicated numerology set. Furthermore, the minimum SI should be designed to support minimum capable UE in the specific operation carrier. In that sense, it could be suggested that at least minimum SI is delivered via the default numerology set from the UE complexity and forward compatibility perspectives.

Proposal 1: At least minimum SI is delivered via default numerology set.

Meanwhile Option 2 and Option 3 may be arguable depending on the necessity of random access procedure in the dedicated numerology set. According to NR requirements in TR 38.913 [2], required C-plane latency in NR is 10 ms regardless of numerology/services/verticals. It means that service-specific random access procedure is not required and common random access procedure is sufficient. However, service-specific C-plane latency may be beneficial if target C-plane latency is much different up to numerology/services. For instance, mMTC UE may be enough to have C-plane latency in a couple of seconds, while URLLC UE may need tighter latency in a couple of milliseconds. In addition, LTE already provides multiple RA preamble formats to compensate different deployment scenarios. Furthermore NB-IoT supports configuration of different subcarrier spacing for UEs in different condition. Even for U-plane latency, the latency for random access procedure may be a part of U-plane latency if short message transmission via random access procedure is considered. Therefore it could be meaningful to discuss Option 2 if the network requests different performance requirements for random access procedure per service type. On the other hand, different UE capability such as BW may impact to RACH opportunity as pointed from the companion contribution [6]. On this issue, minimum supported UE BW as well as potential dedicated numerology should be signalled to UE for flexible system operation. If no different consideration is found from the C-plane latency and UE capability aspects, Option 3 can be a baseline for discussion of initial access procedure.

Proposal 2: The necessity of dedicated numerology for random access procedure should be discussed if service-specific C-plane latency constraint is considered; otherwise, the default numerology set for random access procedure as a common is a baseline.

Based on the above discussion, we investigate a further issue as follows:

Issue 2: The necessity of additional SI and a type of numerology set for detecting the additional SI

The design for minimum SI targets minimum contents and common/essential information/functions for initial access. So as to deliver other remaining system information which are not transmitted in the minimum SI, additional SI should be supported. The additional SI could be transmitted either in cell-specific or UE-specific manner. But whatever transmission type is used, it could be nice, e.g. especially for lower class UE, to configure the dedicated numerology set without mandating default numerology for connected mode operation. Referring to the companion contribution [7], additional complexity and power consumption for connected state is much higher than idle. In this sense, default numerology for idle mode operation is sufficient for detection of additional SI.

Observation 2: Additional SI is beneficial to reduce the overhead of minimum SI as well as to relax UE complexity.

Proposal 3: RAN1/2 should strive for minimizing required UE capability to receive synchronization signal, minimum SI and additional SI, as well as to acquire the dedicated numerology set.

# 3. Summary

Based on the discussion and identified observations in this contribution, we propose followings:

Observation 1: If single or multiple numerologies are derived by the numerology for detecting synchronization signals, we can call those numerologies as a default numerology set. If alternatively other numerologies are indicated by network, we can call those additional numerologies as a dedicated numerology set.

Observation 2: Additional SI is beneficial to reduce the overhead of minimum SI as well as to relax UE complexity.

Proposal 1: At least minimum SI is delivered via default numerology set.

Proposal 2: The necessity of dedicated numerology for random access procedure should be discussed if service-specific C-plane latency constraint is considered; otherwise, the default numerology set for random access procedure as a common is a baseline.

Proposal 3: RAN1/2 should strive for minimizing required UE capability to receive synchronization signal, minimum SI and additional SI, as well as to acquire the dedicated numerology set.

References

[1] R2-160671, New SID Proposal: Study on NR New Radio Access Technology

[2] 3GPP TR 36.889 (2016-03)

[3] R2-162300, “Preliminary view on Initial Access in 5G”, Samsung

[4] ftp.3gpp.org Chairman's Notes RAN1_86 – final.doc

[5] ftp.3gpp.org RAN2#95-Gothenburg-Chairman-Notes-2016-08-26-eom.doc

[6] R1-1609116, “RACH procedure considering flexible UE bandwidth”, Samsung

[7] R2-166538, “Comparison between active and inactive state for small data transmission from energy efficiency perspective”, Samsung