3GPP TSG-RAN WG2 Meeting #95bis	R2-167019

Kaohsiung, Taiwan, October 10 – October 14, 2016

Agenda item:		8.5.3 (LTE_WLAN_aggr-Core)

Source:	LG Electronics Inc.

Title: 	Security update for WLAN

Document for:	Discussion and Decision

1.	Introduction

At the RAN2#95 meeting, the issue of Mobility enhancements of LWA was discussed, and RAN2 made following agreements:

Agreements

1	eNB does not need to change WLAN security key at handover but does need to be replaced at some later time.

In this contribution, we discuss about the need of triggering WLAN security refreshing after S-KWT update.

2.	Discussion

The WLAN security provides following protection:

- Authentication for access control

- Encryption for data protection

For the question about the feasibility of not using WLAN security, RAN2 already have replies from SA3 [1] as below:

It would not be acceptable from security point of view to not use any WLAN authentication for a UE utilizing LTE-WLAN aggregation. 802.11 encryption is not required, but using 802.11 authentication without encryption is currently not allowed as per 802.11 specifications.

The reason that WLAN security is used in LWA is the 802.11 authentication for UE. Because the data is already protected by PDCP ciphering, the 802.11 encryption is not required but used because of the limitation of 802.11 specifications.

Observation 1: LWA uses WLAN security to authenticate a UE to establish a WLAN connection.

Observation 2: LWA does not require 802.11 encryption.

In the case of “Handover without WT change” that is included in WID on LWA enhancements [2], the S-KWT will be updated as the change of KeNB with the handover. The updated S-KWT derives a new PMK, which triggers refreshing WLAN security as stated in 22A.1.8 in [3] of release 13.

As stated earlier, refreshing WLAN security provides both protections again. However, the authentication is not required in this stage because the WLAN already has checked that the UE has proper permission to connect to the WLAN. Besides, the encryption is not required as the observation 2.

In conclusion, refreshing WLAN security is not required during handover without WT change.

Proposal 1: Updated KeNB during the handover without WT change will update S-KWT and PMK. But the new PMK will not refresh the WLAN security with the UEs already authenticated.

The updated PMK does not affect the connection between WLAN and the UEs that are already authenticated with it, but will be used for the future connection request or connection refresh with the WLAN roaming. It is a normal operation and no specification change is required.

Proposal 2: The updated PMK will be used for the future connection request or connection refresh of the WLAN as normally.

In conclusion, WLAN security key does not need to be replaced while the WLAN connection is retained.

Proposal 3: Remove the requirement of WLAN security key refresh after handover from the agreement as below.

eNB does not need to change WLAN security key at handover.

3.	Conclusion

Observation 1: LWA uses WLAN security to authenticate a UE to establish a WLAN connection.

Observation 2: LWA does not require 802.11 encryption.

Proposal 1: Updated KeNB during the handover without WT change will update S-KWT and PMK. But the new PMK will not refresh the WLAN security with the UEs already authenticated.

Proposal 2: The updated PMK will be used for the future connection request or connection refresh of the WLAN as normally.

Proposal 3: Remove the requirement of WLAN security key refresh after handover from the agreement as below.

eNB does not need to change WLAN security key at handover.

Reference

[1] R2-154017, “reply LS on authentication and encryption between UE and WLAN for aggregation”

[2] RP-160600, “New Work Item on Enhanced LWA”, Intel Corporation, Qualcomm Incorporated, China Telecom

[3] 3GPP TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2 (Release 13)”