3GPP TSG-RAN WG2 Meeting #95bis	R2-167039

Kaohsiung, Taiwan, 10 - 14 October 2016

Agenda item:	9.2.2.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Distribution of System Information in NR

WID/SID:	FS_ NR_newRAT - Release 14

Document for:	Discussion and Decision

# 1	Introduction

The system information (SI) that is delivered by the network is essential for the UE to access and operate properly within the NR. Several contributions about the distribution of SI have been already submitted to RAN2#95, e.g., [1][2][3], touching upon the design principles and guidelines along with some implementation details. From the report of email discussion on SI [4], there seems to be a common understanding of the drawbacks of having only an LTE-like SI distribution for NR and the necessity for having new SI delivery approaches that are more efficient for the network and UE.

In this contribution, we summarize our general view on the SI distribution for NR. First, we provide an overall description of SI delivery for NR in which it is proposed to split the SI into two parts: Minimum SI and Other SI. We then discuss the distribution of each part in NR system.

# 2	Discussion

## 2.1 General Description of System Information Delivery for NR

The NR system is foreseen to operate at low carrier frequencies that are below 6 GHz and at high carrier frequencies of 28 GHz and above.  The higher the carrier frequency, the higher the number of antenna elements that can be packed in the transmit panel leading to a more narrow beams. As a result, higher number of beams are needed to cover the same area when broadcasting using beam sweeping [5], which increases in turn the downlink signalling overhead. For this reason, it is proposed for NR system not necessarily to broadcast periodically all SI, but rather to split the SI into Minimum and Other SI as shown in Fig. 1.

Figure 1: Split of SI into Minimum and Other SI

The Minimum SI is broadcasted periodically by each cell of the network. The remaining SI that is not part of the Minimum SI is the Other SI. The Minimum SI comprises at least the information that is essential for initial access but, if configured by the network, could also comprise information for acquiring part of or the entire Other SI.

Proposal 1: System information is split into Minimum SI, which is broadcasted periodically in all cells to provide essential information for all UEs that camp on the cell, and Other SI containing the remaining SI whose transmission is configured by the network.

As the capacity of the broadcast channel in NR system highly depends on the number of beams used for broadcast, the SI delivery procedure should be flexible enough to allow different configurations for the amount of SI to be periodically broadcasted. This provides full flexibility for network operators to control the broadcast signalling overhead without being bound to SI delivery solutions that are scenario- or carrier frequency- specific.

Proposal 2: The SI delivery procedure shall enable a high level of configurability in controlling the amount of SI to be broadcasted periodically.

To enable such configurability, the SI message(s) carrying the Minimum SI can schedule part of the Other SI to be broadcasted periodically. The scheduling of additional SI messages provides the network operators the flexibility to configure and control the amount of SI to be broadcasted periodically. For instance, for NR systems operating at high carrier frequency the network operator can decide to broadcast periodically only the Minimum SI, whereas it can schedule additional SI to be broadcasted periodically in NR systems operating at low carrier frequencies.

Proposal 3: The SI message(s) carrying the Minimum SI can schedule part of the Other SI to be broadcasted periodically.

Note that in one extreme case, the SI message(s) carrying the Minimum SI can schedule the Other SI to be all broadcasted periodically. This would result in the same SI delivery procedure that is used by LTE. As such, the proposed SI delivery procedure enables implicitly the configuration of periodic broadcast of all SI.

The remaining part of the Other SI that is not configured by Minimum SI for periodic broadcast is initiated on demand either by the UE or the network. For example, the UE can trigger the transmission of Other SI upon an access to the network, or the use of a service requiring specific SI; and the network can push unconditionally the Other SI to the UE upon e.g. performing RRC connection. Moreover, the Other SI is sent for the UE either using broadcast or dedicated transmission depending on some criteria. For instance, the network can decide to broadcast the Other SI in case of an update or receiving a request to send EWTS and CMAS warning messages. It has been shown in [6] that the on-demand delivery of Other SI can significantly outperform the periodic broadcast approach of all SI.

Proposal 4: The remaining part of the Other SI that is not configured by Minimum SI for periodic broadcast can be initiated on demand either by the UE or by the network.

## 2.2 Distribution of Minimum SI

As in LTE, the Minimum SI is distributed using a Master Information Block (MIB) having a different periodicity than the other SI message(s) containing the remaining part of Minimum SI that is not carried out by MIB. It is for FFS whether the remaining part of the Minimum SI is carried out by a single or multiple SI messages. The use of different periodicities can help to reduce the downlink signalling overhead resulting from broadcast transmission, which is especially relevant for NR operating at high carrier frequencies which uses beam sweeping for broadcast [5].

The Minimum SI is expected to contain at least the information that is relevant for initial access to avoid any impact on the current PLMN selection and cell-selection procedures. Considering the SI of LTE as a baseline [7], the Minimum SI would contain the relevant information of

- Master Information Block (MIB) containing a limited number of most frequently transmitted parameters required for initial access.

- System Information Block (SIB) 1 containing the parameters needed for PLMN selection, cell selection and access.

- SIB 2 containing the information about radio resource configuration.

- SIB 14 containing the extended access class barring parameters that are defined to avoid an overload from MTC devices.

For NR operating at high carrier frequency, the size of the SI message(s) carrying the remaining part of the Minimum SI that is not in MIB, might be a limiting factor with respect to the number and type of information elements that can be broadcasted. One way to increase the amount of broadcasted SI is to consider having in Minimum SI an index pointing to some default configuration that is saved in the UE and a procedure that allows the network to update some of these default parameters.

Proposal 5: The Minimum SI also contains an index pointing to some default configuration that is saved in the UE and which can be updated by the network.

## 2.3 Distribution of Other SI

There are two options on how to configure the Other SI in NR and they are summarized in Fig. 2.

- Other SI is split into common and cell-specific SI              b.  All Other SI is configured as cell-specific

Figure 2: The two configuration options of the Other SI

The first option shown in Fig. 2a is to split the Other SI into common SI that is valid for a group of cells, e.g., cells within a geographical area, and cell-specific SI that can be different among the cells of the group. This option is relevant if the number of parameters that are common is high resulting in downlink signalling overhead reduction.

Observation 1: Splitting the Other SI into common and cell-specific SI is relevant if the number of common parameters is high.

However, there are some configuration parameters in the SI that are optimized by the mobile operators to enhance the system and UE performance; and many of these optimizations are carried out in a cell-specific manner. In order not to lose this flexibility and feature, the common SI shall contain the parameters that are not subject to cell-specific optimization. These common parameters have to be analysed and determined first by RAN 2 before proceeding with this option.

Proposal 6: RAN 2 to analyse and determine if there are some parameters in Other SI that are not subject to any network optimization.

The second option of Fig 2b is to have all Other SI configured as cell-specific. This option provides full flexibility for the mobile operators to optimize the parameters in a cell-specific way, however, at the expense of a higher signalling overhead.

Observation 2: Configuring all Other SI as cell-specific provides mobile operators full flexibility for network optimization at the expense of an increase in signalling overhead.

For both configuration options shown in Fig. 2, there are two approaches on how the UE can acquire the cell-specific SI as shown in Fig. 3. The first approach is that the UE acquires the cell-specific SI for one cell whereas in the second approach the UE acquires the cell-specific SI for a group of cells.

Figure 3: Cell-specific SI is acquired either for one cell or for a group of cells

The advantages and disadvantages of each approach is summarized in Table 1.

Table 1: The advantages and disadvantages of acquiring the cell-specific SI for one cell or a group of cells.

|  | Approach 1: Cell-specific SI acquired for one cell | Approach 2: Cell-specific SI acquired for a group of cells |
| --- | --- | --- |
| Uplink signalling overhead resulting from sending the request for cell-specific SI | High as new UE request is needed when moving to a new cell | Low as the UE request is needed only when the UE leaves out the coverage of the cells in the group |
| Impact on UE power consumption | High due to more frequent UE request of SI when moving to new cell | Low as the UE request for SI is performed occasionally when leaving out the coverage of the cells in the group |
| Backhaul signalling overhead | No impact as coordination is not needed among group of cells | High if the group of cells have to share their SI in a distributed manner |
| Complexity for updating the SI | Low | High as coordination may be required among the cells of the group |

According to Table 1, the advantage of the second approach compared to the first approach is that the UE does not need to request the SI after each cell change, and consequently, the uplink signalling overhead and UE power consumption are lower. This is relevant if cell-reselection parameters are part of Other SI since the UE can acquire them once while moving in the coverage of the cells in the group. Moreover, the second approach can help keeping the power consumptions of RRC Idle and RRC Inactive Connected states [8] to their minimal levels. On the other hand, these benefits come along with some additional complexity for coordinating the transmission and the update of the SI among the cells in the group.

Observation 3: Acquiring Other SI for a group of cells can help to reduce the uplink signalling overhead and power consumption of the UEs.

# 3	Summary

In this contribution, we have provided an overview about how to distribute the system information in NR. The following has been observed and proposed:

Proposal 1: System information is split into Minimum SI, which is broadcasted periodically in all cells to provide essential information for all UEs that camp on the cell, and Other SI containing the remaining SI whose transmission is configured by the network.

Proposal 2: The SI delivery procedure shall enable a high level of configurability in controlling the amount of SI to be broadcasted periodically.

Proposal 3: The SI message(s) carrying the Minimum SI can schedule part of the Other SI to be broadcasted periodically.

Proposal 4: The remaining part of the Other SI that is not configured by Minimum SI for periodic broadcast can be initiated on demand either by the UE or by the network.

Proposal 5: The Minimum SI also contains an index pointing to some default configuration that is saved in the UE and which can be updated by the network.

Proposal 6: RAN 2 to analyse and determine if there are some parameters in Other SI that are not subject to any network optimization.

Observation 1: Splitting the Other SI into common and cell-specific SI is relevant if the number of common parameters is high.

Observation 2: Configuring all Other SI as cell-specific provides mobile operators full flexibility for network optimization at the expense of an increase in signalling overhead.

Observation 3: Acquiring Other SI for a group of cells can help to reduce the uplink signalling overhead and power consumption of the UEs.

# References

- R2-165549, Guidelines for system information distribution, Ericsson, Meeting #95, Gothenburg, Sweden, 22-26 August, 2016.

- R2-165026, Delivery of system information in NR, Huawei, HiSilicon, Meeting #95, Gothenburg, Sweden, 22-26 August, 2016.

- R2-165576, NR system information provisioning, Qualcomm Incorporated, Convida Wireless, Meeting #95, Gothenburg, Sweden, 22-26 August, 2016.

- R2-165201, Report of email discussion on [94#40][NR] System Information, Samsung (Email Rapporteur), TSG-RAN WG2 95, Gothenburg, Sweden, August 22-26, 2016

- R2-164792, On beam sweeping and its implications, Nokia, Alcatel-Lucent Shanghai Bell, Meeting #95, Gothenburg, Sweden, 22-26 August, 2016.

- R2-167041, Performance evaluation of system information delivery in NR, Meeting #95bis, Kaohsiung, Taiwan, 10-14 October 2016.

- TS 36.331, Radio Resource Control (RRC) Protocol Specification.

- R2-164791, Discussion of RRC states and its configurability for NR, Nokia and Alcatel-Lucent Shanghai Bell, Meeting #95, Gothenburg, Sweden, 22-26 August, 2016.