3GPP TSG-RAN WG3 Meeting #100	R3-182715

# Busan, South Korea, 21st- 25th May  2018

Agenda Item:	10.5.6.1

Source:	Ericsson

Title:	Data forwarding handling from 4G to 5G

Document for:	pCR

# Information

This pCR introduce in the data forwarding 4G to 5G, that the target NG-RAN node when it accepts the proposed E-RAB for data forwarding would provide tunnel end pointer per E-RAB. The UPF provides intermediate tunneling end points for the data forwarding paths on a per E-RAB basis. There is no need for UPF to do the E-RAB to QoS flow/PDU session mapping for the forwarding tunnels.

NGAP already support Data Forwarding Response per DRB ID in the HANDOVER REQUEST ACKNOLEDGE.

The impact is to add the option to do Data Forwarding Response per E-RAB ID. To reuse the current structure:

- Introduce a CHOICE to the current DRB Based data forwarding, so it is possible to do data forwarding either on DRB ID, or on E-RAB ID.

# Text Proposal for NGAP TS 38.413 v0.8.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE.

Editor’s Note:	Further details are FFS.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the target NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for all non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Handover Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in TS xx.xxx [xx]; [FFS pending SA3]

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context. [FFS pending SA3]

Upon receiving the PDU Session Resource Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure.

The target NG-RAN node shall then report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU sessions. In particular, for each PDU session resource successfully setup, it shall include the Handover Request Acknowledge Transfer IE containing the following information:

-	The list of QoS flows which have been successfully established in the QoS Flows Setup Response List IE.

-	The Data Forwarding Accepted IE if the data forwarding for the QoS flow is accepted.

-	The list of QoS flows which have failed to be established, if any, in the QoS Flows Failed to Setup List IE.

-	The transport layer address and tunnel endpoint to be used for the PDU session.

The list of PDU session resources which failed to be setup, if any, shall be reported in the HANDOVER REQUEST ACKNOWLEDGE message with an appropriate cause value within the PDU Session Resource Setup Failed List IE.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the Handover Request Acknowledge Transfer IE to each SMF associated with the concerned PDU session.

When the target NG-RAN node reports unsuccessful establishment of a QoS flow, the cause value should be precise enough to know the reason for an unsuccessful establishment [FFS].

If the HANDOVER REQUEST message contains the Data Forwarding Not Possible IE associated with a given PDU session within the Handover Request Transfer IE set to “data forwarding not possible”, the target NG-RAN node may not include the DL Forwarding Transport Layer Information IE and for intra-system handover the Data Forwarding Response DRB List IE within the Handover Request Acknowledge Transfer IE in the HANDOVER REQUEST ACKNOWLEDGE message for that PDU session.

In case of intra-system handover, if the target NG-RAN node accepts the downlink data forwarding for at least one QoS flow for which the DL Forwarding IE is set to “DL forwarding proposed”, it may include the DL Forwarding Transport Layer Information IE in the Handover Request Acknowledge Transfer IE for the PDU session within the PDU Session Resource Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

In case of intra-system handover, if the target NG-RAN node accepts the data forwarding for a successfully configured DRB, the target NG-RAN node may include the DL Forwarding Transport Layer Information IE for the DRB within the Data Forwarding Response DRB List IE within Handover Request Acknowledge Transfer IE in the HANDOVER REQUEST ACKNOWLEDGE message. [FFS: to be confirmed by SA2]

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding Transport Layer Information IE for a given DRB in the Data Forwarding Response DRB List IE within the Handover Request Acknowledge Transfer IE, it indicates the target NG-RAN node has requested the forwarding of uplink data for the DRB. [FFS: to be confirmed by SA2]

In case of inter-system handover from E-UTRAN, the target NG-RAN node includes the Data Forwarding Accepted IE for each QoS flow that the DL Forwarding IE is set to “DL forwarding proposed” for the corresponding E-RAB in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE and that the target NG-RAN node has admitted the proposed forwarding of downlink data for the QoS flow. If the target NG-RAN node supports data forwarding on E-RAB basis, it may include the DL transport layer information for data forwarding in the Handover Request Acknowledge Transfer IE for that PDU session within the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message for each accepted E-RAB for data forwarding. If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding Transport Layer Information IE for a given E-RAB in the Data Forwarding Response List IE within the Handover Request Acknowledge Transfer IE, it indicates the target NG-RAN node has requested the forwarding of uplink data for the E-RAB. [FFS: to be confirmed by SA2]

The target NG-RAN node shall use the information in the Handover Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the target NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that no roaming and no access restriction apply to the UE. The target NG-RAN node shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the setup PDU sessions includes a particular ARP value (TS 23.501 [9]). [FFS, need to align with PDU Session Management]

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11].

If the Location Reporting Request Type IE is included in the HANDOVER REQUEST message, the target NG-RAN node should perform the requested location reporting functionality for the UE as described in subclause 8.12.

If the UE Security Capabilities IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall handle it accordingly. [FFS pending SA3]

If the RRC Inactive Assistance Information IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

If the KAMF Change Indicator IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use this K_AFM_CI information as specified in TS 33.501 [13].

After all necessary resources for the admitted PDU Session Resources have been allocated, the target NG-RAN node shall generate the HANDOVER REQUEST ACKNOWLEDGE message.

Editor’s Note:	Further details are FFS.

#### 8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: Handover resource allocation: unsuccessful operation

If the target NG-RAN node does not admit at least one PDU Session Resource, or a failure occurs during the Handover Preparation, it shall send the HANDOVER FAILURE message to the AMF with an appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.4.2.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.3.4.11	Handover Request Acknowledge Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL NG-U Address | M |  | UP Transport Layer Information 9.3.2.2 | NG-RAN endpoint for transporting the QoS flows listed in the QoS Flows Setup Response List IE. |
| DL Forwarding Address | O |  | UP Transport Layer Information 9.3.2.2 | To deliver forwarded DL PDCP SDUs. |
| QoS Flows Setup Response List |  | 1 |  |  |
| >QoS Flows Setup Response Item IEs |  | 1..<maxnoofQoSFlows> |  |  |
| >>QoS Flow Indicator | M |  | 9.3.1.51 |  |
| >>Data Forwarding Accepted | O |  | 9.3.1.62 |  |
| QoS Flows Failed to Setup List | O |  | QoS Flow List 9.3.1.13 |  |
| Data Forwarding Response List |  | 0..1 FFS: to be confirmed by SA2 |  |  |
| >Data Forwarding Response Item IEs |  | 1..<maxnoofDRBsERABs> |  |  |
| >>CHOICE Data Forwarding Basis | M |  |  |  |
| >>>Intra System HO |  |  |  |  |
| >>>>DRB ID | M |  | 9.3.1.53 |  |
| >>>Inter System HO |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.3.2.3 |  |
| >>DL Forwarding Address | O |  | UP Transport Layer Information 9.3.2.2 |  |
| >>UL Forwarding Address | O |  | UP Transport Layer Information 9.3.2.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofDRBsERABs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>