3GPP TSG RAN WG3 Meeting #100	Tdoc R3-183217

Busan, Korea, 21st – 25th May 2018

Agenda Item:	31.3.1.3

Source:	Ericsson

Title:	(TP for EN-DC BL CR for TS 38.473): Priority order for SpCell list

Document for:	TP for EN-DC BL CR for TS 38.473

# Introduction

This is a resubmission of R3-182253. At the last meeting, there was a proposal to clarify that the gNB-DU sends the cells in the Potential SpCell List in the UE Context Setup Failure message in a priority order. This allows the gNB-DU to provide some load information to the gNB-CU. The proposal received some supports, but some companies believed that this is only an optimization (not needed).  In this contribution we re-iterate on the importance of introducing this clarification.

# Discussion

In the UE Context Setup Failure message, the gNB-DU sends the Potential SpCell List to the gNB-CU. We believe that it is be beneficial if the gNB-DU sends the cells in a priority order. How this priority is assigned by the gNB-DU may be left up to implementation e.g., it could be based on load. For example, the cells can be included from the least loaded to the most loaded which is an information that can be utilized by the gNB-CU when further selecting the SpCell.

The main benefit is that the gNB-CU can then infer which cells are more / less loaded and it can use this information for SpCell selection. It is worth remembering that there has been discussion about introducing a periodic load reporting function over F1. We believe that a periodic load reporting function adds complexity and may be difficult to use in multivendor environment. However, adding this simple clarification would enable the gNB-CU to infer enough information about load to perform an efficient SpCell selection without introducing the complexity of a periodic load reporting function.

Based on that we propose that the cells in the Potential SpCell list are sent in priority order.

Proposal 1	The cells in the Potential SpCell list are sent in priority order from gNB-DU to the gNB-CU. How the gNB-DU assigns the priority in the Potential SpCell list is up to implementation.

# Conclusion

In this contribution, we discussed issues related to the priority of the cells in UE context setup and made the following proposal.

Proposal 1	The cells in the Potential SpCell list are sent in priority order from the gNB-DU to the gNB-CU. How the gNB-DU assigns the priority in the Potential SpCell list is up to implementation.

Proposal 2	RAN3 is kindly asked to agree with TP in Annex I.

# Annex I: TP for EN-DC BL CR for TS 38.473

Start of Text Proposal 1 for TS 38.473

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 UE context, or cannot even establish one non-GBR bearer it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT SETUP REQUEST message, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value. Further, if the Candidate SpCell List IE is included in the UE CONTEXT SETUP REQUEST message and the gNB-DU is not able to accept the SpCell ID IE, the gNB-DU shall, if supported, include the Potential SpCell List IE in the UE CONTEXT SETUP FAILURE message and the gNB-CU should take this into account for selection of an opportune SpCell. The gNB-DU shall include the cells in the Potential SpCell List IE in a priority order, where the first cell in the list is the one most desired and the last one is the one least desired (e.g., based on load conditions). If the Potential SpCell List IE is present but no Potential SpCell Item IE is present, the gNB-CU should assume that none of the cells in the Candidate SpCell List IE are acceptable for the gNB-DU.

End of Text Proposal for TS 38.473