3GPP TSG RAN WG3 Meeting #100	Tdoc R3-183358

Busan, Korea, 21st – 25th May

Agenda Item:	31.3.1.3

Source:	Ericsson

Title:	(TP for BL SA CR for TS 38.473): UE Context Release

Document for:	TP for BL SA CR for TS 38.473

# Introduction

This is the TP for text proposal to capture the following agreement:

- Reuse the current UE context release procedure to signal UE rejection;

It is related to the following CB:

- CB: # 21_UE_rejection:

  - Reuse current UE context release procedure to signal UE rejection;

  - Clarify that the procedure may be used before UE context has been set up.

# Annex I: TP for TS 38.473

<<<<<<<<<<<<<<		FIRST CHANGE		>>>>>>>>>>>>>>>

### 8.3.3	UE Context Release (gNB-CU initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message.

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU.