3GPP TSG RAN WG3 #100	Tdoc R3-183506

Busan, Korea, 21st- 25th May 2018

Agenda Item:	10.10.7.3

Source:	Ericsson

Title:	(TP for NR BL CR for TS 38.401): Clarification on active cell list

Document for:	TP for NR BL CR for TS 38.401

# Annex 1: TP for the SA BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.401

## 8.5	F1 Startup and cells activation

This function allows to setup the F1 interface between a gNB-DU and a gNB-CU and it allows to activate the gNB-DU cells.

Figure 8.5-1: F1 startup and cell activation

0. The gNB-DU and its cells are configured by OAM in the F1 pre-operational state. The gNB-DU has TNL connectivity toward the gNB-CU.

1. The gNB-DU sends an F1 Setup Request message to the gNB-CU including a list of cells that are configured and ready to be activated.

2. In NG-RAN, the gNB-CU ensures the connectivity toward the core network. For this reason, the gNB-CU may initiate either the NG Setup or the gNB Configuration Update procedure towards 5GC.

3. The gNB-CU sends an F1 Setup Response message to the gNB-DU that optionally includes a list of cells to be activated. If the gNB-DU succeeds to activate the cell(s), then the cells become operational. If the gNB-DU fails to activate some cell(s), the gNB-DU initiates the gNB-DU Configuration Update procedure towards the gNB-CU. The gNB-DU includes in the gNB-DU Configuration Update message the cell(s) that are active (i.e., the cell(s) for which the gNB-DU should be able to serve UEs). The gNB-DU may also indicate that the cell(s) that failed to activate should be deleted, in which case the gNB-CU removes the corresponding cell(s) information.

4. The gNB-CU may send a gNB-CU Configuration Update message to the gNB-DU that optionally includes a list of cells to activated, e.g., in case that these cells were not activated using the F1 Setup Response message.

5. The gNB-DU replies with a gNB-DU Configuration Update Acknowledge message that optionally includes a list of cells that failed to be activated.

6. The gNB-CU may initiate either the Xn Setup towards a neighbour NG-RAN node or the EN-DC X2 Setup procedure towards a neighbour eNB.

NOTE:	In case that the F1 Setup Response is not used to activate any cell, step 2 may be performed after step 3.

Over the F1 interface between a gNB-CU and a gNB-DU pair, the following two cell states are possible:

- Inactive: the cell is known by both the gNB-DU and the gNB-CU. The cell shall not serve UEs;

- Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should be able to serve UEs.

The gNB-CU decides whether the cell state should be Inactive or Active. The gNB-CU can request the gNB-DU to change the cell state using the F1 Setup Response, the gNB-DU Configuration Update Acknowledge, or the gNB-CU Configuration Update messages. The gNB-DU can confirm (or reject) a request to change the cell state using the gNB-DU Configuration Update or the gNB-CU Configuration Update Acknowledge messages.

End of Text Proposal for TS 38.401