3GPP TSG-RAN WG3 Meeting #101	     R3-184929

Gothenburg, Sweden, 20th - 24th August 2018

Agenda Item:	24.1.2.1

Source:	Ericsson

Title:	TP to TR 38.874 on comparison of CP alternatives of architecture 1a

Document for:	pCR to TR 38.874

# Introduction

In RAN3#1807ah meeting, a new CP architecture option for architecture group 1a, namely option 5, was introduced. In addition, a comparison table that summarizes the differences between the 5 CP architecture options was introduced in TR 38.874.

In this contribution, we provide an update and content clarification of the comparison table.

# TP to TR 38.874

### 8.3.4 CP alternatives for architecture 1a

<<skipped until summary subsection>>

The comparison analysis of the five CP alternatives are provided in the Table 8.3.4-x. More comparison aspects are not excluded.

Table 8.3.4-1. Comparison of the five CP alternatives of architecture 1a

| Comparison aspects |  | Alt 1 | Alt 2 | Alt 3 | Alt 4 | Alt 5 | Comparison analysis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transport for CP signaling on wireless plane | UE/IAB-MT’s RRC | SRB in access link, SRB over RLC channel in backhaul links | Same as alt 1 | Same as alt 1 | Same as alt 1 | Same as alt 1 | SRB is recommended to carry UE/IAB-MT’s RRC signaling in all the alternatives. [TBD for IAB DU’s F1AP] |
|  | IAB-DU’s F1AP | SRB of collocated MT | Same as alt 1 | Same as alt 1 | DRB | [TBD] |  |
| Encapsulation on the wireless plane | UE/IAB-MT’s RRC | Within PDCP but without encapsulation in F1-AP of serving IAB node | Within PDCP and F1-AP of serving IAB node | Same as alt 1 | Same as alt 2 | Same with alt 1 | [TBD] |
|  | IAB-DU’s F1AP | Within RRC of collocated MT | Within PDCP of collocated MT | Same as Alt 2 | Within DTLS/SCTP/IP above RLC channel | Within PDCP of collocated MT | [TBD] |
| Encapsulation (over the wired donor CU-DU link) | UE/IAB-MT’s RRC | Native F1-C to donor DU | Within end to end F1-AP/PDCP and Native F1-C to donor DU | Same as Alt1 | Native end to end F1-C | Same as Alt1 | [TBD] |
|  | IAB-DU’s F1AP | Within end to end RRC/PDCP and Native F1-C to donor DU | Within end to end PDCP and Native F1-C to donor DU | Same as Alt2 | Native end to end IP | Within end to end PDCP and GTP-U to the donor | [TBD] |
| Security of F1AP |  | Protected by PDCP | Same as alt 1 | Same as alt 1 | Protected by DTLS | Protected by PDCP | [TBD] |
| Routing of control plane PDUs |  | Adaptation layer is responsible for routing | Same as alt 1 | Same as alt 1 | Same as alt 1 | Same as alt 1 | In all alternatives, the adaptation layer is used for routing. |
| Impact to IAB donor CU |  | Support of using donor F1-AP to directly transfer RRC messages to UE or IAB MT Support of using donor F1-AP to transfer end to end F1-AP to IAB DU. | Support of using donor F1-AP to transfer F1-AP messages to the IAB node | Same as alt 1 for RRC messages to UE or IAB MT Same as alt2 for F1-AP to IAB DU | Support for DTLS | Same as alt 1 for RRC messages to UE or IAB MT Support for transport of F1-AP messages over E1 | [TBD] The detailed impact on native F1-AP needs further study. |
| Impact to IAB donor DU |  | Support for mapping F1-AP messages to the proper adaptation layer address | Same as alt1 | Same as alt1 | Support for IP routing function (the same functionality as in UP) | Same as alt1 for RRC messages to UEs and IAB’s MT Support for mapping GTP-U tunnels to the proper adaptation layer address (for F1-AP to IAB DU) |  |