3GPP TSG-RAN WG3 Meeting #101	R3-185004

# Gothenburg, Sweden, 20th- 24th August  2018

Agenda Item:	31.3.4.3

Source:	Ericsson

Title: 	On signalling of dynamic 5QI values, XnAP

Document for:	Discussion and Approval

# 1	Information

Changes included in this text proposal:

- Update the Dynamic 5QI Descriptor and ASN.1.

- Update the Non dynamic 5QI Descriptor.

- Introduce in ASN.1 an new IE FiveQI

- Modify Non-Dynamic5QIDescriptor to refer to FiveQI.

# 2	Text Proposal on BL R3-184390 on TS 38.423

-------------------------------

Start the first change!

-------------------------------

#### 9.2.3.8	Non dynamic 5QI Descriptor

This IE defines QoS characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0.255) | This IE contains the standardized or pre-configured 5QI value as specified in TS 23.501 [7} |
| Priority level | O |  | 9.2.3.62 | Priority level is specified in TS 23.501 [7]. When included, it overrides standardized or pre-configured value. |
| Averaging window | O |  | 9.2.3.14 | This IE applies to GBR QoS Flows only. Averaging window is specified in TS 23.501 [7]. When included, it overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | 9.2.3.15 | Maximum Data Burst Volume is specified in TS 23.501 [7]. When included, it overrides standardized or pre-configured value. |

#### 9.2.3.9	Dynamic 5QI Descriptor

This IE defines the QoS characteristics for a non-standardized or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | O |  | INTEGER (0.255) | This IE contains the dynamic 5QI value, as specified in TS 23.501 [7} |
| Priority level | M |  | 9.2.3.62 | Priority level is specified in TS 23.501 [7]. |
| Packet Delay Budget | M |  | 9.2.3.12 | Packet Delay Budget is specified in TS 23.501 [7]. |
| Packet Error Rate | M |  | 9.2.3.13 | Packet Error Rate is specified in TS 23.501 [7]. |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (Delay critical, Non-delay critical, ...) | This IE indicates whether the GBR Qos flow is delay critical as specified in TS 23.501 [7]. |
| Averaging window | C-ifGBRflow |  | 9.2.3.14 | Averaging window is specified in TS 23.501 [7]. |
| Maximum Data Burst Volume | O |  | 9.2.3.15 | Maximum Data Burst Volume is specified in TS 23.501 [7]. |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

------------------------

Skip to next change

------------------------

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AreaOfInterest-Item,

id-dataforwardingInfoperPDUSession-Item,

id-DataForwardingResponseDRBItem,

id-DRBsSubjectToStatusTransfer-Item,

id-DRBToQoSFlowMapping-Item,

id-PDUSessionResourceAdmittedResponseTransferItem,

id-PDUSessionResourcesAdmitted-Item,

id-PDUSessionResourcesNotAdmitted-Item,

id-PDUSessionResourcesToBeSetup-Item,

------------------------

Skip to next change

------------------------

#### -- D

DataforwardingInfoperPDUSession ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF ProtocolIE-Single-Container { {DataForwardingInfoperPDUSessionItem-IEs} }

------------------------

Skip to next change

------------------------

Dynamic5QIDescriptor ::= SEQUENCE {

fiveQI						FiveQI													OPTIONAL,

priorityLevelQoS			PriorityLevelQoS,

packetDelayBudget			PacketDelayBudget,

packetErrorRate				PacketErrorRate,

delayCritical				ENUMERATED {delay-critical, non-delay-critical, ...}	OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

averagingWindow				AveragingWindow											OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {Dynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

...

}

Dynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

------------------------

Skip to next change

------------------------

#### -- F

FiveQI ::= INTEGER (0..255)

#### -- G

------------------------

Skip to next change

------------------------

#### -- N

NeighbourInformation-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-E-UTRA-Item

------------------------

Skip to next change

------------------------

NonDynamic5QIDescriptor ::= SEQUENCE {

fiveQI						FiveQI,

priorityLevelQoS			PriorityLevelQoS													OPTIONAL,

averagingWindow				AveragingWindow														OPTIONAL,

maximumDataBurstVolume		MaximumDataBurstVolume 												OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

...

}

------------------------

The End!

------------------------