3GPP TSG RAN WG3 Meeting #101bis	R3-186185

Chengdu/China, 8-12th October 2018

Title:	NTN architecture principles

Source: 	Thales

Type:	Discussion

Document for:	Agreement

Agenda Item:	20.2 Architecture

Study item:	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Abstract

This technical document includes proposals on architecture aspects related to Non-Terrestrial network (study item in RP-181598 Study on solutions for NR to support non-terrestrial network, Thales).

These proposals have been discussed and agreed during the NTN off line discussion that took place on Wednesday 10th October 2018 in Chengdu.

# Non-Terrestrial Networks reference scenarios

Recall of scenario in TR 38.821

Table 4.2-1: Reference scenarios

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network | Scenario C | Scenario D |

Proposal 2.1: The following 6 scenarios are considered in the study item

- Scenario A: Transparent GEO (NTN beam foot print fixed on earth)

- Scenario B: Regenerative GEO (NTN beam foot print fixed on earth)

- Scenario C1: Transparent LEO (NTN beam foot print fixed on earth)

- Scenario C2: Transparent LEO (NTN beam foot print moving on earth)

- Scenario D1: Regenerative LEO (NTN beam foot print fixed on earth)

- Scenario D2: Regenerative LEO (NTN beam foot print moving on earth)

Thales: RAN2 assumed that C1 has to be added

ZTE: What about beam shape that may varied

Thales: the service provider will define a service area fixed smaller than the radio coverage of beams

Proposal 2.2: The study may consider only the most critical scenarios for a given RAN features or constraints (e.g. for Delay, it is sufficient to address the worst case scenario which is GEO Transparent)

ZTE: From RAN2 perspective, this proposal could make sense. From RAN3 perspective, better to address LEO in priority

Proposal 2.3: For LEO it is recommended to consider 600 km as baseline given that it features the worst case Doppler. However assessment for other altitude (e.g. 1200 km) can also be considered

# NG-RAN architecture with satellite

It is suggested to reduce the number of architecture options to be considered in the study.

Proposal 3.0.1: Define reference architectures to be considered in priority

- NG-RAN with transparent satellite (whatever GEO or LEO)

- NG-RAN with regenerative satellite (whatever GEO or LEO)

Proposal 3.0.2: Select architectures that minimize the need to define new interfaces nor new protocols in the NG-RAN

Proposal 3.0.3: Identify optional architectures to be considered in the study but with possibly a lower priority

## Transparent satellite based NG-RAN architecture

Figure 3.1-1: NG-RAN with transparent satellite

Proposal 3.1.1: Satellite provides RF frequency conversion & amplification but no processing of signal.

Proposal 3.1.2: Earth station (NTN gateway) is a transport node.

- A GEO can be served by one or several earth stations simultaneously. Each earth station handling a different set of beams.

- A LEO can be served by up to two earth stations at a time

Proposal 3.1.3: cellular/satellite dual connectivity is considered (UE supporting simultaneously cellular and satellite access links or simultaneously to 2 different satellite access links) in NG-RAN with transparent satellite. May be with lower priority

## Regenerative satellite based NG-RAN architecture with gNB on board

Figure 3.2-1: NG-RAN with regenerative satellite (gNB on board)

Proposal 3.2.1: Regenerative Satellite provides ISL.

Proposal 3.2.2: Regenerative Satellite embarks gNB and some traffic routing functions

Note that the study on the support of these traffic routing functions in the satellite should be considered in SA2 and not in RAN3

Proposal 3.2.3: Xn is transported over ISL

Proposal 3.2.4: SRI (Satellite Radio Interface) can be 3GPP defined (e.g. NR) or non 3GPP defined

Proposal 3.2.5: ISL (Inter Satellite Link) can be 3GPP defined (e.g. NR) or non 3GPP defined

Proposal 3.2.6: Earth station (NTN gateway) is a transport node.

- A GEO can be served by one or several earth stations simultaneously. Each earth station handling a different set of beams.

- A LEO can be served by up to two earth stations at a time

Proposal 3.2.7: cellular/satellite dual connectivity is considered (UE supporting simultaneously cellular and satellite access links) in NG-RAN with regenerative satellite. May be with lower priority

## Regenerative satellite based NG-RAN architecture with gNB-DU on board

Proposal 3.3.1: To consider gNB-DU on board satellite

## Optional NG-RAN architecture with satellite based on IAB

Proposal 3.4.1: To consider the outcomes of IAB SI (TR 38.874) in terms of architecture options

Proposal 3.4.2: R16 may focus on single hop

- It should not exclude to address multi hop, because IAB supports multi hop