3GPP TSG-RAN WG3 Meeting #102	R3-186800

# Spokane, U.S.A, 12th- 16th November  2018

Agenda Item:	9.2

Source:	Ericsson

Title: 	Pending Data Indication

Document for:	Discussion and Approval

# 1	Discussion

It is specified in Ts 23.401, that:

If the MME is aware that some signalling or data is pending in the network for an UE that is known as being unreachable for a long duration, e.g. for UE's having extended idle mode DRX or PSM enabled, the MME may include a Pending Data indication in the next S1-AP message towards an eNB. If the eNB receives this indication, the eNB may take this information into account when determining user inactivity. At inter-RAN node handovers, if some signalling or data are still pending, the target MME may send a Pending Data indication to the target RAN node.

At RAN3#101, it was agreed that the “Pending Data Indication” is introduced to the below procedures from Rel 13.

- INITIAL UE CONTEXT SETUP REQUEST message,

- UE CONTEXT RESUME RESPONSE message,

- HANDOVER REQUEST message,

- PATH SWITCH REQUEST ACKNOWLEDGE message,

- DOWNLINK NAS TRANSPORT message.

In our opinion, the Pending Data Indication is also needed in the “UE Information Transfer” message, which could be “the next S1-AP message towards an eNB” as specified in TS 23.401.

Proposal 1:   RAN3 to agree to include the Pending Data Indication in the UE Information Transfer message.

# 2	Proposal

Proposal 1:   RAN3 to agree to include the Pending Data Indication in the UE Information Transfer message.