3GPP TSG RAN WG3 Meeting #102 	R3-187112

Spokane/USA, 12-16 November 2018															revised R3-186354

Title:	NTN architecture with regenerative satellite

Source: 	Thales, ZTE

Type:	pCR

Document for:	Agreement

Agenda Item:	20.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document aims at correcting clause 5.2 Regenerative satellite based NG-RAN architectures  and clause 3.3 Abbreviations.

# Discussion

During the RAN3 meeting in Chengdu, it was identified that the “Figure 5.3-3: Regenerative satellite based NG-RAN architecture (gNB on board) with QoS flows” Should be revised to take into account the figure 12.1 in 38.300 (the QoS flows should be inside the PDU session, between a UE and the UPF).

Proposal 1: To revise Figure 5.3-3 taking into account the figure 12.1 of 38.300

The Figure 5.3-2: Regenerative satellite with ISL, gNB processed payload needs to clarify that some UE will be served by a gNB on board a satellite but accessing a 5GCN via another satellite.

Proposal 2: To revise Figure 5.3-2 highlighting the traffic and signalling routing in the case of regenerative satellites with ISL

Last, it should be studied the impact of extended delay on the NG protocols . Timers may have to be reviewed.

Proposal 3: To identify that the extended delay may impact NG protocols

# Conclusion

Proposal 1: To revise Figure 5.3-3 taking into account the figure 12.1 of 38.300 (see TP below)

Proposal 2: To revise Figure 5.3-2 highlighting the traffic and signalling routing in the case of regenerative satellites with ISL (see TP below)

Proposal 3: To identify that the extended delay may impact NG protocols (see TP below)

# Text Proposal for TR 38.821 v0.2.0

START OF CHANGES

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EIRP 	Equivalent Isotropic Radiated Power

FSS	Fixed Satellite Services

GEO	Geostationary Earth Orbiting

gNB	next Generation Node B

GW	Gateway

HAPS	High Altitude Platform Station

HEO	Highly Elliptical Orbiting

ISL	Inter-Satellite Links

LEO	Low Earth Orbiting

Mbps	Mega bit per second

MEO	Medium Earth Orbiting

MS	Mobile Services

MSS	Mobile Satellite Services

NAS-MM	Non-Access Stratum Mobility Management

NAS-SM	Non-Access Stratum Session Management

NG	Next Generation

NGAP	NG Application Protocol

NGEO	Non-Geostationary Earth Orbiting

NTN 	Non-Terrestrial Network

RAN	Radio Access Network

RTD	Round Trip Delay

SNR 	Signal-to-Noise Ratio

SRI	Satellite Radio Interface

Rx		Receiver

UAS	Unmanned Aircraft System

UE:	User Equipment

END OF CHANGES

START OF CHANGES

## 5.2	Regenerative satellite based NG-RAN architectures (FFS)

### 5.2.1	gNB processed payload

#### 5.2.1.1	Overview

The satellite payload implements regeneration of the signals received from Earth.

- NR-Uu radio interface on the service link between the UE and the satellite

- Satellite Radio Interface (SRI) on the feeder link between the NTN gateway and the satellite. The SRI transports the NG protocol

SRI (Satellite Radio Interface) is a transport link; it transports 3GPP-specified logical interfaces i.e., transmits the NG interface signalling packets.

Figure 5.3-1: NG-RAN with a regenerative satellite based on gNB

Note: The satellite may embark additional traffic routing functions, that are out of RAN scope. (FFS)

The satellite payload also provides Inter-Satellite Links (ISL) between satellites

- To support the Xn protocol and enable coordination between gNBs on board adjacent satellites, and especially to mitigate services disruption during UE mobility, from a source gNB to a target gNB. (FFS)

- To forward data packets, in case traffic functions are hosted on board the satellites. (FFS)

- To transport the NG protocol for a gNB on board a satellite which is not connected to a NTN Gateway

ISL are transport links.

ISL may be a radio interface or an optical interface that may be 3GPP or non 3GPP defined but this is out of the study item scope.

#### 5.2.1.2	Detailed description of the architecture (FFS)

The architecture of a regenerative-satellite based NG-RAN is depicted on the following figure. The mapping to QoS flows is also highlighted.

Figure 5.3-3: Regenerative satellite based NG-RAN architecture (gNB on board) with QoS flows (FFS)

The UE user plane protocol stack for a PDU session is described hereafter.

Figure 5.3-4: NG-RAN protocol architecture for regenerative satellite (gNB on board): User Plane

The Protocol stack of the Satellite Radio Interface (SRI) is used to transport the UE user plane between satellite and NTN-Gateway.

The User PDUs are transported over GTP-U tunnels, as usual, between the 5GC and the on-board gNB, but via the NTN Gateway.

The UE control plane protocol stack for a PDU session is described hereafter.

Figure 5.3-5: NG-RAN protocol architecture for regenerative satellite (gNB on board): Control Plane

The NG-AP is transported over SCTP, between the 5GC and the on board gNB, as usual, but via the NTN Gateway.

The NAS protocol is also transported by the NG-AP protocol, between the 5GC and the on board gNB, via the NTN Gateway.

#### 5.2.1.3	NG-RAN impacts

NG protocol timers may have to be extended to cope with the extended delay.

In the context of a LEO scenario with ISL, the delay to be mitigated shall encompass at least the feeder link and one or several ISL hops (FFS)

### 5.3.2	gNB-DU processed payload

#### 5.3.2.1	Overview

The satellite payload implements regeneration of the signals received from Earth.

- NR-Uu radio interface on the service link between the satellite and the UE

- Satellite Radio Interface (SRI) on the feeder link between the NTN gateway and the satellite. The SRI transports the F1 protocol.

The satellite payload also provides inter-satellite links between satellites.

SRI (Satellite Radio Interface) are transport links; the logical interface F1 that they transport are 3GPP-specified.

DU on board different satellites may be connected to the same CU on ground.

If the satellite hosts more than one DU, the same SRI will transport all the corresponding F1 interface instances.

Figure 5.3-6: NG-RAN with a regenerative satellite based on gNB-DU

#### 5.3.2.2	Detailed description of the architecture

FFS

#### 5.3.2.3	NG-RAN impacts (FFS)

RRC is terminated in the CU, and is subject to extremely strict timing constraints.

This may preclude the applicability of this architecture to GEO (Geostationary Earth Orbit) satellites.

Its use for LEO (Low Earth Orbit) systems may impact current F1 design.

### 5.3.3	gNB processed payload based on relay-like architectures (Optional) (FFS)

#### 5.3.3.1	Overview (FFS)

How to apply the Integrated Access and Backhaul (IAB) proposed architecture configuration resulting from the IAB SI reflected in the 3GPP TR 38.874 document [4] is for further study.

#### 5.3.3.2	Detailed description of the architecture

FFS

#### 5.3.3.3	NG-RAN impacts

FFS

END OF CHANGES