3GPP TSG-RAN WG3#102	R3-187224

Spokane, USA, 12- 16 November 2018

Agenda item:	10.5.6.1

Source:	Samsung, KT Corp., ZTE, CMCC, Verizon, LG Electronics Inc., CATT, ETRI, Nokia, Nokia Shanghai Bell, Huawei, KDDI, Google Inc., AT&T, China Unicom

Title:	(TP for BL CR for TS 38.300): Handling of end markers in 5g to 4g handovers

Document for:	Discussion and Decision

Text Proposal for TS 38.300

9.3	Inter RAT

[Unchanged text skipped]

9.3.2	From 5GC to EPC

[Unchanged text skipped]

9.3.2.4	Data Forwarding control plane

The control plane support for inter-System data forwarding follows the following key principles:

-	Only indirect data forwarding is supported.

-	PDU session information at the serving NG-RAN node contains mapping information per QoS Flow to a corresponding E-RAB.

-	At handover preparation, the source NG-RAN node shall decide which mapped E-RABs are proposed to be subject to data forwarding and provide this information in the source-to-target container to the target eNB.

-	The target eNB assigns forwarding TEID/TNL address(es) for the E-RAB(s) for which it accepts data forwarding.

-	A single data forwarding tunnel is established between the source NG-RAN node and UPF per PDU session for which at least data for a single QoS Flow is subject to data forwarding.

9.3.2.5	Data Forwarding user plane

The inter-System data forwarding user plane from 5GS to EPS follows the following key principles:

-	For the QoS flows accepted for data forwarding, the NG-RAN node initiates data forwarding to the UPF by the corresponding PDU session data forwarding tunnel(s).

-	The UPF maps forwarded data received from the per PDU session data forwarding tunnel(s) to the mapped EPS bearer(s) removing the QFI.

Handling of end marker packets:

The source NG-RAN node receives one or several end marker packets per PDU session from the UPF. When there are no more data packets to be forwarded for QoS flows mapped to an E-RAB, the source NG-RAN node sends one or several end markers including one QFI of those QoS flows mapped to that E-RAB and sends the end marker packets to the UPF over the PDU session tunnel. From the included QFI in the end markers and its mapping to an EPS bearer ID, the UPF knows which EPS bearer tunnel it needs to forward the end-markers to the SGW. The QFI is removed in the end marker packets sent to the SGW.

References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-184407, Handling of inter-system end markers

- R3-185159, Summary of offline discussions on iRAT forwarding solution

- R3-185361, TP for option 3 in inter-system handover

- R3-185362, TP for option 3 in inter-system handover