3GPP TSG-RAN WG3 #103                                              	　　　　　　　　    　　　　　　R3-190950 (Revision of R3-190488)

Athens, Greece, 25th of February – 1st of March 2019

Source: 	KDDI Corporation

Title:	Issues to be decided in IAB Stage2 design

Tdoc Type:	Discussion

Agenda Item:	13.1 - General

Document for:	Discussion and decision

Release:	Rel-16

Introduction

In this contribution we discuss important aspects for IAB WI, “Address assignment”, “Routing”, “Adaptation Layer”, “Redundancy”. We try to identify the issues, discussion points to be decided through stage2 work.

Discussion

  - Address assignment

In the study item, it was agreed that each IAB node holds IP addresses, however there is no discussion regarding how to assign a new IP address to the IAB node newly connected to the IAB donor. So, we propose to have the following two discussions.

Proposal1: 	RAN3 discuss IP address assignment scheme, whether reusing the existing protocol DHCP or developing a new dedicated mechanism for IAB system.

  - Routing

In the study item, we had some discussion on how to construct topology tables, however we have not concluded how to manage routing for detailed design. So, first we have to discuss which type should be supported, centralized or distributed. And we have to discuss which identities should be included in the routing information table. Moreover, if we adapt distributed routing and IP based routing then we can evaluate whether we can reuse the existing protocol like RIP(RFC1058). We propose to have the following discussions.

Proposal2:	RAN3 discuss which type of routing scheme to be supported, centralized or distributed

      - Option1:	Centralized: IAB donor decides all routing paths, all IAB nodes’ routing tables

      - Option2:	Distributed: Each IAB node constructs/maintains their own routing table

Proposal5: 	If RAN3 introduce distributed routing, then RAN3discuss whether adopt the existing protocol like Routing Information Protocol (RIP) or developing a new scheme for IAB system.

Proposal6: 	If RAN3 introduce centralized routing, then RAN3 develop a new protocol, interfaces and procedures which are necessary to construct the routing tables, which is used for deciding the path for each IAB PDU.

  - Adaptation Layer

Void

  - Redundancy

For redundancy, in the study item, it was confirmed that SCTP is a reasonable solution to achieve redundancy and there are some related sentences in TR38.874 “9.7.9 Adding redundant routes in architecture 1a”. So, we propose the following.

Proposal9: 	RAN3 discuss the redundancy scheme whether reusing SCTP (Stream Control Transmission Protocol, multi homing) for IAB system or developing a new scheme for the IAB system.

Proposal10: If RAN3 agree to adopt SCTP, then RAN3 agree that another IP address will be assigned to a new redundancy path, which is used for multi homing feature in SCTP.

Conclusion

In summary, we propose to have the following discussions to progress the IAB work

Proposal1: 	RAN3discuss IP address assignment scheme, whether reusing the existing protocol DHCP or developing a new dedicated mechanism for IAB system.

Proposal2:	RAN3 discuss which type of routing scheme to be supported, centralized or distributed

      - Option1:	Centralized: IAB donor decides all routing paths, all IAB nodes’ routing tables

      - Option2:	Distributed: Each IAB node constructs/maintains their own routing table

Proposal5: 	If RAN3 introduce distributed routing, then RAN3 discuss whether adopt the existing protocol like Routing Information Protocol (RIP) or developing a new scheme for IAB system.

Proposal6: 	If RAN3 introduce centralized routing, then RAN3 develop a new protocol, interfaces and procedures which are necessary to construct the routing tables, which is used for deciding the path for each IAB PDU.

Proposal9: 	RAN3 discuss the redundancy scheme whether reusing SCTP (Stream Control Transmission Protocol, multi homing) for IAB system or developing a new scheme for the IAB system.

Proposal10: If RAN3 agree to adopt SCTP, then RAN3 agree that another IP address will be assigned to a new redundancy path, which is used for multi homing feature in SCTP.

References

[1] TR 38.874 V16.0.0 TR Study on Integrated Access and Backhaul (Release 16)