3GPP TSG RAN WG3#103bis 	                                                                           R3-191276

# Xian, China, 8th  April – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.821 | CR | pCR | rev |  | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for TR38.821 Corrections of NTN Applicable Mobility Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2019-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current baseline TR38.821, some basic NTN mobility scenarios and their applicable procedures are captured e.g. in table 8.2-1. However, we found out that the yellow hightlighted part are worth further debating. For transparent satellite, even with intra-satellite handover, the same satellite can also connect to different gNBs on ground, via different RF feeding link, so that inter-gNB HO procedure shall be triggered. With inter-satellite handover, different satellites can also connect to the same gNB on ground, so that intra-gNB HO procedure shall be triggered. For gNB on board satellite, with inter access handover, it is also possbile that gNB on board and gNB on ground connect to the same 5GC, subject to operators’ deployment, hence intra-5GC HO procedure shall be triggered. For gNB-DU on board satellite, even with intra-satellite handover, the same satellite can also connect to different gNB-CUs on ground, via different F1 feeding link, so that inter-gNB-CU HO procedure shall be triggered. With inter-satellite handover, different satellites can also connect to the different gNB-CUs on ground, so that inter-gNB-CU HO procedure shall be triggered. With inter access handover, it is also possbile that NTN gNB-CU on ground and gNB on ground connect to the same 5GC, hence intra-5GC HO procedure shall be triggered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To correct the NTN applicable mobility procedures in different scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion or incompleteness exists |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change, ommited text not changed ***

8.2. Connected mode mobility (FFS)

There are different types of hand-overs in Non-Terrestrial networks:

- Intra-satellite hand-over (between cells served by same satellite)

- Inter-satellite hand-over (between cells served by different satellite)

- Inter-access hand-over (between cellular and satellite access)

There can be some variants depending on whether the satellite is transparent or regenerative (gNB or partial gNB on board the satellite)

The table identifies the applicable NG-RAN hand-over procedures for each of the NTN hand-over scenarios.

Table 8.2-1: NG-RAN procedures versus NTN hand-over scenarios

Editors note: Table may be merged into the table below in 8.x.7, FFS

| NTN Hand-over scenarios | Transparent satellite | Regenerative satellite (gNB on board) | Regenerative satellite (gNB-DU on board) |
| --- | --- | --- | --- |
| Intra satellite hand-over | Intra-gNB handover procedure or Inter-gNB handover procedure | intra gNB hand-over procedure | Intra-gNB-CU Mobility/Intra-gNB-DU handover or Inter-gNB-CU handover (See §8.2.1.2 in TS 38.401) |
| Inter satellite hand-over | Inter-gNB handover procedure or Intra-gNB handover procedure (See §9.2.3 in TR 38.300) | inter gNB hand-over procedure (See §9.2.3 in TR 38.300) | Intra-gNB-CU Mobility/ Inter-gNB-DU Mobility or or Inter-gNB-CU handover (See §8.2.1.1 in TS 38.401) |
| Inter access hand-over |  | Inter AMF/UPF hand-over procedure or Intra AMF/UPF hand-over procedure (out of RAN scope) | Inter AMF/UPF hand-over procedure or Intra AMF/UPF hand-over procedure (out of RAN scope) |

In each case, the relevant mobility procedures may require some adaptations to accommodate  the extended latency of satellite access.

An inter-access hand-over (between cellular and satellite access) is considered by utilizing an inter gNB procedure via the 5GCN (e.g. for Satellite with on board processed payload) or via the Xn (e.g. for satellite with transparent payload).

It is assumed that not all UEs are capable of positioning.

*** End change ***