3GPP TSG RAN WG3 Meeting #103bis	R3-191380

Xi’an, P.R. China, April 8th – 12th 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0081 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NGAP Signalling for Remote Interference Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI_RIM |  |  |  |  |  |  | Date: |  | 2019-03-29 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enabling Remote Interference Management backhaul signaling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the Uplink Remote Interference Management Information Transfer and Downlink Remote Interference Management Information Transfer Class-2 procedures. Introduced the RIM Set ID IE This is a backwards-compatible CR. The corresponding discussion is presented in R3-191379. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remote Interference Management not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new), 9.2.x (new), 9.3.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

## 8.1	List of NGAP Elementary Procedures

>>>>>>>>>>>>>>Skipped text<<<<<<<<<<<<<<

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink Remote Interference Management Information Transfer | UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER |
| Downlink Remote Interference Management Information Transfer | DOWNLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER |

-------------------------------------------Change 2-------------------------------------------

## 8.x	Remote Interference Management Information Transfer Procedures

### 8.x.1	Uplink Remote Interference Management Information Transfer

#### 8.x.1.1	General

The purpose of the Uplink Remote Interference Management Information Transfer procedure is to transfer Remote Information Management information from the NG-RAN node to the AMF. This procedure uses non-UE associated signalling.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: Uplink remote interference management information transfer

The NG-RAN node initiates the procedure by sending the UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER message to the AMF.

Upon reception of the UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER message, the AMF shall merge the content of this message with the content of other collected UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER with the same Source RIM Set ID IE and Target RIM Set ID IE values, and forward the merged message to the AMF corresponding to the Target RIM Set ID IE value.

#### 8.x.1.3	Abnormal Conditions

Void.

### 8.x.2	Downlink Remote Interference Management Information Transfer

#### 8.x.2.1	General

The purpose of the Downlink Remote Interference Management Information Transfer procedure is to transfer Remote Information Management information from the AMF to the NG-RAN node. This procedure uses non-UE associated signalling.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Downlink remote interference management information transfer

The procedure is initiated with a DOWNLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER message sent from the AMF to the NG-RAN node.

#### 8.x.2.3	Abnormal Conditions

Void.

-------------------------------------------Change 3-------------------------------------------

### 9.2.x	Radio-Related Information Messages

#### 9.2.x.1	UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION MESSAGE

This message is sent by the NG-RAN node to the AMF to transfer the Remote Interference Management Information.

Direction: NG-RAN  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Source RIM Set ID | M |  | 9.3.1.x | ID of a RIM set. | YES | reject |
| Target RIM Set ID | M |  | 9.3.1.x | ID of a RIM set. | YES | reject |
| List of Cells in Set |  | 0..1 |  |  |  |  |
| > List of Cells in Set Item |  | 1..< maxnoofCellsInSet s> |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  |  |  |
| RIM message | M |  | ENUMERATED(RS detected, RS disappeared …) | The informational contain of Remote Interference Management backhaul message. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsInSet | Maximum number of cells in RIM set. Value is 10000. |

-------------------------------------------Change 4-------------------------------------------

#### 9.2.x.2	DOWNLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION MESSAGE

This message is sent by the AMF to the NG-RAN node to transfer the Remote Interference Management Information.

Direction: AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Source RIM Set ID | M |  | 9.3.1.x | ID of a RIM set. | YES | reject |
| Target RIM Set ID | M |  | 9.3.1.x | ID of a RIM set. | YES | reject |
| List of Cells in Set |  | 0..1 |  |  |  |  |
| > List of Cells in Set Item |  | 1..< maxnoofCellsInSet s> |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  |  |  |
| RIM message | M |  | ENUMERATED(RS detected, RS disappeared …) | The informational contain of Remote Interference Management backhaul message. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsInSet | Maximum number of cells in RIM set. Value is 10000. |

-------------------------------------------Change 5-------------------------------------------

#### 9.3.1.x	RIM Set ID

This IE defines the Remote Interference Management Set ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RIM Set ID | M |  | BIT STRING (SIZE(22)) | The Remote Interference Management Set ID. |

-------------------------------------------End of changes ------------------------------------------