3GPP TSG-RAN WG3 Meeting #104	R3-192752

Reno, NV, USA, May 13th – 17th 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0081 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NGAP Signalling for Remote Interference Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI_RIM |  |  |  |  |  |  | Date: |  | 2019-05-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enabling NGAP signaling for Remote Interference Management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the Uplink Remote Interference Management Information Transfer and Downlink Remote Interference Management Information Transfer Class-2 procedures. Introduced the RIM Set ID IE. This is a backwards-compatible CR. The corresponding discussion is presented in R3-192751. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remote Interference Management signaling not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.x (new), 8.x.1 (new), 8.x.1.1 (new), 8.x.1.2 (new), 8.x .1.3 (new), 9.2.x (new), 9.2.x.1 (new), 9.2.x.1.1 (new), 9.2.x.1.2 (new), 9.2.x.1.3 (new), 9.3.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Change 1-------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

CGI	Cell Global Identifier

CP	Control Plane

DL	Downlink

EPC	Evolved Packet Core

GUAMI	Globally Unique AMF Identifier

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

N3IWF	Non 3GPP InterWorking Function

NGAP	NG Application Protocol

NRPPa	NR Positioning Protocol Annex

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PSCell	Primary SCell

RIM	Remote Interference Management

RS	Reference Signal

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TNLA	Transport Network Layer Association

UP	User Plane

UPF	User Plane Function

-------------------------------------------Change 2-------------------------------------------

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink Remote Interference Management Information Transfer | UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER |
| Downlink Remote Interference Management Information Transfer | DOWNLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER |

-------------------------------------------Change 3-------------------------------------------

## 8.x	Remote Interference Management Information Transfer Procedures

### 8.x.1	Uplink Remote Interference Management Information Transfer

#### 8.x.1.1	General

The purpose of the Uplink Remote Interference Management Information Transfer procedure is to transfer Remote Information Management information from the NG-RAN node to the AMF. The AMF does not interpret the transferred RAN configuration information. This procedure uses non-UE associated signalling.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: Uplink remote interference management information transfer

The NG-RAN node initiates the procedure by sending the UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER message to the AMF.

Upon reception of the UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER message, the AMF it shall transparently transfer it towards the NG-RAN node indicated in the Target RAN Node ID IE.

#### 8.x.1.3	Abnormal Conditions

Void.

### 8.x.2	Downlink Remote Interference Management Information Transfer

#### 8.x.2.1	General

The purpose of the Downlink Remote Interference Management Information Transfer procedure is to transfer Remote Information Management information from the AMF to the NG-RAN node. This procedure uses non-UE associated signalling.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Downlink remote interference management information transfer

The procedure is initiated with a DOWNLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER message sent from the AMF to the NG-RAN node. The NG-RAN node receiving the DOWNLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION TRANSFER message may use the information therein for Remote Interference Management.

#### 8.x.2.3	Abnormal Conditions

Void.

-------------------------------------------Change 4-------------------------------------------

### 9.2.x	Radio-Related Information Messages

#### 9.2.x.1	UPLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION MESSAGE

This message is sent by the NG-RAN node to the AMF to transfer the Remote Interference Management Information.

Direction: NG-RAN  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Target RAN Node |  |  |  |  |  |  |
| >Target RAN Node ID | M |  | 9.3.1.5 |  |  |  |
| >Selected TAI | M |  | 9.3.3.11 |  |  |  |
| >Target RIM Set ID | M |  | 9.3.1.x |  | YES | ignore |
| Source RAN Node |  |  |  |  |  |  |
| >Source RAN Node ID | M |  | 9.3.1.5 |  |  |  |
| >Selected TAI | M |  | 9.3.3.11 |  |  |  |
| >Source RIM Set ID | M |  | 9.3.1.x |  | YES | ignore |
| List of Cells in Set |  | 0..1 |  |  |  |  |
| > List of Cells in Set Item |  | 1..< maxnoofCellsingNB > |  | The list of cells that the conveyed information corresponds to. |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  |  |  |
| RIM message | M |  | ENUMERATED(RS detected, RS disappeared …) | The informational content of Remote Interference Management backhaul message. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsingNB | Maximum no. of cells that can be served by a gNB. Value is 16384. |

-------------------------------------------Change 5-------------------------------------------

#### 9.2.x.2	DOWNLINK REMOTE INTERFERENCE MANAGEMENT INFORMATION MESSAGE

This message is sent by the AMF to the NG-RAN node to transfer the Remote Interference Management Information.

Direction: AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Target RAN Node |  |  |  |  |  |  |
| >Target RAN Node ID | M |  | 9.3.1.5 |  |  |  |
| >Selected TAI | M |  | 9.3.3.11 |  |  |  |
| >Target RIM Set ID | M |  | 9.3.1.x |  | YES | ignore |
| Source RAN Node |  |  |  |  |  |  |
| >Source RAN Node ID | M |  | 9.3.1.5 |  |  |  |
| >Selected TAI | M |  | 9.3.3.11 |  |  |  |
| >Source RIM Set ID | M |  | 9.3.1.x |  | YES | ignore |
| List of Cells in Set |  | 0..1 |  |  |  |  |
| > List of Cells in Set Item |  | 1..< maxnoofCellsingNB > |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  |  |  |
| RIM message | M |  | ENUMERATED(RS detected, RS disappeared …) | The informational content of Remote Interference Management backhaul message. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsingNB | Maximum no. of cells that can be served by a gNB. Value is 16384. |

-------------------------------------------Change 6-------------------------------------------

#### 9.3.1.x	RIM Set ID

This IE defines the Remote Interference Management Set ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RIM Set ID | M |  | BIT STRING (SIZE(22)) | The Remote Interference Management Set ID. |

-------------------------------------------End of changes ------------------------------------------