3GPP TSG-RAN WG3 meeting #104	R3-192807

Reno, Nevada, USA, 13th– 17th May, 2019

Agenda Item:	13.2.1.3

Source: 	Huawei

Title: 		IP address management for IAB nodes

Document for:		Discussion and Decision

- Introduction

Based on the discussion in RAN2 meeting #104, many operators asserted that “The IAB architecture should be aligned with the current operators’ network management/maintenance philosophy, using IP based tools…IAB nodes should be visible from the management nodes which are located outside of the IAB architecture….” [1], and we deduce that the mentioned IP based tools for network management is target at the DU part of IAB node, thus the “IP termination at access IAB node” requires that an IP address should be allocated to the DU part of an IAB node, in order to align with the operators requirement and network management philosophy.

In the RAN3 meeting #103bis, it has been agreed that “DU IP address needs to be different from MT IP address” and “DU IP address needs to be routable” [2].Three candidate solutions are proposed for the IP address allocation for the DU of IAB node, i.e. assigned by the donor DU, assigned by the donor CU, assigned via the OAM.

In this contribution, we are going to discuss IP address management for IAB node for further step.

- Discussion

IP address management for IAB-MT

For the MT part of IAB node, which can be treated as a normal UE during IAB node start up, it will be allocated with an IP address to establish an IP based PDU session towards the IAB node’s UPF. This IP address for MT part should be allocated by core network nodes (e.g. SMF in 5GC for IAB, or PGW in EPC) when the MT attaches to the network (i.e. during IAB node integration to the network).

Proposal 1: The IP address for the MT part of the IAB node is allocated by the core network of the IAB node (e.g. SMF in 5GC for IAB, or PGW in EPC).

IP address management for IAB-DU

For the DU part of IAB node, we will discuss three options about IP address allocation in the following part.

- Option 1. IP address of the IAB node DU is assigned by the IAB donor DU using DHCP (Donor-DU/DHCP based solution.

Considering that the IAB network may change topology, and IAB node can connect to a different IAB donor DU after the topology change. Thus the IAB node should obtain a new IP address from the new IAB donor DU after topology change, this will result in long time interruption of the transmitted traffic since the IP update will cause new TNL connection procedure, as well as the update of F1 interface. It is much possible that IP address reassignment will cause F1-AP re-establishment (e.g. F1 Reset or F1 Setup procedure), in which all the F1AP UE-related contexts and signalling connections are erased, and any existing application level configuration data may be erased. And then all the descendent IAB nodes and corresponding UEs will subject to failure.

Observation 1: For donor-DU/DHCP based solution, change IAB donor DU needs IP address reassignment.

Observation 2: IP address reassignment will cause F1-AP re-establishment (e.g. F1 Reset or F1 Setup procedure), which cause the failure of all the IAB nodes and its descendent IAB nodes and corresponding UEs.

In addition, if IAB node supports multi-connectivity in BH link, an IAB node can connects to the IAB donor CU via two different parent nodes. The two different parent node may connect to the IAB donor CU via more than one IAB donor DU, therefore it still needs to be clarified which IAB donor DU should be responsible for the IP address allocation for a given IAB node if this IAB node connects to multiple IAB donor DUs, and how to enable the IP address allocated by one the IAB donor DU being routable via another IAB donor DU.

Observation 3: Donor-DU/DHCP based solution needs additional work to enable the IP routable in BH DC scenario.

Option 1 requires that the IAB donor DU works as a DHCP proxy or the DHCP server, which will increase the complexity of the IAB donor DU. Moreover, the DHCP based solution requires transmission of the IP broadcast message (e.g. the DHCP Discover message, DHCP Request message, etc.), but the current Uu interface does not support IP broadcast, so it still need to address the problem about how to support the IP broadcast message transmission via multiple wireless backhaul links between the IAB donor DU and the IAB node, which has RAN2 impact. DHCP also requires the client to provide its hardware address (i.e. MAC address) in the DHCP Discover/Request message, however, the IAB node does not have a MAC address, and such problem may result in some modification in the DHCP message, which is out the scope of 3GPP.

Observation 4: The Donor-DU/DHCP based solution requires additional work on enabling the IP broadcast transmission via wireless backhaul links, which has RAN2 impact, and it may also need modification of DHCP messages which is out of 3GPP scope.

- Option 2. IP address of the IAB node DU is assigned by the IAB donor CU (Donor-CU based solution).

In this option, the IAB donor CU can maintain a pool of the IP address for all the connected IAB nodes. After the MT part connect the IAB donor CU, the IAB donor CU can allocate a suitable IP address to this IAB node after the IAB donor CU knows that the connected node is an IAB node rather than a normal UE. The IAB donor CU can include the assigned IP address in some DL RRC message to the MT part of the IAB node. Obviously, this solution is simpler than the option 1, the only RAN2 impact is how to enable the RRC message carry allocated IP address to the IAB node, no additional works are necessary, since the RRC message transfer is supported between the IAB-MT and the IAB donor CU, and the DHCP related 4 steps communication can be avoided also.

Observation 5:  For CU based solution, the IAB donor CU can use RRC message to allocate IP address to the IAB node, this option is simper and has less RAN2 impact than the donor-DU/DHCP based solution.

In addition, it is worth noting that the assigned IP address may relates the IAB donor DU via which the IAB node connects to the IAB donor CU, otherwise, more works about the IP routing configuration between the IAB donor CU and the IAB donor DU are necessary to enable the traffic towards the IAB node can be routed via the IAB donor DU. In fact, this problem is similar to the routing problem in BH DC scenario when adopting option 1, i.e. how to enable the routing between the IAB node and the IAB donor CU through the IAB donor DU, which is not responsible for the IP address allocation of the IAB node.  When IAB node topology changes, IP address reassignment may also be needed, which will result in the failure of all the IAB nodes and its descendent IAB nodes and corresponding UEs

Observation 6: CU based solution also needs additional work to enable IP routable via the donor-DU and also has IP address reassignment issue.

This option has another potential advantage, since all the IP related traffic for the IAB node DU (F1-U, F1-C, OAM, etc.) can be transmitted via the IAB donor CU, it is easy to enable the IAB donor CU to control the BH link configuration (including the routing, bearer mapping, etc.) for carrying the IP related traffic via wireless BH links.

- Option 3. IAB node obtain IP address from the OAM configuration (OAM based solution).

In this option, the IAB node should connect to the OAM before it obtain an IP address for its DU part, thus the IAB node needs to support OAM connectivity through establishing PDU session/PDN connection for its MT part, and download some essential DU related configuration (including the IP address for the DU part of the IAB node) from the OAM. After that, the DU part can connect to the OAM through BH IP routing directly, just like a normal gNB-DU.

Therefore, in this option, the OAM connection will be supported by two steps, the first step is using MT’s user plane connection when the IAB node integrates the network, and the second step is the DU part connects to the OAM directly using IP routing.  This option may require that the OAM being aware of the IAB donor (the IAB donor CU, and/or the IAB donor DU) via which the IAB node is connected. However, since OAM also need information about the IAB donor (at least the IAB donor CU) when configure the cell ID for the IAB node, and extra works are necessary to enable the notification of the IAB donor DU information to the OAM.   Considering that when the IAB node topology changes, IP address reassignment may also be needed if IAB node connect to a new IAB donor DU, which will result in the failure of all the IAB nodes and its descendent IAB nodes and corresponding UEs.

Observation 7: OAM based solution also needs additional work (e.g. providing donor DU information) to enable IP routable via the donor-DU and also has IP address reassignment issue.

Proposal 2: In order to enable IP routable via the donor-DU, all the options have IP address reassignment issue which cause the failure of all the IAB nodes and its descendent IAB nodes and corresponding UEs.

Proposal 3: How to solve IP address reassignment issue should seriously consider for the topology update scenario, and potential enhancement for supporting mobile IAB in future.

IAB donor-CU IP address notification

In order to initialize the TNL setup of F1 interface, the IAB-DU needs to know IP address of the IAB donor-CU. As in CU-DU case, the Donor-CU IP address can be pre-configured in the IAB-DU, which has no impact on standard. Another straight way is the Donor-CU sends its IP address via RRC signalling to the IAB-MT and the IAB-MT forwards the IP address to the IAB-DU. In order to avoid the pre-configuration complexity, it is better to allow the IAB donor-CU sending it IP address to the IAB-node by signalling.

Proposal 4: The IAB donor-CU sends its IP address to the IAB-node by RRC signalling.

- Conclusion

This paper mainly discusses the management of IAB node’s IP address, three candidate solutions are analysed and compared, and then we draw the following observations and proposals:

Observation 1: For donor-DU/DHCP based solution, change IAB donor DU needs IP address reassignment.

Observation 2: IP address reassignment will cause F1-AP re-establishment (e.g. F1AP Reset or F1 Setup procedure), which cause the failure of all the IAB nodes and its descendent IAB nodes and corresponding UEs.

Observation 3: Donor-DU/DHCP based solution needs additional work to enable the IP routable in BH DC scenario.

Observation 4: The Donor-DU/DHCP based solution requires additional work on enabling the IP broadcast transmission via wireless backhaul links, which has RAN2 impact, and it may also need modification of DHCP messages which is out of 3GPP scope.

Observation 5:  For CU based solution, the IAB donor CU can use RRC message to allocate IP address to the IAB node, this option is simper and has less RAN2 impact than the donor-DU/DHCP based solution.

Observation 6: CU based solution also needs additional work to enable IP routable via the donor-DU and also has IP address reassignment issue.

Observation 7: OAM based solution also needs additional work (e.g. providing donor DU information) to enable IP routable via the donor-DU and also has IP address reassignment issue.

Proposal 1: The IP address for the MT part of the IAB node is allocated by the core network of the IAB node (e.g. SMF in 5GC for IAB, or PGW in EPC)

Proposal 2: In order to enable IP routable via the donor-DU, all the options have IP address reassignment issue which cause the failure of all the IAB nodes and its descendent IAB nodes and corresponding UEs.

Proposal 3: How to solve IP address reassignment issue should seriously consider for the topology update scenario, and potential enhancement for supporting mobile IAB in future.

Proposal 4: The IAB donor-CU sends its IP address to the IAB-node by RRC signalling.

References

- R2-1818834, Operators’ joint proposal on IP termination for the IAB architecture.

- Chairman notes of 3GPP TSG RAN WG3 meeting #103bis.