3GPP TSG-RAN3 Meeting #104	R3-192944

Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0147 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dynamic CN Component of PDB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IIOT |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Last SA2#131 meeting agreed to introduce the E2E PDB division in S2-1902833. Specifcally, the dynamic CN component of the PDB may be derived by the NG-RAN node. One way is that: - Configured in the SMF, per delay critical GBR 5QI, for different combinations of PSA-to-NG-RAN-node delays and signalled to NG-RAN during the PDU Session Establishment, PDU Session Modification, Xn/N2 handover and the Service Request procedures. Hence it requires to introduce the dynamic CN component of the PDB so that the RAN can determine the delay budget applicable to the radio interface. At SA2#132 meeting, it was also agreed to shorten the terminology “dynamic CN component of the PDB” to “CN PDB” in S2-1904695. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the CN Packet Delay Budget to GBR QoS Flow Information. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the GBR QoS Flow Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN specification is not aligned with TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2; 9.3.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 0142 TS 38.463 CR 0088 TS 38.473 CR 0368 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

CGI	Cell Global Identifier

CN	Core Network

CP	Control Plane

DL	Downlink

EPC	Evolved Packet Core

GUAMI	Globally Unique AMF Identifier

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

N3IWF	Non 3GPP InterWorking Function

NGAP	NG Application Protocol

NRPPa	NR Positioning Protocol Annex

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PSCell	Primary SCell

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TNLA	Transport Network Layer Association

UP	User Plane

UPF	User Plane Function

----------------------------------------------------------Next Change--------------------------------------------------------

9.3.1.10	GBR QoS Flow Information

This IE indicates QoS parameters for a GBR QoS flow for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.4 | Maximum Bit Rate in DL. Details in TS 23.501 [9]. |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.4 | Maximum Bit Rate in UL. Details in TS 23.501 [9]. |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.4 | Guaranteed Bit Rate (provided there is data to deliver) in DL. Details in TS 23.501 [9]. |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.4 | Guaranteed Bit Rate (provided there is data to deliver). Details in TS 23.501 [9]. |
| Notification Control | O |  | ENUMERATED (notification requested, ...) | Details in TS 23.501 [9]. |
| Maximum Packet Loss Rate Downlink | O |  | Packet Loss Rate 9.3.1.79 | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Details in TS 23.501 [9]. |
| Maximum Packet Loss Rate Uplink | O |  | Packet Loss Rate 9.3.1.79 | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Details in TS 23.501 [9]. |
| CN Packet Delay Budget | O |  | Packet Delay Budget 9.3.1.80 | Delay between the UPF and the NG-RAN node as specified in TS 23.501 [9]. This IE may be present in case of Delay Critical GBR QoS flows, and shall be ignored otherwise. |