3GPP TSG-RAN WG3 Meeting #104	R3-192991

# Reno, U.S.A, 13th – 17th May 2019

Agenda Item:	9.3.16

Source:	Ericsson

Title: 	Discussion on the NAS-PDU IE in PDU Session Resource Release Command

Document for:	Discussion and Approval

# 1	Discussion

In the current NGAP message, PDU SESSION RESOURCE RELEASE COMMAND, there is a top UE level NAS-PDU IE, which is not related to any PDU session.

It is specified that “the purpose of the PDU Session Resource Release procedure is to enable the release of already established PDU session resources for a given UE. The procedure uses UE-associated signalling.”

During the PDU Session Resource Release procedure, there is no mobility aspects. Therefore there is no need to include the UE level NAS-PDU in the N2 message.

On the other hand, the ambiguity may occur if the UE top level NAS-PDU is included in the message. For example, if there is only one PDU session to be released in the message, we could say that the existing NAS-PDU is used to carry the corresponding 5GSM message. But if two PDU sessions are released at the same time, how could the existing NAS-PDU being used?

We have proposed a correction to include the PDU session NAS-PDU in the release command message. On the other hand, we think the UE top level NAS-PDU should be removed or invalidated so that there would be no ambiguity.

There are two alternatives that we propose:

- To remove the UE top level NAS-PDU from the message. Since it is optional present, there is no backwards compatibility problem

- To specify that even the UE is presented as it is, it is not used.

Proposal 1: RAN3 to agree to either remove or invalidate the UE top level NAS-PDU in the PDU SESSION RESOURCE RELEASE COMMAND message.

# 2	Proposal

Proposal 1: RAN3 to agree to either remove or invalidate the UE top level NAS-PDU in the PDU SESSION RESOURCE RELEASE COMMAND message.

The two alternatives are shown in [1] and [2].

# 3	Reference