3GPP TSG-WG3 Meeting #104 	R3-193252

# Reno, Nevada, USA, 13. - 17.05.2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1680 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding PSCell to the User Location Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | User location is reported in several procedures in S1AP for purposes defined in SA2. So far, only PCell is reported there. However, following addition of the PSCell in the Location Report procedure, the location reporting may be enhanced also for those other purposes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PSCell is added to the ULI and procedures where PCell is reported. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only affect the User Location Information IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location accuracy will be different in case of Location Report and other S1AP procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.2, 8.3.3.2, 8.4.3.2, 8.4.4.2, 8.6.2.3, 9.1.5.7, 9.1.5.8, 9.1.7.3, 9.2.1.93, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change, ommited text not changed ***

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: UE Context Release Request procedure. Successful operation.

The eNB controlling a UE-associated logical S1-connection initiates the procedure by generating a UE CONTEXT RELEASE REQUEST message towards the affected MME node.

The UE CONTEXT RELEASE REQUEST message shall indicate the appropriate cause value, e.g., “User Inactivity”, “Radio Connection With UE Lost”, “CSG Subscription Expiry”, “CS Fallback triggered”, “Redirection towards 1xRTT”, “Inter-RAT Redirection”, “UE Not Available for PS Service”, “Release due to pre-emption”, for the requested UE-associated logical S1-connection release.

If the Secondary RAT Usage Report List IE is included in the UE CONTEXT RELEASE REQUEST message, the MME shall handle this information as specified in TS 23.401 [11].

Interactions with UE Context Release procedure:

The UE Context Release procedure should be initiated upon reception of a UE CONTEXT RELEASE REQUEST message. If the UE was configured with EN-DC radio resources at the time UE Context Release Request procedure was triggered, and the PSCell information was available, the eNB shall store the PSCell information in the UE context.

*** Next change, ommited text not changed ***

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release procedure. Successful operation.

The MME initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the eNB.

The UE CONTEXT RELEASE COMMAND message shall contain the UE S1AP ID pair IE if available, otherwise the message shall contain the MME UE S1AP ID IE.

The MME provides the cause IE set to “Load Balancing TAU Required” in the UE CONTEXT RELEASE COMMAND message sent to the eNB for all load balancing and offload cases in the MME.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the eNB shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message. In case of eNB supporting L-GW function for LIPA and/or SIPTO@LN operation, the eNB shall also release any related tunnel resources. In case of successful handover, the eNB using L-GW function for SIPTO@LN operation shall also request using intra-node signalling the collocated L-GW to release the SIPTO@LN PDN connection as defined in TS 23.401 [11].

The eNB shall, if supported, report in the UE CONTEXT RELEASE COMPLETE message location information of the UE in the User Location Information IE. If the UE was configured with EN-DC radio resources at the time UE context release was initiated, and the PSCell information was available, the PSCell Information IE shall be included in the User Location Information IE.

If the User Location Information IE is included in the UE CONTEXT RELEASE COMPLETE message, the MME shall handle this information as specified in TS 23.401 [11].

If the Information on Recommended Cells and eNBs for Paging IE is included in the UE CONTEXT RELEASE COMPLETE message, the MME shall, if supported, store it and may use it for subsequent paging.

If the Cell Identifier and Coverage Enhancement Level IE is included in the UE CONTEXT RELEASE COMPLETE message, the MME shall, if supported, store it and use it for subsequent paging.

If the Secondary RAT Usage Report List IE is included in the UE CONTEXT RELEASE COMPLETE message, the MME shall handle this information as specified in TS 23.401 [11].

*** Next change, ommited text not changed ***

#### 8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Handover notification

The target eNB shall send the HANDOVER NOTIFY message to the MME when the UE has been identified in the target cell and the S1 handover has been successfully completed.

If the Tunnel Information for BBF IE is received in the HANDOVER NOTIFY message, the MME shall, if supported, use it in the core network as specified in TS 23.139 [37].

If the LHN ID IE is included in the HANDOVER NOTIFY message, the MME shall, if supported, use it as specified in TS 23.401 [11].

If the UE is configured with EN-DC radio resources and the PSCell information is available, the PSCell Information IE shall be included in the HANDOVER NOTIFY message.

*** Next change, ommited text not changed ***

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The eNB initiates the procedure by sending the PATH SWITCH REQUEST message to the MME.

If the ‎E-RAB To Be Switched in Downlink List IE in the PATH SWITCH REQUEST message does not include all E-RABs previously included in the UE Context, the MME shall consider the non included E-RABs as implicitly released by the eNB.

When the eNB has received from the radio interface the RRC Resume Cause IE, it shall include it in the PATH SWITCH REQUEST message.

After all necessary updates including the UP path switch have been successfully completed in the EPC for at least one of the E-RABs included in the PATH SWITCH REQUEST E-RAB To Be Switched in Downlink List IE, the MME shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the eNB and the procedure ends. The UE-associated logical S1-connection shall be established at reception of the PATH SWITCH REQUEST ACKNOWLEDGE message.

In case the EPC failed to perform the UP path switch for at least one, but not all, of the E-RABs included in the PATH SWITCH REQUEST E-RAB To Be Switched in Downlink List IE, the MME shall include the E-RABs it failed to perform UP path switch in the PATH SWITCH REQUEST ACKNOWLEDGE E-RAB To Be Released List IE. In this case, the eNB shall release the corresponding data radio bearers, and the eNB shall regard the E-RABs indicated in the E-RAB To Be Released List IE as being fully released.

If the CSG Id IE and no Cell Access Mode IE are received in the PATH SWITCH REQUEST message, the MME shall use it in the core network as specified in TS 23.401 [11]. If the CSG Id IE and the Cell Access Mode IE set to “hybrid” are received in the PATH SWITCH REQUEST message, the MME shall decide the membership status of the UE and use it in the core network as specified in TS 23.401 [11]. If no CSG Id IE and no Cell Access Mode IE are received in the PATH SWITCH REQUEST message and the UE was previously either in a CSG cell or in a hybrid cell, the MME shall consider that the UE has moved into a cell that is neither a CSG cell nor a hybrid cell and use this as specified in TS 23.401 [11].

If the GUMMEI of the MME currently serving the UE is available at the eNB (see TS 36.300 [14]) the eNB shall include the Source MME GUMMEI IE within the PATH SWITCH REQUEST message.

Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message the eNB shall store the received Security Context IE in the UE context and the eNB shall use it for the next X2 handover or Intra eNB handovers as specified in TS 33.401 [15].

The PATH SWITCH REQUEST ACKNOWLEDGE message may contain

-	the UE Aggregate Maximum Bit Rate IE.

-	the MME UE S1AP ID 2 IE, which indicates the MME UE S1AP ID assigned by the MME.

If the UE Aggregate Maximum Bit Rate IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message the eNB shall

-	replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context; the eNB shall use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the UE Aggregate Maximum Bit Rate IE is not contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

In case the EPC decides to change the uplink termination point of the tunnels, it may include the E-RAB To Be Switched in Uplink List IE in the PATH SWITCH REQUEST ACKNOWLEDGE message to specify a new uplink transport layer address and uplink GTP-TEID for each respective E-RAB for which it wants to change the uplink tunnel termination point.

When the eNB receives the PATH SWITCH REQUEST ACKNOWLEDGE message and if this message includes the E-RAB To Be Switched in Uplink List IE, the eNB shall start delivering the uplink packets of the concerned E-RABs to the new uplink tunnel endpoints as indicated in the message.

When the eNB receives the PATH SWITCH REQUEST ACKNOWLEDGE message including the CSG Membership Status IE, and if the cell that serves the UE is a hybrid cell, the eNB shall use it as defined in TS 36.300 [14].

If the MME UE S1AP ID 2 IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall store this information in the UE context and use it for subsequent X2 handovers.

If the Tunnel Information for BBF IE is received in the PATH SWITCH REQUEST message, the MME shall, if supported, use it in the core network as specified in TS 23.139 [37].

If the LHN ID IE is included in the PATH SWITCH REQUEST message, the MME shall, if supported, use it as specified in TS 23.401 [11].

If the ProSe Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, update its ProSe authorization information for the UE accordingly. If the ProSe Authorized IE includes one or more IEs set to “not authorized”, the eNB shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant ProSe service(s).

If the UE User Plane CIoT Support Indicator IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and is set to "supported", the eNB shall, if supported, consider that User Plane CIoT EPS Optimisation as specified in TS 23.401 [11] is supported for the UE.

If the V2X Services Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, update its V2X services authorization information for the UE accordingly. If the V2X Services Authorized IE includes one or more IEs set to “not authorized”, the eNB shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the UE Sidelink Aggregate Maximum Bit Rate IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported:

-	replace the previously provided UE Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for V2X services.

If the Enhanced Coverage Restricted IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the CE-Mode-B Restricted IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and the Enhanced Coverage Restricted IE is not set to restricted and the Enhanced Coverage Restricted information stored in the UE context is not set to restricted, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If information on the UE’s NR security capabilities is available at the eNB (see TS 33.401 [15]) the eNB shall include the NR UE Security Capabilities IE within the PATH SWITCH REQUEST message.

If the NR UE Security Capabilities IE is included in the PATH SWITCH REQUEST message, the MME shall, if supported, consider that the eNB has stored the respective information in the UE context, and proceed as defined in TS 33.401 [15].

If the NR UE Security Capabilities IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 33.401 [15].

If the Aerial UE subscription information IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 36.300 [14].

If the Pending Data Indication IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall use it as defined in TS 23.401 [11].

If the Subscription Based UE Differentiation Information IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, store this information in the UE context for further use according to TS 23.401 [11].

If the UE is configured with EN-DC radio resources and the PSCell information is available, the PSCell Information IE shall be included in the PATH SWITCH REQUEST message.

*** Next change, ommited text not changed ***

#### 8.6.2.3	UPLINK NAS TRANSPORT

Figure 8.6.2.3-1: UPLINK NAS TRANSPORT Procedure

When the eNB has received from the radio interface a NAS message to be forwarded to the MME to which a UE-associated logical S1-connection for the UE exists, the eNB shall send the UPLINK NAS TRANSPORT message to the MME including the NAS message as a NAS-PDU IE. The eNB shall include the TAI and ECGI of the current cell in every S1-AP UPLINK NAS TRANSPORT message.

The NAS-PDU IE contains a UE – MME message that is transferred without interpretation in the eNB.

If the GW Transport Layer Address IE is received in the UPLINK NAS TRANSPORT message, the MME shall, if supported, use it for LIPA operation as specified in TS 23.401 [11].

If the SIPTO L-GW Transport Layer Address IE is received in the UPLINK NAS TRANSPORT message, the MME shall, if supported, use it for SIPTO@LN operation as specified in TS 23.401 [11].

If the LHN ID IE is included in the UPLINK NAS TRANSPORT message, the MME shall, if supported, use it as specified in TS 23.401 [11].

If the UE is configured with EN-DC radio resources and the PSCell information is available, the PSCell Information IE shall be included in the UPLINK NAS TRANSPORT message.

*** Next change, ommited text not changed ***

#### 9.1.5.7	HANDOVER NOTIFY

This message is sent by the target eNB to inform the MME that the UE has been identified in the target cell and the S1 handover has been completed.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| TAI | M |  | 9.2.3.16 |  | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.3 | Indicating HeNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |
| PSCell Information | O |  | 9.2.1.141 |  | YES | ignore |

*** Next change, ommited text not changed ***

#### 9.1.5.8	PATH SWITCH REQUEST

This message is sent by the eNB to request the MME to switch DL GTP tunnel termination point(s) from one end-point to another.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-RAB To Be Switched in Downlink List |  | 1 |  |  | YES | reject |
| >E-RABs Switched in Downlink Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | To deliver DL PDUs. | - |  |
| Source MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| TAI | M |  | 9.2.3.16 |  | YES | ignore |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | ignore |
| CSG Id | O |  | 9.2.1.62 |  | YES | ignore |
| Cell Access Mode | O |  | 9.2.1.74 |  | YES | ignore |
| Source MME GUMMEI | O |  | 9.2.3.9 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.3 | Indicating HeNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |
| RRC Resume Cause | O |  | RRC Establishment Cause 9.2.1.3a |  | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| PSCell Information | O |  | 9.2.1.141 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

*** Next change, ommited text not changed ***

#### 9.1.7.3	UPLINK NAS TRANSPORT

This message is sent by the eNB and is used for carrying NAS information over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| TAI | M |  | 9.2.3.16 |  | YES | ignore |
| GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating GW Transport Layer Address if the GW is collocated with eNB. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating SIPTO L-GW Transport Layer Address if the SIPTO L-GW is collocated with eNB. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |
| PSCell Information | O |  | 9.2.1.141 |  | YES | ignore |

*** Next change, ommited text not changed ***

#### 9.2.1.93	User Location Information

This IE provides location information of a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| User Location Information |  |  |  |  | – | – |
| >E-UTRAN CGI | M |  | 9.2.1.38 |  | – | – |
| >TAI | M |  | 9.2.3.16 |  | – | – |
| >PSCell Information | O |  | 9.2.1.141 |  | YES | ignore |

*** Next change, ommited text not changed ***

-- **************************************************************

--

##### -- Handover Notify

--

-- **************************************************************

HandoverNotify ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { HandoverNotifyIEs} },

...

}

HandoverNotifyIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-EUTRAN-CGI						CRITICALITY ignore	TYPE EUTRAN-CGI				PRESENCE mandatory}|

{ ID id-TAI								CRITICALITY ignore	TYPE TAI					PRESENCE mandatory}|

-- Extension for Release 11 to support BBAI --

{ ID id-Tunnel-Information-for-BBF		CRITICALITY ignore	TYPE TunnelInformation		PRESENCE optional}|

{ ID id-LHN-ID							CRITICALITY ignore	TYPE LHN-ID					PRESENCE optional}|

{ ID id-PSCellInformation				CRITICALITY ignore	TYPE PSCellInformation		PRESENCE optional },

...

}

*** Next change, ommited text not changed ***

-- **************************************************************

--

##### -- Path Switch Request

--

-- **************************************************************

PathSwitchRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PathSwitchRequestIEs} },

...

}

PathSwitchRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-E-RABToBeSwitchedDLList			CRITICALITY reject	TYPE E-RABToBeSwitchedDLList	PRESENCE mandatory}|

{ ID id-SourceMME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-EUTRAN-CGI						CRITICALITY ignore	TYPE EUTRAN-CGI					PRESENCE mandatory}|

{ ID id-TAI								CRITICALITY ignore	TYPE TAI						PRESENCE mandatory}|

{ ID id-UESecurityCapabilities			CRITICALITY ignore	TYPE UESecurityCapabilities		PRESENCE mandatory}|

{ ID id-CSG-Id							CRITICALITY ignore	TYPE CSG-Id						PRESENCE optional}|

{ ID id-CellAccessMode					CRITICALITY ignore	TYPE CellAccessMode				PRESENCE optional}|

{ ID id-SourceMME-GUMMEI				CRITICALITY ignore	TYPE GUMMEI						PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus		PRESENCE optional}|

-- Extension for Release 11 to support BBAI --

{ ID id-Tunnel-Information-for-BBF		CRITICALITY ignore	TYPE TunnelInformation			PRESENCE optional}|

{ ID id-LHN-ID							CRITICALITY ignore	TYPE LHN-ID						PRESENCE optional}|

{ ID id-RRC-Resume-Cause				CRITICALITY ignore	TYPE RRC-Establishment-Cause	PRESENCE optional }|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities			PRESENCE optional}|

{ ID id-PSCellInformation				CRITICALITY ignore	TYPE PSCellInformation			PRESENCE optional },

...

}

E-RABToBeSwitchedDLList					::= E-RAB-IE-ContainerList { {E-RABToBeSwitchedDLItemIEs} }

E-RABToBeSwitchedDLItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSwitchedDLItem			CRITICALITY reject	TYPE E-RABToBeSwitchedDLItem			PRESENCE mandatory	},

...

}

E-RABToBeSwitchedDLItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

iE-Extensions					ProtocolExtensionContainer { { E-RABToBeSwitchedDLItem-ExtIEs} }			OPTIONAL,

...

}

E-RABToBeSwitchedDLItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

*** Next change, ommited text not changed ***

-- **************************************************************

--

##### -- UPLINK NAS TRANSPORT

--

-- **************************************************************

UplinkNASTransport ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{UplinkNASTransport-IEs}},

...

}

UplinkNASTransport-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory}|

{ ID id-EUTRAN-CGI					CRITICALITY ignore	TYPE EUTRAN-CGI						PRESENCE mandatory}|

{ ID id-TAI							CRITICALITY ignore	TYPE TAI							PRESENCE mandatory}|

{ ID id-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress			PRESENCE optional}|

{ ID id-SIPTO-L-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress	PRESENCE optional}|

{ ID id-LHN-ID						CRITICALITY ignore	TYPE LHN-ID							PRESENCE optional}|

{ ID id-PSCellInformation			CRITICALITY ignore	TYPE PSCellInformation				PRESENCE optional },

...

}

*** Next change, ommited text not changed ***

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RABInformationListItem,

id-E-RABItem,

id-Bearers-SubjectToStatusTransfer-Item,

id-Time-Synchronisation-Info,

id-x2TNLConfigurationInfo,

id-eNBX2ExtendedTransportLayerAddresses,

id-MDTConfiguration,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-HO-Cause,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-SignallingBasedMDTPLMNList,

id-MobilityInformation,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-eNBIndirectX2TransportLayerAddresses,

id-Muting-Availability-Indication,

id-Muting-Pattern-Information,

id-NRrestrictioninEPSasSecondaryRAT,

id-NRrestrictionin5GS,

id-Synchronisation-Information,

id-uE-HistoryInformationFromTheUE,

id-LoggedMBSFNMDT,

id-SON-Information-Report,

id-RecommendedCellItem,

id-RecommendedENBItem,

id-ProSeUEtoNetworkRelaying,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-M6Configuration,

id-M7Configuration,

id-RAT-Type,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDL,

id-extended-uEaggregateMaximumBitRateUL,

id-SecondaryRATDataUsageReportItem,

id-E-RABUsageReportItem,

id-UEAppLayerMeasConfig,

id-serviceType,

id-UnlicensedSpectrumRestriction,

id-CNTypeRestrictions,

id-DownlinkPacketLossRate,

id-UplinkPacketLossRate,

id-BluetoothMeasurementConfiguration,

id-WLANMeasurementConfiguration,

id-LastNG-RANPLMNIdentity,

id-PSCellInformation,

maxnoofCSGs,

maxnoofE-RABs,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofPLMNsPerMME,

maxnoofTACs,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofForbLACs,

maxnoofForbTACs,

maxnoofCellsinUEHistoryInfo,

maxnoofCellID,

maxnoofDCNs,

maxnoofEmergencyAreaID,

maxnoofTAIforWarning,

maxnoofCellinTAI,

maxnoofCellinEAI,

maxnoofeNBX2TLAs,

maxnoofeNBX2ExtTLAs,

maxnoofeNBX2GTPTLAs,

maxnoofRATs,

maxnoofGroupIDs,

maxnoofMMECs,

maxnoofTAforMDT,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofCellsforRestart,

maxnoofRestartTAIs,

maxnoofRestartEmergencyAreaIDs,

maxnoofMBSFNAreaMDT,

maxEARFCN,

maxnoofCellsineNB,

maxnoofRecommendedCells,

maxnoofRecommendedENBs,

maxnooftimeperiods,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofBluetoothName,

maxnoofWLANName,

maxnoofConnectedengNBs

*** Next change, ommited text not changed ***

UserLocationInformation ::= SEQUENCE {

eutran-cgi 				EUTRAN-CGI,

tai						TAI,

iE-Extensions			ProtocolExtensionContainer { { UserLocationInformation-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation	CRITICALITY ignore	EXTENSION PSCellInformation			PRESENCE optional},

...

}

*** Remaining text not changed ***