3GPP TSG-RAN3 Meeting #105-Bis	R3-196170

# Chongqing, China, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0252 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UL data forwarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UL Forwarding IE is optional in tabular and mandatory in ASN.1. Functionally, the IE should be optional. However changing the presence in ASN.1 would be a very late non-backward compatible change since the IE is part of handover signalling (as well SN addition/modification). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new IE optional IE (criticality ignore) UL Forwarding Proposal, which inherits the functionality of the existing IE. The UL Forwarding IE is marked as “to be ignored”. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR has an impact on handover, SN addition and SN modification procedures. The change is backwards compatible from ASN.1 perspective. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A tunnel for UL data forwarding is always established during the impacted procedures, regardless of whether it is required. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.3.1.2, 8.3.3.2, 8.3.4.2, 9.2.1.17, 9.2.3.X, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: non-backwards compatible ASN.1 change (mandatory > optional) Rev 1: adopt backwards compatible approach from ASN.1 perspective |  |  |  |  |  |  |  |  |

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

For each E-RAB ID IE included in the QoS Flow To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the content of the IE in the UE context and use it for subsequent inter-system handover.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

At reception of the HANDOVER REQUEST message the target NG-RAN node shall prepare the configuration of the AS security relation between the UE and the target NG-RAN node by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE, as specified in TS 33.501 [28].

Upon reception of the PDU Session Resource Setup List IE, contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as specified in TS 38.413 [5] for the PDU Session Resource Setup procedure. The target NG-RAN node shall report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU session resources. When the target NG-RAN node reports the unsuccessful establishment of a PDU session resource, the cause value should be precise enough to enable the source NG-RAN node to know the reason for the unsuccessful establishment.

For each PDU session if the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store the received PDU Session Aggregate Maximum Bit Rate in the UE context and use it when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [7].

For each QoS flow for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. For each PDU session that the target NG-RAN node decides to admit the data forwarding for at least one QoS flow, the target NG-RAN node includes the PDU Session level DL data forwarding GTP-U Tunnel Endpoint IE within the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resource Admitted Info IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

For each QoS flow for which the source NG-RAN node has not yet received the SDAP end marker packet if QoS flow re-mapping happened before handover, the source NG-RAN node shall include the UL Forwarding Proposal IE within the Data Forwarding and Offloading Info from source NG-RAN node IE in the HANDOVER REQUEST message, and if the target NG-RAN node decides to admit uplink data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted Item IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the uplink data forwarding.

>>>>  NEXT CHANGE  <<<<

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the QoS Flow Level QoS Parameters IE for each QoS flow shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

The S-NG-RAN node shall choose the ciphering algorithm based on the information in the UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the S-NG-RAN node Security Key IE as specified in TS 33.501 [28].

If the Additional QoS Flow Information IE is included for a QoS flow in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE and the Common Network Instance IE is not present, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the Expected UE Behaviour IE, the S-NG-RAN node shall, if supported, store this information and may use it to optimize resource allocation.

If the S-NODE ADDITION REQUEST message contains the Mobility Restriction List IE, the S-NG-RAN node, if supported, shall store this information and use it to select an appropriate SCG.

If the S-NODE ADDITION REQUEST message contains the Index to RAT/Frequency Selection Priority IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NG-RAN node is a gNB and the S-NODE ADDITION REQUEST message contains the PCell ID IE, the S-NG-RAN node shall search for the target NR cell among the NR neighbour cells of the PCell indicated, as specified in the TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the S-NG-RAN node PDU Session Aggregate Maximum Bit Rate IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the MR-DC Resource Coordination Information IE, the S-NG-RAN node should forward it to lower layers and it may use it for the purpose of resource coordination with the M-NG-RAN node. The S-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The S-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the S-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the S-NG-RAN node and the M-NG-RAN node.

If the S-NODE ADDITION REQUEST message contains the NE-DC TDM Pattern IE, the S-NG-RAN node should forward it to lower layers and use it for the purpose of single uplink transmission. The S-NG-RAN node shall consider the value of the received NE-DC TDM Pattern IE valid until reception of a new update of the IE for the same UE.

If the S-NODE ADDITION REQUEST message contains the QoS Flow Mapping Indication IE, the S-NG-RAN node may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

For each bearer for which allocation of the PDCP entity is requested at the S-NG-RAN node:

-	the M-NG-RAN node may propose to apply forwarding of downlink data by including the DL Forwarding IE within PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message. For each bearer that it has decided to admit, the S-NG-RAN node may include the DL Forwarding GTP Tunnel Endpoint IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer.

-	the S-NG-RAN node may include for each bearer in the PDU Session Resource Setup Response Info – SN terminated IE the UL Forwarding GTP Tunnel Endpoint IE to indicates it request data forwarding of uplink packets to be performed for that bearer.

-	the M-NG-RAN node shall include RLC Mode IE for each bearer offloaded from M-NG-RAN node to S-NG-RAN node in the DRBs to QoS Flow Mapping List IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDTION REQUEST message, and the RLC Mode IE indicates the mode that the M-NG-RAN used for the DRB when it was hosted at the M-NG-RAN node.

For each bearer for which the PDCP entity is at the M-NG-RAN node:

-	the M-NG-RAN node shall include the RLC mode IE for each bearer in the DRBs To Be Setup List IE within the PDU Session Resource Setup Info – MN terminated IE of the S-NODE ADDTION REQUEST message to indicate the RLC mode has been configured at the M-NG-RAN node, so that the S-NG-RAN node shall configure the same RLC mode for this MN terminated split bearer.

The M-NG-RAN node may also propose to apply forwarding of UL data when offloading QoS flows for which in-order delivery is requested by including the UL Forwarding Proposal IE in the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message. The S-NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding.

If the Masked IMEISV IE is contained in the S-NODE ADDITION REQUEST message the S-NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

>>>>  NEXT CHANGE  <<<<

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE MODIFICATION REQUEST message, it shall start the timer TXnDCprep.

The S-NODE MODIFICATION REQUEST message may contain

-	within the UE Context Information IE;

-	PDU session resources to be added within the PDU Session Resources To Be Added Item IE;

-	PDU session resources to be modified within the PDU Session Resources To Be Modified Item IE;

-	PDU session resources to be released within the PDU Session Resources To Be Released Item IE;

-	the S-NG-RAN node Security Key IE;

-	the S-NG-RAN node UE Aggregate Maximum Bit Rate IE;

-	the M-NG-RAN node to S-NG-RAN node Container IE;

-	the PDCP Change Indication IE;

-	the SCG Configuration Query IE;

-	the Requested split SRBs IE;

-	the Requested split SRBs release IE;

-	the Additional DRB IDs IE;

-	the MR-DC Resource Coordination Information IE.

If the S-NODE MODIFICATION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE MODIFICATION REQUEST message contains the Mobility Restriction List IE, the S-NG-RAN node shall

-	replace the previously provided Mobility Restriction List by the received Mobility Restriction List in the UE context;

-	use this information to select an appropriate SCG.

If the S-NG-RAN node UE Aggregate Maximum Bit Rate IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall:

-	replace the previously provided S-NG-RAN node UE Aggregate Maximum Bit Rate by the received S-NG-RAN node UE Aggregate Maximum Bit Rate in the UE context;

-	use the received S-NG-RAN node UE Aggregate Maximum Bit Rate for Non-GBR Bearers for the concerned UE as defined in TS 37.340 [8].

If the S-NODE MODIFICATION REQUEST message contains the Index to RAT/Frequency Selection Priority IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE MODIFICATION REQUEST message contains the S-NG-RAN node PDU Session Aggregate Maximum Bit Rate IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE MODIFICATION REQUEST message contains the MR-DC Resource Coordination Information IE, the S-NG-RAN node should forward it to lower layers and it may use it for the purpose of resource coordination with the M-NG-RAN node. The S-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The S-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the S-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the S-NG-RAN node and the M-NG-RAN node.

If the S-NODE MODIFICATION REQUEST message contains the NE-DC TDM Pattern IE, the S-NG-RAN node should forward it to lower layers and use it for the purpose of single uplink transmission. The S-NG-RAN node shall consider the value of the received NE-DC TDM Pattern IE valid until reception of a new update of the IE for the same UE.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the QoS Flow Level QoS Parameters IE for each QoS flow shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

If the Additional QoS Flow Information IE is included for a QoS flow in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE and in the PDU Session Resource Modification Info – SN terminated IE and the Common Network Instance IE is not present, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE and in the PDU Session Resource Modification Info – SN terminated IE, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

If at least one of the requested modifications is admitted by the S-NG-RAN node, the S-NG-RAN node shall modify the related part of the UE context accordingly and send the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message back to the M-NG-RAN node.

The M-NG-RAN node shall include RLC Mode IE for each bearer offloaded from M-NG-RAN node to S-NG-RAN node in the DRBs to QoS Flow Mapping List IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE MODIFICATION REQUEST message, and the RLC Mode IE indicates the mode that the M-NG-RAN used for the DRB when it was hosted at the M-NG-RAN node.

The S-NG-RAN node shall include the PDU sessions for which resources have been either added or modified or released at the S-NG-RAN node either in the PDU Session Resources Admitted To Be Added List IE or the PDU Session Resources Admitted To Be Modified List IE or the PDU Session Resources Admitted To Be Released List IE. The S-NG-RAN node shall include the PDU sessions that have not been admitted in the PDU Session Resources Not Admitted List IE with an appropriate cause value.

If the M-NG-RAN node requests transfer of the PDCP hosting from the S-NG-RAN node to the M-NG-RAN node for a PDU session, in which case the S-NODE MODIFICATION REQUEST message contains an PDU session resource to be released which is configured with the SCG bearer option within the PDU Session Resources To Be Released List IE, the S-NG-RAN node shall include the RLC Mode IE within the DRBs To Be Released List IE in the PDU Session Resources admitted to be released List – SN terminated IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message. The the RLC Mode IE indicates the RLC mode that the S-NG-RAN node uses for the DRB.

If the QoS Flow Mapping Indication IE is included in the S-NODE MODIFICATION REQUEST message for a QoS flow to be modified, the S-NG-RAN node may replace and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the S-NODE MODIFICATION REQUEST message contains for a PDU session resource to be modified which is configured with the SN terminated bearer option, the UL NG-U UP TNL Information at UPF IE the S-NG-RAN node shall use it as the new UL NG-U address.

If the S-NODE MODIFICATION REQUEST message contains for a PDU session resource to be modified which is configured with the MN terminated bearer option, the MN UL PDCP UP TNL Information IE the S-NG-RAN node shall use it as the new UL Xn-U address.

If the S-NODE MODIFICATION REQUEST message contains the QoS flows To Be Released List within the PDU Session Resource Modification Info – SN terminated IE, the S-NG-RAN node may propose to apply forwarding of UL data for the QoS flows for which in-order delivery is requested by including the UL Forwarding Proposal IE in the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Modification Response Info – SN terminated IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

For a PDU session resource to be modified which is configured with the SN terminated bearer option the S-NG-RAN node may include in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the DL NG-U UP TNL Information at NG-RAN IE.

For a PDU session resource to be modified which is configured with the MN terminated bearer option the S-NG-RAN node may include in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the SN DL SCG UP TNL Information IE.

If the PDCP Change Indication IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall act as specified in TS 37.340 [8].

Upon reception of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the M-NG-RAN node shall stop the timer TXnDCprep. If the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message has included the S-NG-RAN node to M-NG-RAN node Container IE, the M-NG-RAN node is then defined to have a Prepared S-NG-RAN node Modification for that Xn UE-associated signalling.

If the SCG Configuration Query IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall provide corresponding radio configuration information within the S-NG-RAN node to M-NG-RAN node Container IE and may provide the corresponding data forwarding related information within the PDU Session Resources with Data Forwarding List IE as specified in TS 37.340 [8].

For each bearer for which allocation of the PDCP entity is requested at the S-NG-RAN node:

-	if applicable, the M-NG-RAN node may propose to apply forwarding of downlink data by including the DL Forwarding IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE MODIFICATION REQUEST message. For each bearer that it has decided to admit, the S-NG-RAN node may include the DL Forwarding GTP Tunnel Endpoint IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer.

-	the S-NG-RAN node may include for each bearer in the PDU Session Resource Setup Response Info – SN terminated IE the UL Forwarding GTP Tunnel Endpoint IE to indicate it requests data forwarding of uplink packets to be performed for that bearer.

The M-NG-RAN node may propose to apply forwarding of UL data when offloading QoS flows for which in-order delivery is requested by including the UL Forwarding Proposal IE in the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Setup Info – SN terminated IE or PDU Session Resource Modification Info – SN terminated IE of the S-NODE MODIFICATION REQUEST message. The S-NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Setup Response Info – SN terminated IE or PDU Session Resource Modification Response Info – SN terminated IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding.

>>>>  NEXT CHANGE  <<<<

### 8.3.4	S-NG-RAN node initiated S-NG-RAN node Modification

#### 8.3.4.1	General

This procedure is used by the S-NG-RAN node to modify the UE context in the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: S-NG-RAN node initiated S-NG-RAN node Modification, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUIRED message to the M-NG-RAN node.

When the S-NG-RAN node sends the S-NODE MODIFICATION REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE MODIFICATION REQUIRED message may contain

-	the S-NG-RAN node to M-NG-RAN node Container IE.

-	PDU session resources to be modified within the PDU Session Resources To Be Modified Item IE;

-	PDU session resources to be released within the PDU Session Resources To Be Released Item IE;

-	the PDCP Change Indication IE;

-	the Spare DRB IDs IE;

-	the Required Number of DRB IDs IE;

-	the QoS Flow Mapping Indication IE;

-	the MR-DC Resource Coordination Information IE.

If the M-NG-RAN node receives a S-NODE MODIFICATION REQUIRED message containing the PDCP Change Indication IE, the M-NG-RAN node shall act as specified in TS 37.340 [8].

If the S-NODE MODIFICATION REQUIRED message contains the MR-DC Resource Coordination Information IE, the M-NG-RAN node may use it for the purpose of resource coordination with the S-NG-RAN node. The M-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE.  The M-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the M-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the M-NG-RAN node and the S-NG-RAN node.

If the M-NG-RAN node receives an S-NODE MODIFICATION REQUIRED message containing the Spare DRB IDs IE, the M-NG-RAN node may take those into consideration to be used for MN-terminated bearers.

If the M-NG-RAN node receives an S-NODE MODIFICATION REQUIRED message containing the Required Number of DRB IDs IE, the M-NG-RAN node shall provide new DRB IDs to be used by the S-NG-RAN node for SN-terminated bearers , if such DRB IDs are available, in the Additional DRB IDs IE included in the S-NODE MODIFICATION CONFIRM message.

If the M-NG-RAN node is able to perform the modifications requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE MODIFICATION CONFIRM message to the S-NG-RAN node. The S-NODE MODIFICATION CONFIRM message may contain the M-NG-RAN node to S-NG-RAN node Container IE.

If the PDCP Duplication Configuration IE in the PDU Session Resource Modification Required Info – SN terminated IE is contained in the S-NODE MODIFICATION REQUIRED message and set to "configured", the M-NG-RAN node shall, if supported, add the RLC entity of secondary path for the indicated DRB. And if the S-NODE MODIFICATION REQUIRED message contains the Duplication Activation IE, the M-NG-RAN node shall, if supported, store this information and use it for the purpose of PDCP duplication.

If the PDCP Duplication Configuration IE in the PDU Session Resource Modification Required Info – SN terminated IE is contained in the S-NODE MODIFICATION REQUIRED message and set to "de-configured", the M-NG-RAN node shall, if supported, delete the RLC entity of secondary path for the indicated DRB.

The S-NG-RAN node may include for each DRB in the DRBs To Be Modified List IE in the S-NODE MODIFICATION REQUIRED message the RLC Status IE to indicate that RLC has been reestablished at the S-NG-RAN node and the M-NG-RAN node may trigger PDCP data recovery.

If the S-NODE MODIFICATION REQUIRED message contains the QoS flows To Be Released List within the PDU Session Resource Modification Info – SN terminated IE, the S-NG-RAN node may also propose to apply forwarding of UL data for which in-order delivery is requested by including the UL Forwarding Proposal IE in the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Modification Required Info – SN terminated IE of the S-NODE MODIFICATION REQUIRED message. The M-NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Modification Confirm Info – SN terminated IE of the S-NODE MODIFICATION CONFIRM message to indicate that it accepts the proposed forwarding.

Upon reception of the S-NODE MODIFICATION CONFIRM message the S-NG-RAN node shall stop the timer.

>>>>  NEXT CHANGE  <<<<

#### 9.2.1.17	Data Forwarding and Offloading Info from source NG-RAN node

This IE contains information from a source NG-RAN node regarding per QoS flow proposed data forwarding and offloading.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flows To Be Forwarded List |  | 1 |  |  | - | - |
| >QoS Flows To Be Forwarded Item |  | 1 .. <maxnoofQoSFlows> |  |  | - | - |
| >>QoS Flow Identifier | M |  | 9.2.3.10 |  | - | - |
| >>DL Forwarding | M |  | 9.2.3.34 |  | - | - |
| >>UL Forwarding | O |  | 9.2.3.90 | This IE shall be ignored. | - | - |
| >>UL Forwarding Proposal | O |  | 9.2.3.X |  | YES | ignore |
| Source DRB to QoS Flow Mapping List | O |  | DRB to QoS Flow Mapping List 9.2.1.15 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

>>>>  NEXT CHANGE  <<<<

#### 9.2.3.X	UL Forwarding Proposal

This IE indicates a proposal for forwarding of uplink packets.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL Forwarding Proposal | M |  | ENUMERATED (UL data forwarding proposed, …) |  |

>>>>  NEXT CHANGE  <<<<

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

>>>>  skip text  <<<<

QoSFLowsToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsToBeForwarded-Item

QoSFLowsToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

dl-dataforwarding			DLForwarding,

ul-dataforwarding			ULForwarding,

iE-Extension		ProtocolExtensionContainer { {QoSFLowsToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFLowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ULForwardingProposal	CRITICALITY ignore EXTENSION ULForwardingProposal	 PRESENCE optional },

...

}

>>>>  skip text  <<<<

UL-UE-Configuration::= ENUMERATED {no-data, shared, only, ...}

ULForwarding	::= ENUMERATED {ul-forwarding-proposed, ...}

ULForwardingProposal	::= ENUMERATED {ul-forwarding-proposed, ...}

>>>>  NEXT CHANGE  <<<<

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

>>>>  skip text  <<<<

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-ULForwardingProposal																				ProtocolIE-ID ::= XXX