3GPP TSG-RAN WG3 Meeting #106	R3-196600

Reno, USA, 18 – 22 November 2019

Title: 	TP to BL CR#1682 “Introduction of MT Early Data Transmission” for TS36.413 MT EDT Data Size

Source: 	Huawei

Agenda item:	14.2

Document for:	other

1. Introduction

In the MT EDT Baseline CR, the value range of Data Size IE is still FFS:

In this contribution, we analyse the RAN3 impacts for NB-IoT and MTC UEs using MT EDT, and provide corresponding Text Proposal.

2. Discussion

The EDT Msg4 (RRCEarlyDataComplete for CP or RRCConnectionRelease for UP) is carried by CCCH/DCCH which are mapped to DL-SCH, and the “Maximum number of DL-SCH transport block bits received within a TTI” is set in TS36.306 as below:

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI |  | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits |
| --- | --- | --- | --- | --- |
| Category NB1 | 680 |  | 680 | 2112 |
| Category NB2 | 2536 |  | 2536 | 6400 |
| DL Category M1 | 1000 |  | 1000 | 25344 |
| DL Category M2 | 4008 |  | 4008 | 73152 |

As the RAN node and CN node are not able to distinguish whether an NB-IoT UE is Cat NB1 or NB2, whether an eMTC UE is Cat M1 or M2, the RAN node can only perform MT EDT in case the DL data size is smaller than 1000 for eMTC, and smaller than 680 for NB-IoT, therefore the maximum data size value defined in the baseline CR, i.e. 1000 is fine. Considering that 0 bit means there is no DL data to be transmitted, it will have the same meaning as the absence of the IE, therefore it is better to update the value range from 0 – 1000 to 1 - 1000.

Proposal: Define the Data Size as INTEGER (1..1,000, …), and remove the FFS.

3. Conclusion

- In this contribution, we analyse the RAN3 impact on MT EDT, get the following proposals:

Proposal: Define the Data Size as INTEGER (1..1,000, …), and remove the FFS.

- The Corresponding Text proposal is provided in section 5.

4. Reference

- R3-196521 Introduction of MT Early Data Transmission, Qualcomm Incorporated, Huawei, Ericsson

5. Text Proposal to the S1AP BL CR#1682 R3-19xxxx

----Start of the Change----

9.1.6	PAGING

This message is sent by the MME and is used to page a UE in one or several tracking areas.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.2.3.10 |  | YES | ignore |
| UE Paging Identity | M |  | 9.2.3.13 |  | YES | ignore |
| Paging DRX | O |  | 9.2.1.16 |  | YES | ignore |
| CN Domain | M |  | 9.2.3.22 |  | YES | ignore |
| List of TAIs |  | 1 |  |  | YES | ignore |
| >TAI List Item |  | 1 .. <maxnoofTAIs> |  |  | EACH | ignore |
| >>TAI | M |  | 9.2.3.16 |  | - |  |
| CSG Id List |  | 0..1 |  |  | GLOBAL | ignore |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Paging Priority | O |  | 9.2.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.2.1.103 |  | YES | ignore |
| Paging eDRX Information | O |  | 9.2.1.111 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.46 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.2.1.115 |  | YES | ignore |
| NB-IoT UE Identity Index value | O |  | 9.2.3.47 |  | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| CE-Mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Data size | O |  | INTEGER (1..1,000, …) | The unit is: bit | YES | ignore |

---End of the Changes----