3GPP TSG-RAN WG3 #106																			R3-197573

Reno, NV, USA, 18-22 November 2019

Agenda item:		10

Source:	Vice Chair (Intel Corporation)

Title:	NR SON/MDT Session Report

Document for:		Agreement

# 1	Agreements

Agreements:

| R3-196509 | Addition of SON features (Huawei) | CR1710r4, TS 36.413 v15.7.1, Rel-16, Cat. B Endorsed as BL |
| --- | --- | --- |
| R3-197566 | Addition of SON feature (CATT) | CR1373r5, TS 36.423 v15.7.0, Rel-16, Cat. B Revision of R3-196510 Endorsed unseen as BL |
| R3-196511 | Addition of SON feature (CMCC,Huawei) | draftCRr, TS 38.300 v15.7.0, Rel-16, Cat. B Endorsed as BL |
| R3-196527 | Addition of SON feature (Huawei) | CR0237r4, TS 38.413 v15.5.0, Rel-16, Cat. B Endorsed as BL |
| R3-196496 | Addition of MDT feature (Huawei) | CR0280r1, TS 38.413 v15.5.0, Rel-16, Cat. B Endorsed as BL |
| R3-196512 | Addition of SON feature (Samsung) | CR0221r5, TS 38.423 v15.5.0, Rel-16, Cat. B Endorsed as BL |
| R3-197567 | Addition of SON features (Ericsson) | CR0142r4, TS 38.463 v15.5.0, Rel-16, Cat. B Revision of R3-196485 Endorsed unseen as BL |
| R3-197568 | Addition of SON feature (Huawei) | CR0441r5, TS 38.473 v15.7.0, Rel-16, Cat. B Revision of R3-196513 Endorsed unseen as BL |
| R3-197165 | Inter-System Ping-Pong (Huawei) | draftCRr, TS 36.300 v15.7.0, Rel-16, Cat. Endorsed as BL |

We will have email discussion after the meeting for ASN.1

The following papers have been treated:

R3-196509, R3-196510, R3-196511, R3-196527, R3-196496, R3-196512, R3-196485, R3-196513, R3-196409, R3-197133, R3-197138, R3-197028, R3-197099, R3-197100, R3-197101, R3-197102, R3-197164, R3-197165, R3-197151, R3-197395, R3-197152, R3-196839, R3-196969, R3-196841, R3-197409, R3-196571

# 2	Comebacks

CB: SON-MDT # 0_HO_Type_indication

-  produce summary of offline discussion

(CATT) summary of offline in R3-197576

CB: SON-MDT # 1_OAM

-  remove failure report section

- discuss details

(ZTE) TP in R3-197577

CB: SON-MDT # 2_InterSystemPingPong

-  merge from R3-197102

- check details

(E///) rev in R3-197578

CB: SON-MDT # 3_UnnecessaryHO

-  merge from R3-197395

- merge from R3-197166, if agreeable

(CATT) rev in R3-197579

CB: SON-MDT # 4_SNChange

-  discuss which node makes root cause analysis

-  discuss UE report

-  produce TP if you are lucky

(Nokia) summary of offline R3-197580

CB: SON-MDT # 5_MLB_Metrics

-  discuss metrics

-  summary of offline, TP if there is convergence

-  can merge from other papers, if agreeable

(DCM) summary of offline in R3-197581

CB: SON-MDT # 6_RACH

-  produce summary offline

-  can discuss proposals from other papers

-  agree on the approach

-  produce TP if there is convergence

(E///) summary of offline in R3-197582

CB: SON-MDT # 7_MDT

-  discuss something

(Samsung) summary of offline R3-197583

# 3	Minutes

| # 10. SON/MDT Support for NR WI (RAN3-led) WID [NR_SON_MDT]: RP-191594 (target: RAN #87) [TU: 1 (1 1)] |  |  |
| --- | --- | --- |
| ## 10.1. General Time plan, skeletons, BLs BL CRs endorsed; all proposals should be in the form of TPs toward the appropriate BL CR(s) |  |  |
| R3-196509 | Addition of SON features (Huawei) | CR1710r4, TS 36.413 v15.7.1, Rel-16, Cat. B E///: editor’s note should be removed in accordance with RAN2 agreements Samsung: RAN2 agreement is about another direction, the note is still needed CATT: LTE RLF can be reported in NR Endorsed as BL |
| R3-196510 | Addition of SON feature (CATT) | CR1373r4, TS 36.423 v15.7.0, Rel-16, Cat. B E///: discrepancy with Xn CR, partial success is present, but not in Xn; also periodicity of the report starts at 1000ms Samsung: agree with E/// CATT: partial success should be removed - Remove partial success Rev in R3-197566, with this change it is Endorsed unseen as BL |
| R3-196511 | Addition of SON feature (CMCC,Huawei) | draftCRr, TS 38.300 v15.7.0, Rel-16, Cat. B Endorsed as BL |
| R3-196527 | Addition of SON feature (Huawei) | CR0237r4, TS 38.413 v15.5.0, Rel-16, Cat. B Endorsed as BL |
| R3-196496 | Addition of MDT feature (Huawei) | CR0280r1, TS 38.413 v15.5.0, Rel-16, Cat. B Endorsed as BL |
| R3-196512 | Addition of SON feature (Samsung) | CR0221r5, TS 38.423 v15.5.0, Rel-16, Cat. B Endorsed as BL |
| R3-196485 | Addition of SON features (Ericsson) | CR0142r3, TS 38.463 v15.5.0, Rel-16, Cat. B E///: load should be capacity - HW load -> HW capacity Rev in R3-197567 with this change it is Endorsed unseen as BL |
| R3-196513 | Addition of SON feature (Huawei) | CR0441r4, TS 38.473 v15.7.0, Rel-16, Cat. B HW: one editorial E///: partial stop issue - Remove partial stop Rev in R3-197568 with this change it is Endorsed unseen as BL |
| We will have email discussion after the meeting for ASN.1 |  |  |
| ## 10.2. Signaling Support for SON In cooperation with RAN2 |  |  |
| ### 10.2.1. Mobility Robustness Optimization QUOTA: 8 Both intra- and inter-system MRO mechanism shall support Rel-15 UEs Introduce failure indication message and HO report in Xn (message names can be revised offline) mobility information of source gNB should be included in HANDOVER REQUEST message UL and DL RAN configuration transfer mechanism is used to exchange MRO information between LTE and NR (i.e. in NG and S1) Unnecessary HO to another system and ping-pong to be discussed in the upcoming meetings UE RLF report from CU to DU to be continued… |  |  |
| ##### 10.2.1.1. Intra-System and Inter-System Connection Failure Intra-system: - In failure indication over Ng, include the RLF report - HO report procedure over NG and S1 shall be supported - For the connection failure due to intra-system mobility, add the description about how to use the failure indication message and HO report in case of RRC re-establishment and RLF report - For the connection failure due to intra-system mobility, add the description about retrieval of information needed for problem analysis. The description of LTE is the baseline |  |  |
| R3-196409 | Reply LS on RACH Optimization and Mobility Robustness Optimization (3GPP RAN2, CATT) | LS in Move to 10.2.1.1 noted |
| R3-197133 | (TP for SON BL CR for TS 38.413) MRO (Samsung) | other Proposal 1: Add the information of Failure Indication into the definition of Inter-system SON Configuration Transfer in TS 38.413. Proposal 2: Add the information of the CGI of the cell the UE connects to after RLF into the definition of NR RLF report and LTE RLF report. It should inform RAN2 to take it into account. Proposal 3: Add the information of the CGI of the cell the UE connects to after RLF in inter system handover report defined in TS 38.413. The cell could be a NR-RAN cell or E-UTRAN cell. Proposal 4: Reuse the IE Re-establishment cell CGI in SON information defined in TS 38.413 to transfer the information of the CGI of the cell the UE connects to after RLF. E///: you propose failure indication over Ng and S1, we have global node id in these messages Samsung: failure happens in LTE, but selected cell is NR ZTE: is it “too early” or “too late”? HW: we are in NG-RAN, we go to E-UTRAN, then failure happens? We didn’t consider this scenario before HW: do we need to change other CGIs as well? Samsung propose to add a new scenario to be future proof Noted |
| R3-197138 | (TP for SON BL CR for TS 38.423): MRO (Samsung) | other Proposal 1: It is proposed to define the structure of FAILURE INDICATION as a choice depending on the UE Report Container. Proposal 2: Keep the structure of HANDOVER REPORT and update the semantic of “Target cell CGI”. E///: we would like to make failure indication a generic procedure, the current structure allows that, but not the one being proposed Noted |
| R3-197028 | Necessity of Handover Type Indication in RLF Report (CATT) | discussion Move to 10.2.1.1 Rev in R3-197515 Proposal1: It is proposed to ask RAN2 to introduce Handover Type Indication IE in UE RLF report. Proposal 2 :It is proposed to ask RAN2 to introduce TAI of failed cell IE in UE RLF Report. Nokia: don’t understand the need HW: agree with Nokia E///: some indication of whether the source node is in the same system or not maybe beneficial Samsung: proposal 2 is valid E///: proposal 2 assumes that NR RLF report is always reported in NR CB: SON-MDT # 0_HO_Type_indication - produce summary of offline discussion (CATT) summary of offline in R3-197576 |
| R3-197099 | MRO Leftover issues (ZTE) | discussion Proposal 1: Regarding RLF report, Send LS to RAN2 with potential two approaches. Proposal 2:Capture MRO O&M requirement and failure report to TS 38.300.The description of LTE is the baseline. Proposal 3: Capture E-UTRAN to NG-RAN ping-pong in TS 36.300 and TS 36.423. E///: we are ok with proposal 3 HW: also ok with O&M req noted |
| R3-197162 | (TP for SON BL CR for TS 38.413): Intra-System and Inter-System Connection Failure (Huawei) | other |
| R3-197394 | Xn Signalling for Accessibility Measurement (Ericsson) | other |
| R3-197029 | LS on introduction of handover type indication in UE RLF Report (CATT) | LS out Move to 10.2.1.1 |
| R3-197100 | (TP for [NR_SON_MDT] BL CR for TS 38.300)MRO stage 2 CR (ZTE) | other E///: mobility settings change hasn’t been discussed, we should have an FFS, not sure about “OAM may define multiple coverage configurations” CB: SON-MDT # 1_OAM - remove failure report section - discuss details (ZTE) TP in R3-197577 |
| R3-197139 | (TP for SON BL CR for TS 38.300): MRO (Samsung) | other |
| R3-197163 | (TP for SON BL CR for TS 38.423): Intra-System and Inter-System Connection Failure (Huawei) | other |
| R3-197392 | Addition of SON features (Ericsson) | other |
| R3-197393 | Signalling of Connection Establishment Failure from gNB-CU to gNB-DU (Ericsson) | other |
| ##### 10.2.1.2. Inter-System Ping-Pong and Unnecessary Handover Unnecessary HO from NR to E-UTRAN is supported, E-UTRAN to NG-RAN is not supported in Rel16 NG-RAN to E-UTRAN ping-pong (and vice versa) shall be supported, including ng-eNB |  |  |
| Inter-system ping-pong |  |  |
| R3-197101 | CR for MRO inter system ping-pong in TS36.300 (ZTE) | other HW: our paper in 7164 addresses the same issue in slightly different way Noted |
| R3-197102 | CR for MRO inter system ping-pong in TS36.423 (ZTE) | other merged |
| R3-197164 | (TP for SON BL CR for TS 36.423): Inter-System Ping-Pong (Huawei) | other ZTE “Encoded according to NG-RAN CGI IE in TS 38.423” should be 38.413 E///: procedural text should be written with respect to the receiver CB: SON-MDT # 2_InterSystemPingPong - merge from R3-197102 - check details (E///) rev in R3-197578 |
| R3-197165 | Inter-System Ping-Pong (Huawei) | draftCRr, TS 36.300 v15.7.0, Rel-16, Cat. Endorsed as BL |
| Inter-system Unnecessary Handover |  |  |
| R3-197151 | Consideration on inter-system measurement configuration of unnecessary HO (CATT) | discussion noted |
| R3-197395 | Measurement configuration for unnecessary IRAT HO (Ericsson) | other noted |
| CATT: Proposal 1: SSB frequency, ssbSubcarrierSpacing, BandIndicator and SMTC information should be provided by the source gNB to target eNB. Proposal 2: It is proposed to include maxRS-IndexCellQual IE and threshRS-Index IE in source eNB to target eNB transparent container. Proposal 3: It is proposed to include ThresholdNR IE and Measurement Duration IE in source eNB to target eNB transparent container. E///: Proposal 1: the range of the RSRP and RSRQ should be INTEGER(0..127) for both Proposal 2: the ThresholdNR should be optional Proposal 3: Include in the Inter-system measurement Configuration IE the following elements: a. ssb-ToMeasure b. ss-RSSI-Measurement c. quantityConfigSet d. blackCellList Nokia: are there real differences between the proposals? E///: band indicator is one difference, and blackcelllist CATT: there is no blackcelllist in LTE, further discussion on this is needed; Samsung: blackcelllist is not needed |  |  |
| R3-197166 | (TP for SON BL CR for TS 36.413): Clarification to unnecessary HO (Huawei) | other |
| R3-197140 | (TP for SON BL CR for TS 36.413): Inter-system unnecessary handover (Samsung) | other |
| R3-197152 | (TP for SON BL CR on 36.413) Addition of measurement configuration for inter-system unnecessary HO (CATT) | other CB: SON-MDT # 3_UnnecessaryHO - merge from R3-197395 - merge from R3-197166, if agreeable (CATT) rev in R3-197579 |
| ##### 10.2.1.3. SN Change Failure |  |  |
| R3-196839 | MRO for SN change failure (Nokia, Nokia Shanghai Bell) | discussion Proposal 1: An RLF Indication like message e.g. SN Failure Indication is is needed for scenarios where the MN does not perform MRO root cause analysis after or while SN initiated SN change and just forwards the received measurements to the currently serving SN. Proposal 2: Introduce a Handover Report like message from MN to SN for optimizing the triggers of the SN initiated SN Change. Proposal 3: A SN receiving a Handover Report like message providing a MRO root cause may, if capable, ignore that parameter and use the provided measurements to perform MRO root cause analysis. Samsung: we need to discuss if MN or SN make decision E///: analysis should happen in SN CB: SON-MDT # 4_SNChange - discuss which node makes root cause analysis - discuss UE report - produce TP if you are lucky (Nokia) summary of offline R3-197580 |
| R3-197137 | (TP for SON BL CR for TS 38.423) XnAP impacts for the solution of SN change failure (Samsung) | other |
| R3-197103 | Detection solution for SN change failure (ZTE) | discussion |
| R3-197147 | Consideration on support of SN change failure in case of MR-DC (CATT) | discussion |
| R3-197167 | (TP for SON BL CR for TS 38.423): SN Change Failure (Huawei) | other |
| R3-197104 | (TP for [NR_SON_MDT] BL CR for TS 38.300) Detection solution for SN change failure (ZTE) | other |
| R3-197105 | (TP for [NR_SON_MDT] BL CR for TS 38.423) Detection solution for SN change failure (ZTE) | other |
| ##### 10.2.1.4 .CU-DU Aspects for MRO |  |  |
| R3-196871 | RLF information signaling between gNB-CU and gNB-DU (LG Electronics) | discussion |
| R3-197396 | Signalling of RLF information between gNB-CU and gNB-DU (Ericsson) | other |
| R3-197397 | Handling of RLF in the gNB-DU (Ericsson) | other |
| R3-197398 | Signalling of RLF Report to gNB-DU (Ericsson) | other |
| ##### 10.2.1.5. Successful Handover Report |  |  |
| R3-197134 | Discussion for Successful HO report in MRO features (Samsung) | discussion |
| R3-197399 | Considerations on successful HO report (Ericsson) | other |
| R3-197135 | (TP for SON BL CR for TS 38.413) NGAP impacts for Successful HO report in MRO features (Samsung) | other |
| R3-197136 | (TP for SON BL CR for TS 38.423) XnAP impacts for Successful HO report in MRO features (Samsung) | other |
| ##### 10.2.1.6. UE Reported Mobility History |  |  |
| R3-196787 | UE mobility history report (Qualcomm Incorporated) | discussion |
| R3-197148 | Enhancement of UE history information (CATT) | discussion |
| R3-197149 | (TP forSON BL CR on 38.413)Enhancement of UE history information (CATT) | other |
| R3-197150 | (TP for SON BL CR on 38.423)Enhancement of UE history information (CATT) | other |
| R3-197168 | (TP for SON BL CR for TS 38.423) UE reported history information (Huawei) | other |
| R3-197169 | (TP for SON BL CR for TS 38.413) UE reported history information (Huawei) | other |
| ### 10.2.2. Mobility Load Balancing QUOTA: 8 Intra-system Add RESOURCE STATUS REQUEST/RESPONSE/UPDATE procedures for Xn, X2 (for EN-DC), F1 and E1 interfaces (IEs for each interface are to be discussed separately) Acknowledge the need of reporting spatial load distribution of cells; a solution is needed; RAN3 will work on a solution. Details on solutions are FFS. To be continued... Inter-System MLB is not in WI scope |  |  |
| R3-197424 | Addition of Load Sharing and Load Balancing Optimisation (Ericsson) | CR0142r4, TS 38.463 v15.5.0, Rel-16, Cat. B |
| ##### 10.2.2.1. MLB for Xn/X2/F1/E1 CAC shall be supported on F1, Xn, X2 TNL load shall be supported on F1 and E1 Per-slice load: previous summary of offline disc in R3-196161, noted; to be continued… |  |  |
| Load metrics over Xn/X2/F1/E1 |  |  |
| R3-196969 | Metrics for MLB (NTT DOCOMO, INC, CMCC, Verizon wireless) | discussion rev in R3-197509 noted |
| R3-196841 | Load metric based on PRB usage (Nokia, Nokia Shanghai Bell) | discussion noted |
| DCM: Proposal 2: RAN3 to add “number of active UEs” in load reporting and reflect the cardinality defined in RAN2 Proposal 3: RAN3 to add RRC connections related information in load reporting Proposal 4: RAN3 to add PRB Usages in load reporting Nokia: Proposal: Use Composite Available Capacity for NR load reporting. BT: support DCM, wouldn’t the receiving node know SCS? Nokia: not necessarily ZTE: support DCM proposals 2 and 3 E///: parameters agreed for reporting to OAM, not necessarily for dynamic interpretation; how can we interpret these parameters for load balancing? DCM: disagree with Nokia, total number of PRBs is stable over a certain time period; the usage of these metrics is explained in our paper CMCC: metrics proposed have been used for load balancing CB: SON-MDT # 5_MLB_Metrics - discuss metrics - summary of offline, TP if there is convergence - can merge from other papers, if agreeable (DCM) summary of offline in R3-197581 |  |  |
| R3-197268 | Load metric of RRC connections (CMCC) | other |
| R3-197091 | Further Discussion on Load Information Metric for MLB (ZTE) | discussion |
| R3-196840 | [TP for SON BL CR for TS 38.423] Discussion and updates of slice load, beam load and TNL load reporting (Nokia, Nokia Shanghai Bell) | other |
| R3-197404 | Discussion on load information for MLB (Ericsson) | discussion Resp in R3-197495 |
| R3-197170 | (TP for SON BL CR for TS 38.423): MLB (Huawei) | other |
| R3-197092 | Discussion on Per Slice Load Information for MLB (ZTE) | discussion |
| R3-197019 | Discussion on per-band MLB for NR (CATT) | discussion |
| R3-196872 | Load information reporting via Xn and F1 interface (LG Electronics) | discussion |
| Mobility setting procedure |  |  |
| R3-197097 | Mobility Settings Change procedure for MRO/MLB (ZTE) | discussion |
| R3-197173 | (TP for SON BL CR for TS 38.300): Mobility settings change (Huawei) | other |
| R3-197269 | Procedures of MLB (CMCC) | discussion |
| R3-197400 | Mobility Settings Change procedure for NG RAN (Ericsson) | discussion |
| Overload |  |  |
| R3-196842 | [TP for SON BL CR for TS 38.473] Signalling of overload information per slice (Nokia, Nokia Shanghai Bell) | other |
| R3-196843 | [TP for SON BL CR for TS 38.473] Slice Load and TNL Load reporting updates (Nokia, Nokia Shanghai Bell) | other |
| R3-196844 | [TP for SON BL CR for TS 38.463] Discussion and updates of TNL and HW load reporting (Nokia, Nokia Shanghai Bell) | other |
| R3-196873 | (TP for NR_SON_MDT BL CR for TS 38.423) Load information reporting via Xn interface (LG Electronics) | other |
| R3-196874 | (TP for NR_SON_MDT BL CR for TS 38.473) Load information reporting via F1 interface (LG Electronics) | other |
| R3-196970 | (TP for SON BL CR on 36.423)Metrics for MLB (NTT DOCOMO, INC.) | other |
| R3-196971 | (TP for SON BL CR on 38.423)Metrics for MLB (NTT DOCOMO, INC.) | other |
| R3-196972 | (TP for SON BL CR on 38.473)Metrics for MLB (NTT DOCOMO, INC.) | other |
| R3-197020 | (TP for SON BL CR on 38.300) Support on per-band and MR-DC MLB (CATT) | other |
| R3-197023 | (TP for SON BL CR on 38.473) Support on per-band MLB (CATT) | other |
| R3-197094 | CR on Addition of Load Sharing and Load Balancing Optimisation for TS38.420 (ZTE) | other |
| R3-197095 | CR on Addition of Load Sharing and Load Balancing Optimisation for TS38.460 (ZTE) | other |
| R3-197096 | CR on Addition of Load Sharing and Load Balancing Optimisation for TS38.470 (ZTE) | other |
| R3-197098 | (TP for [NR_SON_MDT] BL CR for TS 38.300)Mobility Settings Change procedure for MRO/MLB (ZTE) | other |
| R3-197171 | (TP for SON BL CR for TS 38.473): MLB (Huawei) | other |
| R3-197172 | (TP for SON BL CR for TS 36.423): MLB (Huawei) | other |
| R3-197174 | (TP for SON BL CR for TS 38.423): Mobility settings change (Huawei) | other |
| R3-197267 | (TP for SON BL CR for 38.300) MLB (CMCC) | other |
| R3-197427 | [TP for SON BL CR for TS 36.423] TNL Load reporting updates (Nokia, Nokia Shanghai Bell) | other |
| R3-197401 | Support for Information on Load distribution over Xn (Ericsson) | other |
| R3-197402 | Addition on load information over E1 (Ericsson) | other |
| R3-197403 | Support for information on load distribution over F1 (Ericsson) | other |
| R3-197449 | Addition of Mobility Setting Change procedure (Ericsson) | other |
| ##### 10.2.2.2. Load for Spatial Distribution |  |  |
| R3-197272 | On Per SSB Area Load Reporting (CMCC) | other |
| R3-197175 | (TP for SON BL CR for TS 38.423): Load for Spatial Distribution (Huawei) | other |
| R3-197176 | (TP for SON BL CR for TS 38.473): Load for Spatial Distribution (Huawei) | other |
| R3-197177 | (TP for SON BL CR for TS 36.423): Load for Spatial Distribution (Huawei) | other |
| ##### 10.2.2.3. MLB for MR-DC |  |  |
| R3-197024 | Discussion on MLB for DC scenario (CATT) | discussion |
| R3-197093 | Discussion on Mobility Load Balancing for EN-DC (ZTE) | discussion |
| R3-197450 | Load Balancing in DC scenarios (Ericsson) | discussion |
| R3-197021 | (TP for SON BL CR on 36.423) Support on MLB for EN-DC scenario (CATT) | other |
| R3-197022 | (TP for SON BL CR on 38.423) Support on per-band and MR-DC MLB (CATT) | other |
| R3-197405 | (TP for SON BL CR on 36.423) Support on MLB for EN-DC scenario (Ericsson) | other |
| ### 10.2.3. RACH Optimization QUOTA: 5 |  |  |
| ##### 10.2.3.1. RACH Optimization Enhancements |  |  |
| R3-196845 | Further discussion on RACH optimization (Nokia, Nokia Shanghai Bell) | discussion |
| R3-196883 | (TP for SON BL CR for TS 38.473): RACH report for RACH Optimization (Huawei) | other |
| R3-197409 | Solution for RACH Optimization in NG-RAN (Ericsson) | other Proposal: It is proposed to adopt a solution for RACH configuration conflict detection and resolution based on the following steps: - The gNB-DU detects potential cases of RACH configuration conflicts by monitoring reception of Msg1 followed by absence of Msg3 - The gNB-DU signals the gNB-CU of a possible RACH configuration conflict involving e.g. Cell A - The gNB-CU verifies that other cells in Cell A neighbourhood have an overlapping RACH configuration with Cell A and signals to gNB-DU the RACH configurations to avoid when choosing a new configuration for Cell A - gNB-DU selects a new RACH configuration taking the limitations from the gNB-CU into account - gNB-DU signals gNB-CU of the newly adopted RACH configuration in gNB-DU configuration Update HW: does not have to be triggered by DU Nokia: this is for a very specific case CB: SON-MDT # 6_RACH - produce summary offline - can discuss proposals from other papers - agree on the approach - produce TP if there is convergence (E///) summary of offline in R3-197582 |
| R3-197407 | On the use cases and required F1 signaling for RACH optimization at gNB-DU (Ericsson) | discussion |
| R3-197106 | RACH report for NR RACH optimization (ZTE) | discussion |
| TPs |  |  |
| R3-196847 | [TP for SON BL CR for TS 38.423] Introduction of RACH failure information (Nokia, Nokia Shanghai Bell) | other |
| R3-196884 | (TP for SON BL CR for TS 38.401): RACH report for RACH Optimization (Huawei) | other |
| R3-196885 | (TP for SON BL CR for TS 38.470): RACH report for RACH Optimization (Huawei) | other |
| R3-197266 | TP to 38.300 on support of RACH optimization (CMCC) | other |
| R3-197406 | Stage 2 description for RACH Optimisation solution (Ericsson) | other |
| R3-197408 | RACH Optimisation solution over F1 (Ericsson) | other |
| R3-197410 | Solution for RACH Optimisation over the Xn (Ericsson) | other |
| R3-196846 | [TP for SON BL CR for TS 38.473] Introduction of RACH failure information (Nokia, Nokia Shanghai Bell) | other |
| ##### 10.2.3.2. Configuration Conflicts for RACH Optimization |  |  |
| R3-196886 | (TP for SON BL CR for TS 38.423): PRACH configuration exchange for RACH Optimization (Huawei) | other |
| R3-196887 | (TP for SON BL CR for TS 38.473): PRACH configuration exchange for RACH Optimization (Huawei) | other |
| R3-197107 | RACH report for NR RACH optimization in TS38.473 (ZTE) | other |
| ### 10.2.4. PCI Selection Depending on work progress, this may be discussed in the later part of the WI |  |  |
| R3-197411 | Discussion and Solution for Centralised PCI Selection (Ericsson) | discussion |
| R3-197412 | Discussion and Solution for Distributed PCI Selection (Ericsson) | discussion |
| R3-197413 | Discussion and solution on PCI Reconfiguration (Ericsson) | other |
| ### 10.2.5. Energy Saving OAM requirements only Depending on work progress, this may be discussed in the later part of the WI |  |  |
| ## 10.3. Signaling Support for Minimization of Drive Testing QUOTA: 10 For identified use cases, including coverage optimization, QoS verification via MDT, indoor MDT improvement, location info reporting, and sensor data collection (in cooperation with RAN2) For stand-alone, NR-DC and EN-DC, including CU-DU split architecture W1 specification work is not in the scope Initial Context Setup, Handover Request and Trace Start Message, and retrieve UE context response are used for signaling based MDT activation Deactivate trace and trace failure indication are used for MDT deactivation UE Context Request and Trace Start on F1, Bearer context Setup and Trace Start on E1 are used for signaling based MDT activation. The EM of each node may send the MDT activation to CU-CP, DU, and CU-UP directly. If a gNB-CU receives a management based MDT activation, it may propagate the MDT configuration to DU and/or CU-UP over F1 and E1 if needed. The EM of each node may send the MDT deactivation to CU-CP, DU, and CU-UP directly. If a gNB-CU receives a management based MDT deactivation, it may propagate the management based MDT deactivation to DU and/or gNB-CU-UP if needed. In non-split RAN architecture, the NG-RAN node reports the MDT data to TCE. In split RAN architecture, the MDT data is reported to TCE by each node directly; it is FFS whether the gNB-CU-CP may combine MDT data received by other nodes to report to TCE |  |  |
| ### 10.3.1. MDT Activation and Reporting Signaling based logged MDT configuration includes following parameters (NG): - MDT mode configuration, i.e., logged MDT only; - Area scope of MDT, including cell list of E-CGI or N-CGI, TAC list of serving PLMN, TAI list, and PLMN wide; (may need to be updated pending RAN2 discussion) - Logging interval; - Logging duration; - Bluetooth Measurement Configuration; - WLAN Measurement Configuration; Signaling based immediate MDT configuration includes following parameters (NG): - MDT mode configuration: immediate MDT only, immediate MDT and trace; - Area scope of MDT, including cell list of E-CGI or N-CGI, TAC list of serving PLMN, TAI list, and PLMN wide; (may need to be updated pending RAN2 discussion) - MDT location information, enumerated type; - Signaling based MDT PLMN List. NG-RAN receives the management based MDT allowed information in the NG Initial Context Setup Request message. The management based MDT allowed information includes the Management Based MDT Allowed indication and optionally the Management Based MDT PLMN List |  |  |
| R3-196571 | Discussion the impacts of management based MDT in split RAN (Samsung Electronics Co., Ltd) | discussion Proposal 1: It is proposed to include Management Based MDT Allowed IE in the F1-UE CONTEXT SETUP REQUEST message and E1-BEARER SETUP REQUEST message. Proposal 2: It is proposed UE serving Cell ID shall be send from the CU-CP to the CU-UP. Proposal 3: It is proposed to add CELL TRAFFIC TRACE message, sending from DU/CU-UP to CU-CP, in F1 and E1. E///: it is proposed to only signal management-based MDT allowed, why not list of PLMNs? We don’t think signaling of cell id is needed Need to discuss if cell traffic trace is needed Prefer stage-2 description in one spec Nokia: Cell id on E1 is not needed ZTE: agree with E/// and Nokia Samsung: proposal 1 is agreeable, cell id is required CB: SON-MDT # 7_MDT - discuss something (Samsung) summary of offline R3-197583 |
| R3-196888 | MDT Activation and Reporting (Huawei) | other |
| R3-197108 | Signalling support of NR MDT (ZTE) | discussion |
| R3-197414 | Proposals on Agreements and way forward on MDT (Ericsson) | discussion Move to 10.3.1 |
| R3-197039 | Discussion on signalling based MDT configuration for Split RAN (CATT) | discussion |
| TPs |  |  |
| R3-196572 | F1AP impacts for management based MDT (Samsung Electronics Co., Ltd) | CR0492r, TS 38.473 v15.7.0, Rel-16, Cat. B |
| R3-196573 | E1AP impacts for management based MDT (Samsung Electronics Co., Ltd) | CR0180r, TS 38.463 v15.5.0, Rel-16, Cat. B |
| R3-196574 | MDT activation procedure in split RAN (Samsung Electronics Co., Ltd) | CR0099r, TS 38.401 v15.6.0, Rel-16, Cat. B |
| R3-196889 | MDT Activation and Reporting (Huawei) | CR0276r, TS 38.423 v15.5.0, Rel-16, Cat. B |
| R3-196890 | MDT Activation and Reporting (Huawei) | CR0499r, TS 38.473 v15.7.0, Rel-16, Cat. B |
| R3-196891 | MDT Activation and Reporting (Huawei) | CR0466r, TS 38.463 v15.5.0, Rel-16, Cat. B |
| R3-197109 | Introduction of MDT to TS38.423 (ZTE) | other |
| R3-197110 | Introduction of MDT to TS38.473 (ZTE) | other |
| R3-197111 | Introduction of MDT to TS38.463 (ZTE) | other |
| R3-197415 | MDT configuration support for XnAP (Ericsson) | CR1405r, TS 38.423 v15.5.0, Rel-16, Cat. B Move to 10.3.1 |
| R3-197416 | MDT configuration support for NGAP (Ericsson) | other Move to 10.3.1 |
| R3-197417 | MDT configuration support for E1AP (Ericsson) | CR0474r, TS 38.463 v15.5.0, Rel-16, Cat. B Move to 10.3.1 |
| R3-197418 | MDT Configuration support for F1AP (Ericsson) | CR0519r, TS 38.473 v15.7.0, Rel-16, Cat. B Move to 10.3.1 |
| ### 10.3.2. MDT for Inactive UEs |  |  |
| R3-196892 | (TP for MDT BL CR for TS 38.423): Support of MDT for RRC_INACTIVE Ues (Huawei) | other |
| R3-197419 | Logged MDT availability indicator signal over Xn Discussion (Ericsson) | discussion Move to 10.3.2 |
| R3-197420 | Logged MDT availability indicator signal over Xn CR (Ericsson) | CR1406r, TS 38.423 v15.5.0, Rel-16, Cat. B Move to 10.3.2 |
| ### 10.3.3. MDT for MR-DC |  |  |
| R3-196893 | MDT in MR-DC (Huawei) | discussion |
| R3-196786 | MDT for MR-DC (Qualcomm Incorporated) | discussion |
| R3-197036 | Discussion on MDT configuration for EN-DC scenario (CATT) | discussion |
| R3-197037 | (TP on BL CR for 36.413)Support of MDT configuration for EN-DC (CATT) | other |
| R3-197038 | (TP on BL CR for 36.423)Support of MDT configuration for EN-DC (CATT) | other |
| ### 10.3.4. Void |  |  |
| ### 10.3.5. Specification of Layer 2 Measurements In cooperation with RAN2 |  |  |
| R3-197222 | (TP for MDT BL CR for TS 38.413) E2E delay measurement (Huawei) | other |
| R3-197223 | E2E delay measurement (Huawei) | CR0011r, TS 38.415 v15.2.0, Rel-16, Cat. B |
| R3-197246 | (TP for MDT BL CR for TS 38.463): E2E delay measurement (Huawei) | other |
| R3-197247 | (TP for MDT BL CR for TS 38.473): E2E delay measurement (Huawei) | other |
| R3-197421 | Updates on MDT measurement classification to RAN nodes (Ericsson) | discussion Move to 10.3.5 |
| R3-197422 | Buffered PDCP throughput measurement computation (Ericsson) | discussion |
| R3-197423 | Draft LS on the feasibility of buffered PDCP throughput computation (Ericsson) | LS out |

Agenda color coding

10. Agenda Item

10.1. Sub Agenda Item

10.1.1. Sub-sub Agenda Item

##### 10.1.1.1. Sub-sub-sub Agenda Item

TOPIC GROUPING (used to group and highlight a topic, but it is not an Agenda Item)

Chairman’s notes color coding

| R3-xxxxxx | Available but not yet treated document | Chairman notes |
| --- | --- | --- |
| R3-xxxxxx | This document has low priority |  |
| R3-xxxxxx | This document was not available at submission deadline | Chairman notes |
| R3-xxxxxx | This document was treated at this meeting and a decision has been made | Chairman notes |
| R3-xxxxxx | Agreed (or approved) proposal: e.g. Working Assumption, tdoc proposal, etc... | Chairman notes Approved used for Report and Agenda Agreed used for CR to be sent to RAN or LS out p-Approved (partially approved) used for BL CR or TR subject to tdoc allocation by MCC for next meeting Merged used for a tdoc agreed to be merge in a BL CR or TR Endorsed used for CR to be agreed by other WG e.g. TS 36.300 or Draft Status Report (DSR) produced in meeting |
| R3-xxxxxx | Request for ComeBack (CB) during the meeting | Chairman notes a Come Back is required during the meeting CB: # n_FolderName - comments (Company Owner) |
| R3-xxxxxx | Open issue which might require further clarification in next meeting | Chairman notes Comments (no agreement) |
| R3-xxxxxx | E-mail discussion | Chairman notes Email#01 Deadline (Company) |
| R3-xxxxxx | “To be continued” discussion: there was no agreement at this meeting and the discussion may continue at the next meeting | Chairman notes To be continued |
| R3-xxxxxx | Important warning for consideration | Chairman notes Important warning for consideration |